# nABBREVIATIONS AND ACRONYMS

BK:			Bank of Kigali.

BOQ: 			Bills of Quantities

BPR:  			Bank populaire du rwanda.

CAD: 			Computer Aided Design

CAD:			Computer Aided Design

CBD: 			Central Business District

DDP: 			District Development Plan

DTM: 			Digital Terrain Model

EDPRS: 		Economic development and Poverty Reduction Strategy

EDPRS:		Economic Development and Poverty Reduction Strategy

EIA: 			Environmental Impact Assessment

EWSA: 	Energy Water and Sanitation Authority FGD: Focus Group Discussion

FAO:          		 Food and Agricultural Organization

GIS: 			Geographic Information Systems

GOR: 			Government of Rwanda

GPS:			Global positioning System

ICT:			 Information and Communication Technology

IDDP: 			Integrated District Development Plan

LDP:  			Local Development Plan

LIS: 			Land Information Systems

LUDP: 			Local Urban Development Plan

MDGs: 			Millennium Development Goals

MINALOC:  		Ministry of Local Government

MINECOFIN:		Ministry of Economy and Finance

MTEF:			Medium-Term Expenditure Framework

NGO:         		 None Governmental Organizations

NICI:			National Information and Communications Infrastructure

NIS:			National Investment Strategy

NISR2012: 		National institute of Statistic Rwanda, 2012

NLUMP: 		National Land Urban Master Plan

OP:			 Operational Paper

PIP:			Public Investment Programmes

REMA: 		Rwanda Environmental Management Authority

RHA:           		Rwanda Housing Authority

RNRA:         		 Rwanda Natural Resources Authority

RTDA:         		Rwanda Transport Development Authority

TIFF:           		Tag Image File Format

UTM:          		Universal Transverse Mercator

VISION 2020: 	The Rwanda’s Vision at the 2020 Horizon.

WSP’s:			Water supply pipe

# EXECUTIVE SUMMARY

The rate of urbanization in Rwanda has been increasing at unprecedented rate especially in the last 10 years. The country envisions accommodating 35% of her population in cities by 2020.Through EDPRS I and II (2013-2018), focus has now shifted towards promotion of urban growth that is socially inclusive, environmentally sustainable and economically viable green integrated cities.

To meet the challenges, the Government has set herself targets that promote sustainable development for this anticipated rapid urban growth, it has embarked on promoting urbanisation by prioritizing secondary cities where this rapid urban growth linked with economic development will be focused.

Ngarama town has been categorized as one of the emerging center by the National Government. This therefore calls for appropriate highly visionary urban planning and development tools, which can generate economic transformation and be a foundation for sustainable urban living, at the same time generating attractiveness for people to live, work, and explore nature playfully.

Planning Challenges identified in Ngarama

The challenges faced by towns and other urban centres in Rwanda, in order to meet EDPRS II target of 35% urban Population by 2020 especially Ngarama include;

- Uncontrolled rapid urban growth

- Lack of detailed physical plans

- Huge unexploited touristic potential

- Lack of adequate sanitation, amenities and utilities facilities

These challenges fall into four main categories and are common in most cities: Economic, Socio-spatial, Environmental and Institutional challenges.

Ngarama town is growing rapidly, not only in population but also in development projects and programs. With this growth however, social urban issues such as;

- Demand for affordable housing

- Need for quality transportation sector

- Fast and uncontrolled urban growth

- Uneven distribution of infrastructure and services

- Unplanned opportunities will continue to exert more pressure on the existing services and infrastructure if not checked and controlled.

It is for the above reasons that, through a public tendering process, VCOM Company (a locally registered company with expertise and experience in large scale strategic and integrated spatial planning, urban design and project designing) was entrusted with the enormous task to design and develop the elaboration of Ngarama Local Urban Development Plan.

Study areas

During Consultations with the District and other stakeholders at Ngarama Sector, priority areas covering 2000 ha were selected for elaboration of the Local urban Development Plan. However, 500Ha were selected in order to put in place a detailed study to guide the implementation of the Plan. These areas were selected based on the District’s needs as well as other policy regulations of managing urbanization. The areas selected

are in Ngarama and Nyarubungo. All these areas have a land use plan that have been proposed  based on the principles of efficient land use plan of the National Land Use and Development Master Plan.

Identified projects to spur the Vision and catalyze the implementation process

By reviewing the master plan/LUDP, the report sought to redefine the Vision and planning concepts for Ngarama with the aim of enhancing the stated goals of EDPRS II. The vision and concept agreed during a consultative meeting at the District recommended a mixed use concept that takes advantage of the existing conditions on the ground. In order to start of the new Ngarama Vision a number of projects have been identified per site that will help to spur sustainable economic growth of the area. These projects have been identified and grouped into phases so that the ones that are easy to implement can be done within the next 5 years under phase1, the next 10 years under phase 2 and so on. The projects that were identified include;

Phase 1- 2015-2020

- Roads- where we have recommended roads which have enough space and do not call for expropriation can be developed within the next five years. These main roads will serve as a linkage between the various neighborhoods and is aimed at establishing in the long run an inner city transport. It is also recommended that the new roads should be designed based on the typological profile provided which takes into consideration both pedestrian and cycle lanes. The responsible actors for implementing these identified projects especially the District Roads include RODA, RTDA, RHA and Ngarama sector.

- Integrated water supply- It was identified that there are so many water springs in the study area with very clean water. By tapping these resources and integrating into the main supply after treatment we shall contribute to even more sustainable use of water resources. This is not an expensive venture and can be done within the next 5 years by WASAC the new water body. Further as a way of managing water harvesting we have recommended that any new building permit issued by the District should also include a water harvesting provision.

- Develop the market and the Agakiriro near in the study area and the clinic should also be developed. The market will take advantage of the open air market that sometimes takes place near the area. While the Agaciriro will provide the much needed building materials for the construction industry.

- Readjustment of plots. This should be done through the District Budget or Central government support. All plots in the priority areas should be re-demarcated and boundary marks placed on the ground, New lease contract should also be issued within the next five 2 years so as to enable those who wish to develop use it as collateral to borrow money from financial institutions and develop their lots.

Phase 2 and 3 (2020-2030)

- This phase will include among others;

- Allocating and aiding acquisition of land to developers

- Continue with Infrastructure development and expansion

- Construction of cultural community hall

- Development of the workshop facility and green parks

- Road expansion in Gatungo and Ngarama

As the area most adjacent to the urban core, these areas is recommended for high density mixed use and create a bustling urban area of mixed use.

- Land acquisition of the area marked for the development of the CBD core.

- Infrastructure development linking with the CBD

- Land sale of mixed use parcels.

Development of other areas

Other areas that lie outside the selected priority areas of Ngarama and Gitinda will also be converted into mixed use urban neighborhoods, commercial, light industrial, and other important uses

- Expansion of roads

- Development of infrastructure

- Development of pedestrian areas

Phase 4 (2030 onwards)

Continuous review of the projects and the ones that have been carried out will continuously be evaluated. More consideration in terms of scope, time frame and boundary will be monitored.

Monitoring and Evaluation

The implementation of the master plan is estimated to take 25 years to complete. Within this period the population of Ngarama is expected to increase 3 times on a low growth rate and 5 times on a higher growth rate. Several factors will come into play One of the key factors that will ease the implementation of the master plan include the development and strengthening of One stop centre through allocating an urban planner to be based at Ngarama sector. Since the overall management of the LUDP will be at the District, they should take the step of sensitizing other stakeholders like WASAC, Energy provider and RTDA as well the infrastructure office at the District so that they can all use the same information. The provided zoning guidelines are detailed enough to guide the implementation process. A simple manual may be developed with more details for ease of reference which should include green growth guidelines in order to encourage the people of Ngarama to develop this new area.

# TABLE OF CONTENTS

ABBREVIATIONS AND ACRONYMS	1

EXECUTIVE SUMMARY	3

TABLE OF CONTENTS	8

1.1  Project overview	23

1.2  Scope of the study	24

1.3  Location of the study area both in National and local context	24

1.4  Goals and work strategies of the LUDP	26

1.5   Provisional scoping	27

1.5.1 Adverse impacts	27

1.5.2 Alternatives	29

1.6  Reporting	30

2.2 GIS Mapping, field data collection and photography	33

2.3 Consultative meetings with residents	33

2.4 Households Interviews	34

CHAPTER TREE:  EXISTING SITUATION ANALYSIS OF THE STUDY AREA	36

3.1  Site boundarie	36

3.5. Topography of the study area	43

3.7  Existing Natural Resources	47

3.7.1 Flora	47

3.7.2 Fauna	47

3.7.3 Tourism	47

3.7.4 Mining in Ngarama sector	48

3.8 Socio- economic assessment	50

3.8.1 Demographic information	50

3.8.2 Economic activities	58

3.8.2.1 Source of income	58

3.8.2.2 Light industries and Handicrafts	59

3.8.2.3 Tourism and Hotel Industries	60

3.8.2.4 Business activities	60

3.9. EXISTING INFRASTRUCTURE ASSESMENT	63

3.9.1. Introduction	63

4.3 Health facilities	68

3.12. Energy for cooking and lighting	75

3.13. Water drainage system	78

3.14.	LAND AND HOUSING OWNESHIP	80

3.14.1.	Introduction	80

3.14.2.  Land-use and settlement outlines	80

3.14.3.	Land and house ownership	81

3.14.4.	Existing housing typologies	83

3.15. Sanitation analysis	88

3.15.1.	Sanitation behavior in Ngarama Urban Centre	88

3.15.2.	Storm water management	89

CHAPTER FOUR: PROPED PLANNING SITUATION	89

4.2.	Detailed zoning	91

4.2.1. Purposed of zoning regulations	91

4.2.2. Ngarama town Zoning Map and interpretation	97

4.2.3. Identification of Zoning Categories for Ngarama Centre	97

4.2.3.1. Residential	97

4.2.3.2. Commercial	97

4.2.3.3. Industrial	97

4.2.3.4. Parks and Open Space	97

4.2.4. Proposed Zoning (land use) Map of Ngarama town	98

4.2.5. Building guideline zoning regulation residential	103

4.2.5.1. low density	103

4.2.5.2. High density zones	105

4.2.5.3. High Density Zone (Rural Residential)	107

4.2.5.4 Medium Density zone	109

4.2.5.5.  Zoning Regulations – Commercial	111

Zoning Regulations – Industrial	115

4.2.5.7. General industry	119

Zoning Regulations – Parks and Open Spaces	122

4.2.5.8. Passive recreational district	122

4.4.5.8. Agricultural Area	125

4.2.5.9.	Protected Area	127

4.3. Planning principles, Visioning and proposed land-use plans	129

4.3.1 Overview	129

4.3.2. Plan Typologies and Key Planning Principles	130

4.3.3. Determinants of Land-Use Patterns in Ngarama town	134

4.3.3.1. Natural Environment	135

4.3.3.2. Demographic and Economic Factors	135

4.3.3.3. Infrastructure	136

4.3.3.4. Land Ownership, Policy Regulations and Management	136

4.3.4. Planning Model and Vision Based on Ngarama Settlement Patterns.	138

4.3.4.1. Basic Principles of Smart Growth	139

4.3.4.2. Rationale for Smart Growth	140

4.3.5.1 Principles of Intelligent Urbanism as a form of Smart Growth	140

5.1.1. Self-Upgrading	147

5.1.2. Incremental Improvement:	148

5.1.3. Creation of Access:	148

5.1.4. Identification of Resettlement Sites:	149

CHAPTER SIX:  PROPOSED HOUSE MODELS	149

6.1. Introduction	149

6.2. Guiding principles for designing housing typologies	150

4.4 Affordable housing and demand	153

4.4.1. Density and dwelling size	154

6.4.2. High density residential two in on	155

6.5. Tree bed room single houses	158

6.6 Maisonette	161

6.7 Guest house	164

6.8 Mixed commercial building	166

6.9 Proposal for community facilities	168

6.9.1 Proposed School	168

6.9.2 Proposed stadium	172

CHAPTER SEVEN: AERIAL VIEW OF NGARAMA AFTER IMPLEMENTATION	173

7.1. Aerial view Ngarama after implementation of LUDP	173

7.2. Cost reducing measures for housing	173

7.3. Planning scenario for houses	174

7.3.1. Low standing housing scenario	174

7.3.2. Medium standing housing scenario	174

CHAPTER EIGHT: PROPOSED AND EXISTING ROAD AND DRAINAGE DESIGN	175

8.1. Introduction	175

8.2. Upgrading of existing road and footpath	177

8.3. Accessibility to each plot in the planned area	177

8.4. Road dimensions	177

8.5. Proposed road network in Ngarama urban center entire area	179

8.6. Soil analysis (SEE ANNEXE)	182

8.6.1. Penetration test	182

8.6.2. Dry density	182

8.6.3. CBR test	182

8.6.4. Compaction test	182

8.6.5. Sieve analysis	182

8.6.6. Atterberg limit analysis	182

8.7. Non-motorized transport	182

8.8. Transportation network connectivity	183

CHAPTER NINE:  ROAD DESIGN AND COSTING	183

9.1. Road design and cost estimate for Ngarama site	183

CHAPTER TEN:  STORM WATER MANAGEMENT	184

10.1. Catchment area	184

10.2. Drainage map of the study area	185

10.3. Drainage design	186

CHAPTER ELEVEN: WATER SUPPLY	190

11.1. Water network in Ngarama Urban center	190

11.1.1. Proposed water network in Ngarama town	190

11.1.2. Existing water consumption	191

11.2. Water demand  for Ngarama Urban center regarding the population	192

11.3. Other water sources that can be used	192

11.4. Water supply options	192

11.5. Proposed water distribution network map	194

11.6. Conclusion on water supply planning	195

CHAPTER TWELVE:  PROPOSED ENERGY PLANNING	195

12.1. Existing situation of the energy in Ngarama urban centre	195

12.1. Proposed electricity network in the study area	197

12.2. Alternative source of energy	199

12.2.1.  Biogaz	199

12.2.2. Solar energy	199

12.2.3. Air energy	199

12.3. Conclusion on energy planning	199

CHAPTER THIRTEEN: SANITATION	200

13.1. Introduction	200

13.2. In situ disposal option	200

13.2.1. In situ disposal	202

13.2.2. Off-site disposal	204

13.2.3. Waste water and excreta treatment	206

13.3. Solid waste management	209

13.4. Waste generation and composting	209

13.4.1. Generation	209

13.4.2. Compositions	209

13.4.3. On site storage	209

13.4.4. Collection of waste	210

13.5. Land fill location	212

13.5.1. Location	212

13.5.2. Site criteria	214

13.5.3. Conclusion	215

CHAPTER QUATEREIZE: GREENING AND BEAUTIFICATION	215

14.1. Beautification of residential area	215

14.2. Street trees	216

14.3. Beautification of schools, hospitals and administrative domains	217

14.4. Trees planting on streets	217

14.5. Improved pedestrian environment	218

CHAPTER QUINZE: conlusion CHALLENGE AND RECOMMANDATIONS	218

CHAPTER SEIZE: IDENTIFICATION OF IMPACT WHEN LUDP IMPLEMENTED	219

16.1. Legal framework and policies:	219

16.2. Land conditions	220

Ngarama and Gatungo;	220

16.3. Biotic community	221

16.4. Water bodies	221

16.5. Socio-economic state	222

16.6. Mitigation measures	222

CHAPTER DIXSEPT: IMPLEMENTATION STRATEGIES AND PHASING	225

17.1. Introduction	225

17.1.1. Implementation Strategies	225

17.1.2. Land Readjustment and Land Pooling	226

17.1.3. Capital Improvement Plan	226

17.1.4. Identified Projects under CIP	227

17.1.5. Other Development (Primary School, Hotel to be constructed)	228

17.1.6. The Estates with Rain water Harvesting	228

17.1.7. Key projects to transform the area	228

17.1.8. Quick Impact Projects to spur the development	229

17.1.9. Selection Criteria	229

17.2. Implementation Strategy Activities, Action Plans, Actors and Source of Funding	237

18.2. Overview	242

18.3. Main Project Components	242

18.3.1. Main Implementation Activities	244

18.4. Introduction and Baseline Information	244

18.4.1. Methodology	244

18.4.7. Detailed site survey	246

18.6. Legal Policy and Institutional Framework	252

18.6.1. The Constitution of the Republic of Rwanda	252

18.6.2. Organic Law on Environment	253

18.6.3. Vision 2020	254

18.6.4. National Policy on Environment	255

18.6.5. Institutional Framework on Environment	256

18.7. Nature and Design Components	260

18.7.1. Existing Land-Use	260

7.2. Site Landscaping	262

18.8. Project Activities	262

18.8.1. Overview	262

18.8.2. Site Preparation for Development	263

18.8.3.	Excavation and Earth Works	263

18.8.4. Utility Services	263

18.9. Final Inspection	266

18.10. Project Material and Products	266

18.10.1. Project Material	266

18.10.2. Project Products	267

18.11. Potential Environmental Impacts	267

18.11.1. Overview	267

18.12.	Social Impact Assessment Aspects	277

18.12.1. Consultations with Interested and Affected Parties	277

18.12.2. Census of Households in the Plan Implementation	278

18.12.3. Socio-Economic Survey	278

18.12.4. Recording of Assets	278

18.12.5. Stakeholders Consultation	278

18.13. Social Impact Aspects	280

18.14.	Health, Safety and Accident Prevention Plan	281

18.14.1. Site Organization	281

18.14.2. Project Team	281

18.14.3. Enforcement of Standards and Legal Requirement	281

18.14.4. Activities of Workers	282

18.14.5. Activities by Machinery and Light Equipment	282

18.15.	Environmental Management Plan	283

18.15.1. Introduction	283

18.16. Mitigation Measures	295

18.16.1. Overview	295

18.16.2. Appraisal of Alternative Development Options	295

REFERENCE :	300

APPENDIX	301

# LIST OF MAPS

Map 1: Ngarama study area in district context	25

Map 2: Ngarama study area in the District context	26

Map 4: Site boundaries	36

Map 5: Study area location map on othophoto	38

Map 6: Proposed zones for detailed local urban development plan	39

Map 7: Existing plot layout	40

Map 8: Hydrologic map of the study area	42

Map 9: Slop map in district context	43

Map 10: Slop map of the study area	44

Map 11: Topographical map of the study area	45

Map 12: study area soil Map	46

Map 13: Population density in the study area	53

Map 14: Population density for current situation of the study area 	56

Map 15	71

Map 16: Health and education facilities distribution in the study area	71

Map 17: Existing road network in Ngarama urban center	73

Map 18: Water distribution network	77

Map 19: Scattered settlement along the main roads	81

Map 20: Existing Land Use	82

Map 21: Proposed Layout plan of Ngarama	89

Map 22: Layout plan of Gatungo site	90

Map 23: Proposed land use for entire area	98

Map 24: Proposed land use of Gatungo site	99

Map 25: Proposed land use for Ngarama site	100

Map 26: Proposed road network in Ngarama site	180

Map 27: Proposed road network for Gatungo	181

Map 28: Ngarama site drainage map	185

Map 29: Gatungo site drainage map	186

Map 30: Existing water network	190

Map 31: Proposed water distribution network map for Ngarama site	194

Map 32: Proposed water distribution network map for Gatungo site	195

Map 33: Existing situation of the energy  in Ngarama urban center	196

Map 34: Proposed electricity network in Ngarama site	197

Map 35: Proposed electricity network in Gatungo site	198

Map 36: Sewarage system in Ngarama site	206

Map 37: Sewage system in Gatungo site	207

Map 38: Location of land fill and cemetery	214

Map 39: Phasing map of Ngarama LUDP	235

# LIST OF FIGURES

Figure 1: Methodological approach	31

Figure 2: Consultative meeting with stakeholders in Nyarubungo, Ngarama  and Bugamba cells	34

Figure 3: Interview to people living in Ngarama urban center	34

Figure 4: Ngarama rocks an historical site to be transformed in touristic area	48

Figure 5: Example of savings group	60

Figure 6: Nursery school	65

Figure 7: Primary school and secondary school ADEPR Ngarama	67

Figure 8: Ngarama Hospital	69

Figure 9: Status of Road Kabarore – Ngarama	72

Figure 10: Electricity network	76

Figure 11: electricity connection to the households	77

Figure 12: Storm water	79

Figure 13: Drainage channel in bad condition	80

Figure 14: Housing typologies	84

Figure 15: New constructed houses	85

Figure 16: Detached building presentation	91

Figure 17: semidetached building presentation	92

Figure 18: Attached building presentation	92

Figure 19: Building Height	93

Figure 20: Building Setback	94

Figure 21: Smart growth concept	139

Figure 22: Setback for middle and corner plot	152

Figure 23: Plot and building coverage	152

Figure 24: Attached and semidetached housing models	152

Figure 25: High density residential two in one	155

Figure 26: tree bedroom single house front and plan vues	158

Figure 27: Perspective and plan vues for Maisonette (villas)	161

Figure 28: Front and plan vues for a proposed guest house	164

Figure 29: mixed commercial building front and plan vue	166

Figure 30: Front and plan vues for proposed primary school	168

Figure 31: perspective and plan vue of proposed secondary school	170

Figure 32: Perspective and plan vue of proposed stadium	172

Figure 33: Cross section and drainage system along the road.	181

Figure 34: Transportation network connectivity	183

Figure 35: Rainfall intensity graph in rural and urban areas	184

Figure 36: Single-chamber septic tank	202

Figure37: pit latrine	204

Figure 38: Ventilated vault (VV) toilets	204

Figure 39: Waste water treatment in WSP	209

Figure 40: Route collection from two sides of the road	211

Figure 41: Total generated waste	212

Figure 42: Natural Attenuation Landfill	214

Figure 43: Example of land scape	215

Figure 44: Typical street beautification layout	216

Figure 45: Beautification of school	217

Figure 46: Trees planting along the streets	217

# LIST OF TABLES

Table 1: Adverse impacts of the project	27

Table 2: sectors, cells and villages covered by the study area	36

Table 3: Proportion area taken in the cell of the study	39

Table 4: Soil type /FAO CLASSIFICATION	46

Table 5: Emerging issues	48

Table 6: Population specifics of eastern province by district in 2002 and 2012	50

Table 7: Trend of Population change for Gatsibo district	51

Table 8: population distribution in the study area by village (2012)	53

Table 9: current population of study area	54

Table 10: Population projection in ngarama urban center	56

Table 11: Sample size	58

Table 12	58

Table 13: cooperatives in Ngarama urban center	61

Table 14: Primary school ownership	64

Table 15: estimated electricity length in the study area	78

Table 16: Electricity and water points	78

Table 17: exiting land use coverage	83

Table 18: Plot categories and size	90

Table 19: Land use types in Ngarama urban center	101

Table 20: Land use types in priority site	102

Table 21: Low density zone planning guidelines	103

Table 22: High Density Zone planning guidelines	105

Table 23: High Density Zone (Rural Residential) planning guidelines	107

Table 24: Medium Density zone planning guidelines	109

Table 25: Mixed Use Commercial zone guidelines	112

Table 26: Light Industrial District	116

Table 27: General Industrial Zone guideline	119

Table 28: Passive Recreational zoning guidelines.	123

Table 29: Agricultural zoning guidelines	125

Table 30: Protected Area	127

Table 31: Estimate cost for high density residential two in one house	156

Table 32: Estimate cost for single house tree bedroom	160

Table 33: Estimate cost for maisonette (villas)	163

Table 34: Suitability mode for each category considered for NMT	176

Table 35: Road category in study area	178

Table 36: Projection of population and water demand	192

Table 37: Proposed electricity network	198

Table 38: Drainage system categories and length	207

Table 39: Proposed phase	232

Table 40: Implementation strategy	237

Table 41: Detailed Impact Matrix for the Proposed Developments	268

Table 42: Anticipated Environmental Impacts and Mitigation Measures at the end of the project Life Cycle	271

Table 43: Environmental Management Plan Matrix for the Proposed Development	283

Table 44: Potential Negative Impacts and Mitigation Measures	297

# LIST OF CHARTS

Chart 1: Stakeholders involved in consultative process	35

Chart 2: Population distribution	52

Chart 3: Sample size by cell	57

Chart 4: Number of children per family	58

Chart 5: Source of income	59

Chart 6: Average monthly income	62

Chart 7: Monthly expenditure	63

Chart 8: Primary school ownership	66

Chart 9: Walking distance to education infrastructure	68

Chart Chart10: Walking distance to health facilities	70

Chart 11: Sources of drinking water	74

Chart 12: Distance to the water source	74

Chart 13: Energy used for cooking	75

Chart 14: Land ownership	82

Chart 15: The estimate cost for constructing a house	85

Chart 16: Construction material for walls   Chart 17: Construction material for the roof	86

Chart 18: Number of rooms per house	86

Chart 19: Preferred house to live in	87

Chart 20: Parcels dimensions	87

CHAPTER ONE:  GENERAL INTRODUCTION TO THE STUDY

## Project overview

This planning document is concerned with the formulation of the Local Urban Development Plan (LUDP) FINAL Report of Ngarama Urban Center within Gatsibo District. This center was chosen with the consent of Rwanda Housing Authority and local government authorities of Gatsibo District and Ngama, Gitinda and Nyagihanga sectors in the contextual plan of urban development strategies established by the Government of Rwanda through the institution of Rwanda Housing Authority (RHA). V.CO Ltd bid and were awarded the contract for consultancy services for the Establishment of Local Urban Development Plans for the Ngarama URBAN CENTRE. This center is located in GATSIBO District, in Eastern Province of Rwanda. Within the framework of the studies, the terms of reference provide the production of the following features:

- The implementation of strategic broad guidelines from a simplified scheme of  development of the District including general part of planning and development, the settlement pattern of lands, the preferential location of equipment and layout large zoning.

- The implementation of the Local Plan and Urban Development (LPUD) of  the urban center of Ngarama of an area of 2,000 ha (to be agreed with local authorities and the technical staff of the sectors and  District) across 1/ 5000.

- Developing demonstration and test operation of a restructuring plan property in one of the areas identified as priorities for the town of Ngarama and the plan of subdivision of a resettlement site of populations (1/2000). This will be an overall area of 500ha of the pilot zone, in the site area to be parceled out.

The overall aim of the project is to provide a comprehensive and inclusive process to achieve MININFRA's stated goal of producing a Local Land Urban Development Plan (LLUDP) in a concerted and shared approach in order to achieve sustainable economic social development and balance and provide visibility regarding investments in the short, medium and long term Vision.

The Local Land Urban Development Plan of the urban center of Ngarama centre   is a local development program that sets policies for the long term, in 2020, and which quantifies the actions to undertake in the short and medium terms. It is expected to responds well to a new urban strategy by identifying actions needed to launch operational and development projects.

## Scope of the study

The scope of the study captures 2000ha that was selected and clearly demarcated by the client’s representatives, Ngarama officials and the consultant’s technical team. The study areas will be divided according to priority needs of the urban center with more emphasis on Ngarama urban center.

## Location of the study area both in National and local context

Ngarama urban center is located in Ngarama sector and some villages of Gatsibo sector and Nyagihanga Sector, Gatsibo district of the Eastern Province of Rwanda. Gatsibo district borders four other districts comprising: Rwamagana, Nyagatare, Gicumbi and Kayonza   (Map.1). There are 2 cells in Ngarama Sector namely: Ngarama and Nyarubungo cells,  2 villages of  Nyabicwamba cell, Gatsibo sector, 3 villages of Bugamba cell, Ngarama sector and 2 villages of Gitinda cell, Nyagihanga sector. The study area is bordered by Gatsibo sector, Nyagihanga (Map 2). This study focuses on two cells within Ngarama sector which were selected through an all-inclusive consultative process involving the various stakeholders. These cells are Ngarama and Nyarubungo adding two villages of Nyabicwamba of Gatsibo sector and two villages of  Gitinda cell of  Nyagihanga sector.

The geographical analysis of Ngarama urban center provides a perception into possibilities and limitations due to its prime location along in heart of Gatsibo district, housing conditions, tourism opportunities, and commercial activities as an important part of the overall spatial and economic development strategy. Improved infrastructure and commercial viability of Ngarama urban center will impact positively on the residents of the as well as in the entire neighborhood. Below are maps showing Ngarama study area both in the national and local context.

Map 1: Ngarama study area in National  context

Source: GIS Data, V.COLtd, 2015

Map 2: Ngarama study area in the District context

Source: GIS Data, V.COLtd, 2015

## Goals and work strategies of the LUDP

The purpose of urban planning and land use development of urban and rural communities is aimed at improving the living environment as well as to develop long and short-term plans to maximize the efficient usage of the land for sustainable growth and revitalization. During the planning process, various impact issues must be considered, including social, environmental, engineering, economical, and transportation impacts, etc.

Local urban development uses strategies that are working to achieve the community’s vision where changes to both the plan and the future growth might be warranted. Identifying where various “character areas” will be distributed across the city, this report lays the groundwork for a transition to Local Urban Development.

This first stage of the project is of extreme importance and the consultant cannot overstate the impact it will have on the development of the project at subsequent stages. During this middle stage, the consultant team became more familiar with the specific characteristics of the project and prepared, in collaboration with the representatives of Rwanda Housing Authority and other beneficiaries especially the sector officials and district staff, procedures for planning and launching structure plan devoted to the following points:

- Arrangements for working together and developing synergies;

- Assurance of office operations based at District Headquarters to the Consultants was given.

- Expectations vis-à-vis the technology transfer process was shared; especially the application of aerial photography and associated software to the District Council.

## Provisional scoping

### 1.5.1 Adverse impacts

The adverse impacts that may be caused by the project are as the following table;

Table 1: Adverse impacts of the project

| No | Consideration |  | Expected impact |
| --- | --- | --- | --- |
| Social environment |  |  |  |
| 1 | Involuntary resettlement | Number of relocate possibly occurs depending on the road network and facility. |  |
| 2 | 2 Local economy such as employment and livelihood, etc. | Unsuitable planning possibly increases economic gap among communities. |  |
| 3 | Land use and utilization of local resource | There are some possibilities of reduction of agricultural land due to urbanization |  |
| 4 | Split of Communities | Accessibility to social services and communication among community will be promoted |  |
| 5 | Traffic / Existing and new Public Facilities | Traffic volume, especially heavy vehicles, possibly increase |  |
| 6 | Cultural heritage | Historical background conserved and increase in tourism trends |  |
| 7 | Water Usage and accessibility | Potable water accessibility for all will be increased |  |
| 8 | Health care | Deseases will be reduced due to the improvement and extensions of health facilities and programs |  |
|  | Natural environment |  |  |
| 9 | Topography and geographical features | Proper land reclamation possibly decreases overflow frequency at lower areas in the city |  |
| 10 | Soil Erosion | Proper Land Use Plan possibly increases green covered areas. As a result, possible soil erosion may prevented during heavy rains |  |
| 11 | Landscape | Extent of impact is unknown at a present stage. However, there is some possibility that new infrastructures are likely to change the town landscape |  |
| 12 | Global Warming | Extent of impact is unknown at a present stage. However, CO2 likely to increase due to increase of traffic volume and industries |  |
| 13 | Fauna, Flora and Biodiversity | Conserved due to different laws and regulations introduced in favor of environment protection and biodiversity conservation |  |
| Pollution |  |  |  |
| 14 | Air pollution | Increased due to the traffic volume increase |  |
| 15 | Waste Capacity of Solid Waste Management and Sewerage | System will be available and developed compare to the population growth. |  |
| 16 | 16 Noise and vibration | Level of noise/vibration possibly increases as traffic volume and industrial activities increase. |  |

Source: author, 2015

### 1.5.2 Alternatives

This Study, which is a policy or a large-scale programme consisting of multiple projects, aims to formulate a comprehensive urban development master plan. The ideas of the Strategic environmental assessment need to be introduced in the course of the Study, in order to select the combination of projects that has the smallest negative impact. When a comprehensive urban development master plan is formulated, multiple alternatives are considered from the broad perspective including various means. These alternatives, including the ‘without project’ option, are assessed for their environmental and social impacts together with expected benefits. Based on this assessment, alternatives are evaluated by the stakeholders to select the best option. The following table gives the summary of alternatives:

Table1.2: Summary of the project alternatives

| Alternative | Descriptions |
| --- | --- |
| Conducting the Study | To formulate a comprehensive urban development master plan considering consensus building among stakeholders as well as communities. |
| Not Conducting the Study | No action will be taken. Without any comprehensive plan, problems such as air pollution, disorderly transportation system, water shortage, water pollution, inundation, toxic hazardous waste disposal, and so on, are likely to be accelerated |

Source: author, 2015

## Reporting

Reference made to the Terms of References of the elaboration of Ngarama center  Local Urban Development Plans, the key reports were organized as below:

1st Report: The Inception and Diagnostic Report

2nd Report: The Draft Detailed Final Report

3rd Report: The Final Report will include all details regarding the existing conditions and the prospective conditions and a good summary from the report will be presented on geographical maps, as shown below:

- Zoning Plans showing the existing and the prospective situations, using a 1:5000 scale;

- Transportation network Plans showing the existing and the prospective situations, using a 1:5000 scale;

- Water supply network Plans showing the existing and the prospective situations, using a 1:5000 scale; Energy and electricity network Plans showing the existing and the prospective situations, using a 1:5000 scale; Urban Restructuring Plans and Urban Resettlement Plans on 500 hectares, using a 1:2000 scale;

CHAPTER TWO:   METHODOLOGY

Figure 1: Methodological approach

This chapter presents the methodological approach engaged in this study to achieve the desired objectives. There are four main categories of data collections tools active in this study, namely: literature review, GIS Mapping and photographing, interviews (household, official and key-informants) and consultative meetings and focused group discussions. In order to take full advantage of the quality of the deliverable product and have a continuous work plan we went through the following methodology.

STAKEHOLDER PARTICIPATION , REVIEWS & COMMENTS

ARCHITECTURAL & ENGINEERING DESIGNS & COSTING

GIS MAPPING & TOPOGRAPHIC DETAIL MAPPING & PLANNING

LITERATURE REVIEW & RECCE SURVEY & SITE DERMARCATION

METHODOLGY

2.1 Literature review

## To realize the objectives of this study, a review of the existing literature

## addressing the urban planning and land-use issues of Ngarama Urban Centre as a whole as well as the surrounding neighbourhood was conducted. A comprehensive analysis of the existing land-uses within the priority cells was carried out to gain an understanding into the existing situation on the ground. Some of the key documents reviewed for this project are:

## Urban Planning policies ; Gatsibo District Development plan; Gatsibo EICV3 District Profile;  Rwanda National Land-use Master plan Plan; Gatsibo financial plans; The Roads Act of 2012; The existing infrastructure and service maps in Gatsibo district; Expropriation Act of 2008.

Other relevant national documents related to land and housing reviewed so as to guide the process of developing and elaborating LUDP of Ngarama are:

- The Vision 2020, which identifies land as the most important yet scarce

resource thus ranks the National Land Policy among the country’s vital and key policies. It is anchored on five basic pillars namely: Good governance, human resource development, infrastructure development, private sector development, agricultural improvement.

- The National Land Policy, would counter the obstacles hindering efficient use

and management of land while guaranteeing safe and stable form of land tenure that would bring about a rational and planned use of land while ensuring sound land management which would culminate to sustainable development. Its implementation will be based on the major policies contained in Rwanda’s National Development Strategy by 2020 commonly known as “Vision 2020”.

## 2.2 GIS Mapping, field data collection and photography

Using aerial photographs with resolution of 0.25cm per pixel, various maps were prepared prior to the study so as to provide location and other spatial information of Ngarama. As a spatial basis for the study, a GIS basemap was developed showing the major public infrastructure and utilities e.g. roads, schools, hospitals, stadia, memorial site etc. Other activities conducted at this stage included:

  - Reconnaissance survey,

  - Transect walks and field observations (Picture2),

  - Photographing various aspects of urban development and taking notes,

  - Overlaying developed basemap on the existing LUDP to compare proposed land-uses versus existing situation on the ground,

  - Surveying using Geodetic GPS to collect data and verify information,

  - Generating DTMs and design of topographic maps to understand topography of the area.

## 2.3 Consultative meetings with residents

The goal here was to recognize and map the priority areas for implementation of LUDP. This process was spearheaded by the Urban Center authorities and all other stakeholders were involved. They include the Consultant’s team, RHA technicians, Sector officials and Community groups among others. After extensive consultation with all stakeholders, a consensus on the areas for design of elaborated LUDP of Ngarama based on the priority needs of the urban center was reached. These areas cover the cells of Ngarama, Bugamba, Nyarubungo and Gitinda.

Figure 2: Consultative meeting with stakeholders in Nyarubungo, Ngarama  and Bugamba cells

Source: field photos, V.COLTD, 2015

## 2.4 Households Interviews

Figure 3: Interview to people living in Ngarama urban center

Source; Field photo, V.COLtd, 2015

This was done to capture the socio-economic situation of the selected site as well as get a view from residents on the challenges they face in accessing and utilizing various facilities and services e.g. schools, hospitals, market etc. The technique employed in this study was interviews conducted using formal and informal methods, structured and semi-structured questionnaires prepared at the preliminary stage of the study.

This involved the use of closed and open-ended questions to generate relevant and adequate data. To select households to be interviewed, random sampling technique was used. The principle behind this method is that each household within the selected site had equal chance of being sampled. The supporting goal of household interviews and Focused group discussion meetings/consultative meetings is to make the planning exercise for implementation of Ngarama LUDP to be both inclusive and participatory. Participation in development planning is a process of equitable and active involvement of all stakeholders in the formulation of development policies and strategies and in the analysis, planning and implementation, monitoring and evaluation of development activities (FAO, 2007)

Chart 1: Stakeholders involved in consultative process

National and Local leaders

ADMINISTRATIVE CONSULTATIVE PROCESS

Community Groups

Other stakeholders

Religious Group

Professional Groups

# CHAPTER TREE:  EXISTING SITUATION ANALYSIS OF THE STUDY AREA

## 3.1  Site boundarie

Map 4: Site boundaries

Source: V.COLtd, 2015

Table 2: sectors, cells and villages covered by the study area

| Sector | Cellule | Village | Area of village(Sqm) |
| --- | --- | --- | --- |
| Gatsibo | Nyabicwamba | Gatungu | 74.84 |
| Gatsibo | Nyabicwamba | Rutovu | 108.00 |
| Sector Total area |  |  | 182.84 |
| Ngarama | Bugamba | Akagerero | 171.00 |
| Ngarama | Bugamba | Kajevuba | 103.00 |
| Ngarama | Bugamba | Kinyinya | 113.08 |
| Ngarama | Ngarama | Amahoro | 40.60 |
| Ngarama | Ngarama | Gatungo | 65.13 |
| Ngarama | Ngarama | Ibare | 15.52 |
| Ngarama | Ngarama | Intsinzi | 59.83 |
| Ngarama | Ngarama | Kabeho | 261.55 |
| Ngarama | Ngarama | Kiyovu | 109.22 |
| Ngarama | Ngarama | Rugarama | 90.82 |
| Ngarama | Ngarama | Urukundo | 18.89 |
| Ngarama | Nyarubungo | Burambira | 143.15 |
| Ngarama | Nyarubungo | Kintarama | 118.73 |
| Ngarama | Nyarubungo | Murama | 86.04 |
| Ngarama | Nyarubungo | Rugarama | 114.69 |
| Ngarama | Nyarubungo | Ruhengeri | 131.51 |
| Ngarama | Nyarubungo | Rutovu | 131.64 |
| Ngarama | Nyarubungo | Rwiri | 119.26 |
| sector Total area |  |  | 1893.67 |
| Nyagihanga | Gitinda | Gatungo | 54.63 |
| Nyagihanga | Gitinda | Isangano | 43.97 |
| sector Total area |  |  | 98.60 |
| Total area |  |  | 2227 |

SOURCE: Field data, and GIS data processing, 2015

The main part of the study area is located in Ngarama sector in nyarubungo, Bugarama and Ngarama cells. The remaining part is located in Gatsibo Sector in Nyabicwamba and Gatsibo cells and Nyahinga sector in Gitinda cell. The selected site has a total area of 2227ha.

Map 5: Study area location map on othophoto

Map 6: Proposed zones for detailed local urban development plan

As it is requested in tender document, the consultants have to make the detailed local urban development plan of 500Ha in the study area. The map above shows the proposed sites within the study area to be developed. The selection was based on the current development and Social economics activities. Thus Ngarama urban center, Gatungo commercial center and Ruhengeri center was proposed.

Table 3: Proportion area taken in the cell of the study

| Sector | Cellule | Area of cells of the study area | Area taken In the cell | % of the cell area taken in a cell |
| --- | --- | --- | --- | --- |
| Gatsibo | Nyabicwamba | 1678.35 | 182.84 | 10.89 |
| Ngarama | Bugamba | 701.24 | 387.08 | 55.20 |
|  | Ngarama | 661.57 | 661.57 | 100.00 |
|  | Nyarubungo | 983.59 | 845.03 | 85.91 |
| Nyagihanga | Gitinda | 396.32 | 98.60 | 24.88 |
| Total |  | 4421.06 | 2175.12 | 49.20 |

Source: Field data and GIS Data processing, V.COLtd, 2015

3.2 Existing plot layout of Ngarama Urban centre

Map 7: Existing plot layout

3.3 Hydrology

Most of Rwanda lies to the east of the Congo-Nile Divide and drains into the Nile basin. The main rivers to the east of Congo-Nile are Mwogo, Rukarara, Mukungwa, Base, Nyabarongo and the Akanyaru. The Nyabarongo is called the Akagera after receiving the waters of Rweru Lake.[1]

- Kagera River : Nyabarongo River / Akagera ; Mbirurume; Mwogo ; Rukarara; Mukungwa; Base; Akanyaru.

Gatsibo District is known of the low rainfall and high temperatures that limit the availability of water. The hydrography of Gatsibo District is largely constituted of Lake Muhazi to the south, streams and rivers such as; Walfu, Karungeri, Cyamuganga, Kanyonyomba, Rwangingo, Kabahanga, Kagina, Kagende, Rwagitima and Ntende. This hydrographic network combined with the aforementioned relief offers timeliness of irrigation in the District (www.gatsibo.gov.rw) accessed 20th march 2015.

In fact, to party the river Umuvumba and Akagera as well as the lake Muhazi, there is not any other river pérenne exploitable by the inhabitants of Gatsibo.  Gatsibo disposes also rivers marécageuses and intermittent between them.  They are Kanyonyomba, Rwangingo, Kabahanga, Kagina, Kagende, Rwagitima and Ntende.  This hydrographic network combined with the aforementioned relief offers timeliness of irrigation.

The River Muvumba that cut across the western part of the District is a result of the reunion of many small rivers in the sectors of Gitoki, Kageyo, Gatsibo, Ngarama and Nyagihanga bordering Gicumbi District. Rice is grown in its large valley. The river is the main water reserve for the people and the cattle in the large dry land. There is no other consistent river that can be exploited by the population in Gatsibo. Few of the rivers found there such as Kanyonyomba, Rwangingo, Kabahanga, Kagina, Kagende are erratic and intermittent. The weak river network constitutes a serious handicap to responding to the needs of water for people and animals especially in Ngarama sector.   This hydrographic network combined with the aforementioned relief offers timeliness of irrigation in the District

Map 8: Hydrologic map of the study area

3.4 Topography of Ngarama urban center

Map 9: Slop map in district context

SOURCE: GIS Mapping and Field data processing, V.COLtd, 2015

## Topography of the study area

The map below shows the percentage of slope of study area and area occupied by each range of slope. The slope of the area helps to know the developable area in chosen site. The proposed study area is located on moderated hills with a few lowlands. It’s almost covered by gentle slope with 57,9 % of total area located on slope ranging from 5 to 10 % and 0.4 % which can be considered as steep slope range from to 20 to 30%. The slope of the study area makes the study area to be suitable for a well-planned urban Centre

Map 10: Slop map of the study area

Map 11: Topographical map of the study area

3.6 Soil Maps

The map and table below show the type of soil and area occupied by each category of soil type.

Map 12: study area soil Map

Table 4: Soil type /FAO CLASSIFICATION

| Soil type /FAO CLASSIFICATION | Area Ha | % |
| --- | --- | --- |
| Dystric Cambisols | 158.78 | 7.30 |
| Dystric Regosols / Dystric Leptosols | 283.80 | 13.05 |
| Eutric Vertisols | 123.83 | 5.69 |
| Humic (Haplic) Ferralsols | 59.52 | 2.74 |
| Humic (Rhodic) Ferralsols | 441.25 | 20.29 |
| Humic Acrisols / Humic Ferralsols | 6.29 | 0.29 |
| Humic Ferralsols | 975.95 | 44.87 |
| Umbric Gleysols | 125.70 | 5.78 |
| Total | 2175.12 | 100.00 |

## 3.7 Existing Natural Resources

### 3.7.1 Flora

The flora of the District of Gatsibo as well as Ngarama sector and other surrounding sectors is characterized by a vegetation cover of steppe wooded. Its hills are covered by short grasses as well as small trees and shrubs. Concerning afforestation, the District of the study area has access to Eucalyptus and of Pinus trees that are generally as a result of afforestation. Agro-forest is also predominant in the hilly sides surrounding Ngarama sector.

### 3.7.2 Fauna

As for the wildlife, the District of Gatsibo has access to an inheritance of the former domain of hunting from Akagera National Park with diversified birds notably the rapacious ones are sparrow hawks, the owls, the sparrows, the guinea fowls, the partridges, the heroes, the ibis, the crows, the prick beef, etc. Also the Hares, the wild boars, the monkeys and other rodents live in the hills where there are small natural shrubs. The hippopotamus are met in the river Umuvumba and in the lake Muhazi. The crocodiles exist also in certain valleys dams as to Rwimbogo. The antelopes, the buffalo, other ruminating animals occupying the Akagera National Park (www.gatsibo.gov.rw) accessed 20th March 2015.

### 3.7.3 Tourism

The tourism is developed towards Akagera National Park, but Ngarama sector is not benefiting the chance of being part of the Akagera National park. The only touristic area is Ngarama rocks which is not actually well prepared for touristic attraction.

Figure 4: Ngarama rocks an historical site to be transformed in touristic area

Source: Field photo of Ngarama rock, V.CO Ltd, 2015

### 3.7.4 Mining in Ngarama sector

Ngarama and other closed Sectors to Ngarama have no major minerals for mining. Quarries contain stones, sand which are used locally as building and construction materials and clay for pottery and bricks making are available in Ngarama sector.

3.7.5 Emerging issues

Table 5: Emerging issues

| Physical characteristic | Issues to be addressed |
| --- | --- |
| Relief and Topography of the study area | Main constructing structures within the Urban Center are not standards ; Irregular plots is a disorder to development of infrastructure, plot restructuring is necessary to provide each household with access, houses in high risks zones have to be removed and reconstructed in a suitable areas. |
| Marshlands and wetlands | Impediments to settlements, provides unique character to the Urban Center, lowest points and recipient of all runoff thus provides natural drainage, Unique and fragile ecosystem and fertile and productive land. Good for agriculture and farming especially fish in the rice in marshy lands. |
| Soils and Mining | - Sandy and clay soil that provides construction materials - Granite stones also used in construction are available in some point of the area |
| Forests and Environment | Ngarama sector has no natural forest, the trees found in the sector are planted by resident, the hilly zone surrounding the study area are not well protected by trees were vegetation seems to disappear in dry season |
| Climatic conditions | - In Ngarama area the dry season is very hot so that the high temperature reaches 32 degree Celicius. The seasons are alternatives with predominance of dry season. Sometimes crops suffer from it. |
| Hydrology | - Plays a very important role in agriculture - Marshy lands very fertile |

## 3.8 Socio- economic assessment

### 3.8.1 Demographic information

# Table 6: Population specifics of eastern province by district in 2002 and 2012

| District | Total population 2002 | Population 2012 |  |  | Population change 2002-2012 | Sex ratio | Average Annual growth (2002-2012) | Population Density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rwamagana | 220,502 | 149,214 | 161,024 | 310238 | 40.7 | 93 | 3.5 | 455 |
| Nyagatare | 255,104 | 228,610 | 238,334 | 466,944 | 83 | 96 | 6.2 | 243 |
| Gatsibo | 283,456 | 208,355 | 225,642 | 433,997 | 53.1 | 92 | 4.4 | 275 |
| Kayonza | 209,723 | 167,767 | 178,984 | 346,751 | 65.3 | 94 | 5.2 | 179 |
| Kirehe | 229,468 | 164,012 | 176,971 | 340,983 | 47.5 | 92 | 4.0 | 286 |
| Ngoma | 235,109 | 162,388 | 176,174 | 338,562 | 45.0 | 93 | 3.7 | 393 |
| Bugesera | 266,775 | 177,404 | 185,935 | 363,339 | 53.0 | 94 | 4.3 | 282 |
| Eastern Province | 1,700,137 | 1,257,750 | 1,343,064 | 2,600,814 | 53.0 | 94 | 4.3 | 275 |

Source: NIRS, 2012

The aim of design and implementation of Local Development plan is to make it possible for enhanced living conditions for the people. An improved standard of living means a growing capacity to afford both the material and non-material profits which a modernized economy makes possible. For most people in Ngarama the first requirements are improved housing conditions, better infrastructure e.g. roads for accessing homes communication with others sectors, improved access to education and health care and more opportunity for lucrative employment.

The growth of the national income, has been adjusting to the growth of population. Thus the relationship between the growth of a nation’s income and that of its population is fundamental to the improvement of human welfare.  As shown in Table 3, Gatsibo district registered the third highest population growth rates (53.1%) in Eastern province after Nyagatare and Kayonza district with records 83%, 65.3% respectively. This rapid change is mainly as a result of rural-urban migration, with more people moving to urban centers such as Kabarore Kiramuruzi and Ngarama to seek better means of earning living conditions.

Table 7: Trend of Population change for Gatsibo district

| Sectors | men | Women | Sex ration | Total | Density(sq.km). |
| --- | --- | --- | --- | --- | --- |
| Gasange | 8,524 | 9,234 | 92 | 17,758 | 464 |
| Gatsibo | 17,594 | 19,225 | 92 | 36,819 | 600 |
| Gitoki | 15,907 | 17,639 | 90 | 33,546 | 448 |
| Kabarore | 25,087 | 25,324 | 99 | 50,411 | 240 |
| Kageyo | 10,433; 11,397; 92; 21,830; 390. | 11,397 | 92 | 21,830 | 390 |
| Kiramuruzi | 14,723 | 16,232 | 91 | 30,955 | 512 |
| Kziguro | 14,248 | 15,791 | 90 | 30,039 | 489 |
| Muhura | 13,992 | 15,211 | 92 | 29,203 | 524 |
| Murambi | 13,612 | 15,383 | 88 | 28,995. | 485 |
| Nyagihanga | 11,581 | 12,645 | 92 | 24,226 | 337 |
| Remera | 12,469 | 13,714 | 91 | 26,183 | 501 |
| Rugarama | 18,019 | 19,183 | 94 | 37,202 | 493 |
| Rwimbogo | 17,737 | 18,803 | 94 | 36,540 | 56 |
| Ngarama | 14,429 | 15,861 | 91 | 30,290 | 518 |

Source: NISR Provisional Population and Housing Census 2012

Chart 2: Population distribution

Source: Source: NISR Provisional Population and Housing Census 2012

At the sector level, Gatsibo is the most densely populated sector followed by Muhura and Ngarama with a population density of 600; 524 and 518 persons per square kilometer respectively. However, it can be noted from Table3 that the population density of Ngarama is the third in entire district especially rural to urban migration is still at manageable levels which can be accommodated in the development of detailed urban development plan of Ngarama urban center.

Table 4: depicts the population density in all sectors of Gatsibo in colour-enhanced format to facilitate easy understanding and comparison of the data presented in Ta

Map 13: Population density in the study area

Table 8: population distribution in the study area by village (2012)

| Sector | Cellule | Population | Household | Village | Shape (sqkm) | DENSITY |
| --- | --- | --- | --- | --- | --- | --- |
| Gatsibo | Nyabicwamba | 487.00 | 125.00 | Gatungu | 0.75 | 650.69 |
| Gatsibo | Nyabicwamba | 472.00 | 95.00 | Rutovu | 1.08 | 437.04 |
| Ngarama | Bugamba | 721.00 | 173.00 | Akagerero | 1.71 | 421.64 |
| Ngarama | Bugamba | 670.00 | 173.00 | Kajevuba | 1.03 | 650.47 |
| Ngarama | Bugamba | 672.00 | 142.00 | Kinyinya | 1.13 | 594.27 |
| Ngarama | Ngarama | 515.00 | 120.00 | Amahoro | 0.41 | 1268.33 |
| Ngarama | Ngarama | 337.00 | 66.00 | Gatungo | 0.65 | 517.40 |
| Ngarama | Ngarama | 528.00 | 113.00 | Ibare | 0.16 | 3401.66 |
| Ngarama | Ngarama | 363.00 | 110.00 | Intsinzi | 0.60 | 606.73 |
| Ngarama | Ngarama | 831.00 | 198.00 | Kabeho | 2.62 | 317.72 |
| Ngarama | Ngarama | 1200.00 | 120.00 | Kiyovu | 1.09 | 1098.72 |
| Ngarama | Ngarama | 430.00 | 98.00 | Rugarama | 0.91 | 473.49 |
| Ngarama | Ngarama | 526.00 | 127.00 | Urukundo | 0.19 | 2784.37 |
| Ngarama | Nyarubungo | 467.00 | 111.00 | Burambira | 1.43 | 326.24 |
| Ngarama | Nyarubungo | 767.00 | 181.00 | Kintarama | 1.19 | 646.01 |
| Ngarama | Nyarubungo | 471.00 | 105.00 | Murama | 0.86 | 547.40 |
| Ngarama | Nyarubungo | 582.00 | 120.00 | Rugarama | 1.15 | 507.46 |
| Ngarama | Nyarubungo | 626.00 | 144.00 | Ruhengeri | 1.32 | 476.00 |
| Ngarama | Nyarubungo | 523.00 | 121.00 | Rutovu | 1.32 | 397.28 |
| Ngarama | Nyarubungo | 443.00 | 99.00 | Rwiri | 1.19 | 371.45 |
| Nyagihanga | Gitinda | 548.00 | 125.00 | Gatungo | 0.55 | 1003.04 |
| Nyagihanga | Gitinda | 413.00 | 91.00 | Isangano | 0.44 | 939.27 |

The mean density of the population of the study area is estimated to 510 inhabitants/km2. .As it is observed in the table4, of the population density, Ngarama and Nyarubungo cells have a high density.

Table 9: current population of study area

| villages | Population Year 2014 | Population year 2015 |  |  |
| --- | --- | --- | --- | --- |
| Gatungu | 691 | 707 |  |  |
| Rutovu | 575 | 590 |  |  |
| Akagerero | 956 | 981 |  |  |
| Kajevuba | 772 | 792 |  |  |
| Kinyinya | 797 | 817 |  |  |
| Amahoro | 1160 | 1190 |  |  |
| Gatungo | 637 | 654 |  |  |
| Ibare | 601 | 616 |  |  |
| Intsinzi | 1259 | 1292 |  |  |
| Kabeho | 1291 | 1325 |  |  |
| Kiyovu | 1387 | 1423 |  |  |
| Rugarama | 499 | 512 |  |  |
| Urukundo | 663 | 680 |  |  |
| Burambira | 500 | 513 |  |  |
| Kintarama | 695 | 713 |  |  |
| Murama | 528 | 542 |  |  |
| Rugarama | 672 | 690 |  |  |
| Ruhengeri | 768 | 788 |  |  |
| Rutovu | 543 | 557 |  |  |
| Rwiri | 523 | 536 |  |  |
| Gatungo | 393 | 403 |  |  |
| Isangano | 494 | 507 |  |  |
| Total | 16404 | 16828 |  |  |

Ngarama Hospital updated population served by hospital, 2015

The effective   population of the study area is estimated to be 16828 inhabitants meaning 14610 (86.82%) for Ngarama sector, 1216 (7.22%) for Nyagihanga sector and 1002 (5.95%) for Gatsibo sector.

Map 14: Population density for current situation of the study area

Table 10: Population projection in ngarama urban center

|  | 2015 | 2020 | 2025 | 2030 |
| --- | --- | --- | --- | --- |
| Population of the study area | 16828 | 17202 | 17584 | 17974 |

Considering the population growth of Gatsibo district, and computing the projection along the time, it seems that Ngarama study areas will have the population increasing year by year

Sample size

For the purpose of evaluating and getting realistic information that will guide the design of elaborated local development plan of Ngarama, household interviews were conducted at the selected cells. The sample size for this study was 50 households distributed fairly equal among the five cells (Chart 5). To allow generalization of research findings, the selected households were spatially distributed over the cells of interest, a research technique called stratified random sampling.

Marital status

As shown in Chart 6, 80% of the respondents are married, 8% single, 8% widowed while 4% are either divorced or separated. An assessment of family size was also relevant to plan for the needs of families, both spatially and socio-economically. The average number of children per family in the study area is ranging between 3-5, most of families have 4 children (34%)

Chart 3: Sample size by cell

Source: field data, V.COLtd, 2015

Table 11: Sample size

| Cells | Sample size | Percentage (%) |
| --- | --- | --- |
| Ngarama | 25 | 50 |
| Nyarubungo | 15 | 30 |
| Nyabicwamba | 4 | 8 |
| Bugamba | 4 | 8 |
| Gitinda | 2 | 4 |

Table 12

| Marital status | Number | % |
| --- | --- | --- |
| Married | 40 | 80 |
| Single | 4 | 8 |
| Divorced | 2 | 4 |
| Widower | 4 | 8 |

Source: field data, V.COLtd, 2015

Chart 4: Number of children per family

Source: Field survey, V.COLtd, 2015

### 3.8.2 Economic activities

#### 3.8.2.1 Source of income

The greatest part of the population of Ngarama gets their living over agriculture and employment. They produce the following crops by order of importance: Maize, Beans, bananas, sorghum, grand nuts, and others none crops cash.

Grave problems fronting food production in Ngarama urban center are not only associated to poor farming practices but also to the great land fragmentation and very small scale agriculture production by family. Poverty among farmers has hindered the capacity of families to invest in the improvement of their land capital and scattered housing system makes land development very difficult and production low

Chart 5: Source of income

Other factors which have made the agricultural situation even worse include:

- Soil degradation;

- Lack of improvised agricultural practice and technology;

- Reduction of farming land for agricultural activities per family due to demographic pressure;

- Irregular weather conditions of the region;

- Exploitation of the natural resource e.g. forest.

#### 3.8.2.2 Light industries and Handicrafts

This comprises activities such as basket weaving, welding, carpentry and brick-making is widely practiced in the urban center. Other light income-generating activities include repair of watches, radios and bicycles which constitute a source of revenue for a number of people at Ngarama market.

Worthiness among lending institutions like Umurenge Savings and Credit Cooperative (Picture 4).

Figure 5: Example of savings group

Ngarama Bank Populaire                                Ngarama Umurenge SACCO

Source: Field photos, 2014.

#### 3.8.2.3 Tourism and Hotel Industries

Ngarama urban center is located far from Akagera National park; the only opportunity to tourism is Ngarama rock which actually is being valorized in a strategic tourism zone Ngarama urban center does not have any noticeable accommodation which meets the national standard that can host tourists. This sector needs to be analyzed well and proper projection made with the aim of tapping the tourist potentials.

#### 3.8.2.4 Business activities

The urban center of Ngarama increases of vibrant business that is growing at a medium rate. Apart from the moto taxi park located at the CBD and Ngarama market, there are various types of shops and banking institutions like BPR, BK, RIM, URWEGO OPPORTUNITY BANK, VISION FINANCE, Umurenge SACCO and insurance companies.

3.8.2.5 Cooperatives

Ngarama urban center has not left behind in domain of cooperatives. From data collection Ngarama urban center has 5 cooperatives with ‘’ubuzima gatozi’’ and 7 cooperatives not yet recognized officially by the law. The respondents from surveying cited the advantages of cooperative. The common advantages were:

- Getting loan and reduce poverty in the population;

- Raising the understanding of the population towards socio-economic activities;

- Sustaining government programs

Table 13: cooperatives in Ngarama urban center

| COOPERATIVES WITH LEGALLY ACCEPTED |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Cooperative | Activity | Cell | Village |
| 1 | COVEPANG | Selling non cash crops | Ngarama | Insinzi |
| 2 | CODEN | Livestock firming | Ngarama | Gatungo |
| 3 | NTUKIHEBE | Maize crop firming | Karambi | Karambi |
| 4 | COVICISIVINGA | Stocking and selling | Ngarama | Amahoro |
| 5 | UMOJA | Carpentry and trading | Ngarama | Ibare |
| COOPERATIVE ILLEGALLY FUNCTIONAL |  |  |  |  |
| 1 | Inganji idategwa | Maize firming | Kigasha | Akagarama |
| 2 | COPEDURWA | Maize firming | Kigasha | Rwangingo |
| 3 | COPAMCYA | Maize firming | Bugamba | Kajevuba |
| 4 | UMURAVA | Soy firming | Ngarama | Ibare |
| 5 | ABUNGANIZI MU MIHIGO |  | Garama | Urukundo |
| 6 | COMONGA |  | Ngorama | Insinzi |

Source: sector Agronomist officer, Ngarama

Cumulatively, from all the economic activities conducted in Ngarama, ranging from agriculture, employment, business, light industries to modest activities like bicycle transport; it was recognized through socio-economic survey that majority of the people earn a maximum of 30,000 (33.3%) Rwf. while another sizeable percentage generate up to Rwf. 21,000 (22.2%) (chart 9), less people are gaining more than 150,000Rwf/month (3.7%). This information vis-à-vis the population data will help in making proposals such kind of housing typology for implementation upon completion of the elaborated LUDP of Ngarama urban center. The income of the majority of the people is 1000 Rwf/day. This shows that the great part of the study area is rural zone.

Chart 6: Average monthly income

Source: field data, V.COLtd, 2015

Chart 7: Monthly expenditure

Source: field survey, V.COLtd, 2015

The majority of the people expense between 22,000 - 30000Rwf (44.45%); another sensitive group (29.63%) expenses 21,000 Rwf in maximum; only 7.4% of people expense more than 31,000Rwf.

## EXISTING INFRASTRUCTURE ASSESMENT

## Introduction

The following chapter delivers a detailed assessment of the several infrastructural systems of Ngarama urban center. Among them are: health, education, administration, roads, water, electricity and storm water management.

    - Education

The education system in the study area comprises 3 stages, nursery, primary and secondary schools.

    - Nursery education

In general nursery education in the study area, is characterized by lack of recreational infrastructure which can generate  and facilitate the better education environment as it is defined by the law N°48/01 du 10/08/2009 related to educational environment.

The attitude of Some parents towards nursery education is still  perplex for interests of their children , forgetting that the knowledge given to children is a basic material to go ahead with primary school without. Some parent is also unable to pay for school fees to their children given that nursery schools are sponsored by parents themselves to pay for teaching staff.  Ngarama sector has about 16 nursery schools, 16 teachers, 455 students among them 230 are male and 225 female, as shown in the table below:

Table 14: Primary school ownership

| No | Names of schools | Cell | ownership |
| --- | --- | --- | --- |
| 1 | Ngarama ADEPR | Ngarama | Private |
| 2 | NPE | Ngarama | Private |
| 3 | Immaculee conception | Ngarama | Private |
| 4 | Amahoro | Ngarama | Private |
| 5 | Kiyovu | Kigasha | Private |
| 6 | Bushyanguhe | Kigasha | Private |
| 7 | Gatubgo | Ngarama | Private |
| 8 | CECA Nyarurambi | Kigasha | Private |
| 9 | Murama | Nyarubungo | Private |
| 10 | Ruhengeri | Nyarubungo | Private |
| 11 | Rushenyi | Karambi | Private |
| 12 | Mishenyi | Karambi | Private |
| 13 | Cyankondo | Bugamba | Private |
| 14 | Kinihira | Bugamba | Private |
| 15 | Kinyinya | Bugamba | Private |
| 16 | Akajevuba | Bugamba | Private |

SOURCE: Education officer Ngarama sector, 2015

The general strategy   adopted by officials is to approach parents and sensitize them about major interests of nursery education. All nursery schools are under parents the government helps in preparing curriculums.

Figure 6: Nursery school

SOURCE: Feld photo, V.COLtd,  2015

Primary education

The primary education schools of Ngarama study area are concentrated in the urban center. Ngarama has five primary schools, 79 rooms, 118 teachers, 7182 students, 3360(46.78%) male and 3822(53.22%) female. The rate of qualification at primary level is 100 %( A2). From field data, it was found out that 60% of the school attended is public schools while 40% are private schools or subsidiary schools. The average  density of students in a class is high( 90 student per class) .As the number of students at primary level increase annually so that the construction of others rooms to reduce density is necessary.

Chart 8: Primary school ownership

Figure 7: Primary school and secondary school ADEPR Ngarama

Secondary education

The Ngarama study area has 4 secondary schools, I subsidiary and 3 public. The numbers of teachers is 53; number of students is 1666; 747 (44.84%) male and 919(55.16%) female and the number of class is 37. The average number of student per class is 45 at the secondary school level the situation is normal in terms of density in class. In total Ngarama sector has 9625 students among them 4528 male and 5097 female; 187 teachers and 132 class rooms with average number of student of 72 per class. This show that others rooms are necessary to be constructed principally at primary level. This implies need of school infrastructure extension. The new planning will take into consideration that issue

Working distance to education infrastructure

According to the respondents the results show that working distance to education infrastructure is : 8%  of respondents are doing between 0 and 500m to attend school infrastructure, 44%  between 600-1000m; 8% between 1000-15000m; 11% between 1600-2000m; 15% between 2100-3000 and 15% between  as shown to the chart below

Chart 9: Walking distance to education infrastructure

The survey shows that more than 40% percent are doing more than 2km to attend

School. The analysis shows that the density of schools has to increase to reduce walking distance and facilitate students to attend schools easily.

## 4.3 Health facilities

There is one public health center in the study area specifically Ngarama Health Centre located at Ngarama urban center, one Hospital and 3 health posts. These serve the whole sector and some cells of Gatsibo and Nyagihanga sector. The initial assessment shows that the facilities in this health center are inadequate. The Local Development Plan process will produce the optimal location of support health center based on population distribution in the area of study.

Figure 8: Ngarama Hospital

Ngarama Hospital and Ngarama health center

Ngarama Hospital was constructed in Ngarama sector, Gatsibo district in 1980, it actualy serves 149, 911 population from 8 health centers of Gatsibo District. The hospital uses 9 doctors, 56 nurses and 53 others staff. Base on the data below, one doctor serves 16 657 people, 0ne nurse serve 2677 people. The hospital shall need extension of building facilities considering the frequencies of patients (2000patients/month) to the hospital and the growth of population.

Ngarama health center is working in the same area with hospital so that the climate is the same. This health center covers the cells of Ngarama; Nyarubungo; Bugamba; Kigasha; Karambi; Nyabicwamba; Mayange; Nyagitabire, Gitinda;  and Kibare with served population of  47,377. The staff is composed of 14 nurses 18 supporting staff. The health center facilities ae planned to be constructed while today is working in hospital enclosure.

The frequent diseases observed (updated data) are:

- Malaria (8.8%)

- Diarrhea

- Gastrite urinaru infection (13%)

- Accidents (6%)

- Tuberculosis

- Others..

The reason for this frequency is; Stagnant waters dues to the flat terrain of the area, critical sanitation in some households, the local beer mixed with chemical sugar and produces uncontrolled alcohol or other harmful chemical products to human body.

Walking distance to health facilities

According to the respondents the results show that working distance to attend health facilities is : 7.4%  of respondents are doing between 0 and 500m to attend health facilities, 33.3%  between 600-1000m; 22.2% between 1000-15000m; 7.4% between 1600-2000m; 25.9% between 2100-3000m and 3.7% between  3000-Plus as shown to the chart below:

Chart Chart10: Walking distance to health facilities

Map

Map 16: Health and education facilities distribution in the study area

Source: field data and GIS Mapping

  - Roads and Transport

The main highway from Kayonza to Nyagatare in Rwanda/Uganda border which is passing far from the Ngarama urban center (km) . the urban center of Ngarama   has the district roads connected to the National road Kayonza Kagitumba and that road crosses ngarama urban center. The challenge is that the road is not tarmacked so that the transport is sometimes inhibited frequently in rain season. The others roads connecting cells in the study area are murram and not well maintained. If the urban a lot of changes will happen and the development of touristic zone shall be facilitated by construction of new modern roads the center is well developed to meet the national

Figure 9: Status of Road Kabarore – Ngarama

Standards as provided by the guidelines in the LUDP.  The transportation in Ngarama is a challenge so that movement of population is still limited to few people.

Map 17: Existing road network in Ngarama urban center

In terms of road network, the site has national roads and feeder roads networks with distance of 14730.7m for national road and 49591 m for other road. The site does not have tarmac road and the many roads are found in Ngarama urban center.

  - water supply network

About portable water, the urban center of Ngarama has the same challenge like many other towns. Regardless of major effort undertaken by the government to install water network and water points, some tanks and water taps are empty; there is shortage in the water network.  the residents still have to pay  expensively to access water frequently in dry season.

As shown in chart 8, 91% of those interviewed use piped water for their domestic use while 7% rely on rain-water and 2% elsewhere. Although rain-water harvesting is an initiative being advocated for by the Government, most residents are yet to embrace this idea mainly because of high cost of installing the necessary infrastructure i.e. water tank and piping

Chart 11: Sources of drinking water

Chart 12: Distance to the water source

Map 14 shows the water distribution network of Ngarama urban center. It is clear that there is one major line of main water pipe running along the district roads and Water reservoirs which are used for storage before water is dispersed to other areas downhill by gravity. Most of residents of Ngarama study area are fetching water in 500m (80%). There are no people who are using more than 2000m to get to the water source.

## Energy for cooking and lighting

The main source of energy for cooking in the case study area is firewood (76%) and second source is charcoal (24%). This is because majority of the people live in periphery of the urban center where firewood is readily available from the small forests and shrubs.

Chart 13: Energy used for cooking

Source: Field data, 2014.

Due to the mainly subsistence form of agriculture that is practiced in the area, few people use biomass to generate energy for their cooking needs. This situation shall have negative impact to the environment where trees are destroyed constantly. Measure should be taken to overcome the situation before meeting consequences.

Chart 17: Energy challenges

Source: field survey data, 2015

From the respondents, 61% say that the challenge to get electricity in their house is the cost of service, 28% lack of connection to electricity network, 10% interruption or unstable electricity and 5% said nothing.

Figure 10: Electricity network

Figure 11: electricity connection to the households

Source: field photos, 2015

Map 18: Water distribution network

Source: GIS Mapping, V.coLtd, 2015

Table 15: estimated electricity length in the study area

| Lot_Name | Distance |
| --- | --- |
| HIGH VOLTAGE | 4799.857 |
| LOW VOLTAGE | 9340.601 |
| MEDIUM VOLTAGE | 6275.98 |

Table 16: Electricity and water points

| Type | Number |
| --- | --- |
| Fontain | 5 |
| HV Transformer | 3 |
| MV Transformer | 2 |
| Power Industry | 4 |
| Transformer | 1 |
| Waste Disposal | 1 |
| Water (not in use) | 4 |
| Water tank | 14 |
| Water tap | 12 |

## 3.13. Water drainage system

Owing to deficiency of water drainage channels in Ngarama urban center and the erratic weather conditions, there is a lot of surface run-off generated in rain season. This water causes a lot of damage to roads during the rainy seasons. Consequently, there are many environmental effects associated with water e.g soil erosion, degradation of infrastructure especially earth and murram roads and flooding Along the existing road network the local authority has tried to construct drainages to limit the problems produced by surface runoff as shown on the  Figure12.

Figure 12: Storm water

Along main murram roads Kabarore- Ngarama, servicing of water channels is available but routine maintenance is necessary to enable efficient flow of rainwater and thus avoid flooding menace and disruption of infrastructural services during rainy seasons. It should be noted that majority of the people still use pit latrines in rural areas whereas in urban areas toilets connected to septic tanks. There is no sustainable sewerage system in the urban center. Flooding during rainy season may cause spill-over of human waste from these facilities and this may result in outbreak of water-borne diseases.

Figure 13: Drainage channel in bad condition

Source: field photo, V.COLtd, 2015

# LAND AND HOUSING OWNESHIP

## Introduction

Land and housing are the two making components in the process of preparing a detailed local development plan of any urban environment. The existing land-use, settlement patterns and predominant house typologies play a key role in proposing and providing guidance to housing development. This chapter therefore provides a detailed analysis of land and housing patterns in Ngarama urban center including the existing settlement patterns, kind of houses, wall and roofing materials, cost of building.

### Land-use and settlement outlines

Majority of the population in Ngarama live in the rural areas, in the traditional isolated or grouped estates scattered in the rural landscape where agriculture is the mainstay. Secondly, infrastructural development has attracted some residents to live along transport corridors.

As a effect, two settlement patterns emerge in Ngarama: linear pattern along roads e.g. Kabarore-Ngarama murram road and others roads in the study area and agglomerated settlements in areas served by infrastructural services such as water networks, health facilities, schools, electricity and accessible roads. Another important factor is its proximity to fields for farming or areas suitable for cultivation. Isolated settlement structure with satellite residential areas are also found in the urban environs.

Map 19: Scattered settlement along the main roads

Source: field photos, V.COLtd, 2015

The introduction of individual ownership of land, has led, in many areas, to an even more scattered settlement pattern as many people have started building individual houses on their own plot agri-residential. Commercial areas are found at the CBD

where retail shops, banks, boutiques and other commercial services are to be found.

## Land and house ownership

Majority of the respondents clearly indicated that 81% of the resident owns the land in which the household lives in whiles the remaining 14% of the respondents are either renting the house or live on the land for other purposes 5%

Chart 14: Land ownership

Source: field data, 2015

Map 20: Existing Land Use

Table 17: exiting land use coverage

| Land use | Number | Area_Ha |  | % |
| --- | --- | --- | --- | --- |
| Administration |  | 3.08 | 2175.021 | 0.14 |
| Agriculture |  | 1308.13 | 2175.021 | 60.14 |
| Church | 13 | 5.19 | 2175.021 | 0.24 |
| Cemetery | 1 | 4.39 | 2175.021 | 0.20 |
| Commercial | 6 | 7.12 | 2175.021 | 0.33 |
| Dairy | 1 | 0.14 | 2175.021 | 0.01 |
| Education | 9 | 19.08 | 2175.021 | 0.88 |
| Forest |  | 185.54 | 2175.021 | 8.53 |
| Health facilities | 1 | 7.29 | 2175.021 | 0.34 |
| Industry | 1 | 1.68 | 2175.021 | 0.08 |
| Butcher | 1 | 0.24 | 2175.021 | 0.01 |
| Market | 1 | 0.30 | 2175.021 | 0.01 |
| Military Camp | 1 | 31.36 | 2175.021 | 1.44 |
| Mixed Land use (Residential and agriculture | 1 | 51.91 | 2175.021 | 2.39 |
| Mosque | 1 | 0.11 | 2175.021 | 0.00 |
| Ngarama Stone | 1 | 3.80 | 2175.021 | 0.17 |
| Playground | 1 | 0.17 | 2175.021 | 0.01 |
| Power industry | 1 | 1.89 | 2175.021 | 0.09 |
| Prison | 1 | 3.67 | 2175.021 | 0.17 |
| Residential |  | 376.02 | 2175.021 | 17.29 |
| Swamp |  | 163.91 | 2175.021 | 7.54 |
|  |  | 2175.02 |  |  |

## Existing housing typologies

Most of the people in Ngarama urban center live in bungalow and maisonettes houses which range between two bedrooms to four bedrooms. The houses are mainly constructed using mud bricks and corrugated iron sheets for the roof, while the floor is mainly mud mortar or cemented.

Figure 14: Housing typologies

Source: field photos, V.COLtd, 2015

Even though residents of Ngarama study area have shown great effort in improving their housing conditions as shown in photos   (New bungalows in Ngarama CBD), are being constructed illegally with mud brisks (RUKARAKARA) because it is not only cheap but is also quick.

Figure 15: New constructed houses

Source: Field photo, V.COLtd, 2015

Regardless of the efforts being done by the resident in housing construction ,Ngarama urban center area and its peri-urban area still facing a challenge of informal houses characterized by dilapidated houses, lack of access roads and poor drainage and sanitation still remains a challenge.

On construction, majority of residents (59%) spent over 500,000 while 25% spent between Rwf 500,000-1,000,000; few people use more than 2,000, 000 for constructing their house. This justifiable because the majority of house are constructed with RUKARAKARA and don’t; have cement pavement

Chart 15: The estimate cost for constructing a house

Source: Field survey data, V.COLtd, 2015

Chart 16: Construction material for walls   Chart 17: Construction material for the roof

Source; Field survey data, V.COLtd, 2015

Chart 18: Number of rooms per house

Source: Field survey data, V.COLtd, 2015

The common material for building house walls are earth bricks (chart 21) while roofing is mainly done using iron sheets. As shown in chart….., most houses have between 1 and 3 rooms while others have between 5 and 7  (chart 22 ). Chart 22 shows that majority of the resdients of Ngarama prefer to live in bungalow-type (56%); maisonette (17%); detached house (5%); apartment(3%) and semi-tetached houses( 9%).

Chart 19: Preferred house to live in

Chart 20: Parcels dimensions

Source: field data, 2015

According to the respondent, the lands on which houses are constructed have various chape and surface area. Most of the have parcels with surface area ranging between 300m2 and 700m2; there also people who do not have parcels (8%).

## Sanitation analysis

## Sanitation behavior in Ngarama Urban Centre

Similar to other developing urban centers and towns in Rwanda, Ngarama urban center is facing a rapid urbanization development that requires environmental intervention plan, to take control of the environmental issues.

An increase in housing development in Ngarama urban area has unavoidable increased pressure on urban infrastructure and services, much of which have no sustainably been provided to jumble through with rapid urban growth; therefore, resulting to poor urban service delivery including uncollected solid waste in most of the residential and commercial units. As shown on photo 7 , the social economic analysis conducted confirmed that waste is great challenge to the residents of Ngarama were disposal methods are done in different ways by the residents. This is contributed by lack of a proper sanitary landfill in the area.

To the one part, solid waste has high capability of menacing and degrading environmental resources e.g. polluting air through noxious smell, polluting surface and underground water through leakage of dumped and decayed wastes if not properly managed and above all frightening investments in settlements.

In order to deal with the challenge of solid waste management a sanitary landfill is required for Ngarama urban center  in order help the entire area and V.COLtd consultants together with the District environmental expert will choose a proper site and provide a proper guidelines on how the site will be selected based on environmental guidance. For liquid waste most of the household’s uses pit latrine and waste water from domestic use are disposed to the environment either through septic tanks or Soak pit.

## Storm water management

Storm water management is of great concern in any developing city since it takes into the account of the collection of all water surface run-offs. Ngarama urban center has a good topographic terrain that support development of the infrastructure services such as roads, buildings among others. However, in the case of Ngarama urban center only a few roads are serviced by drainage channels while others are faced by heavy damage of storm water discharges from buildings and collected water service into specific channel according to the topography of the site due to lack of proper drainage channels

## CHAPTER FOUR: PROPED PLANNING SITUATION

4.1. Proposed layout plans

Map 21: Proposed Layout plan of Ngarama

Map 22: Layout plan of Gatungo site

Table 18: Plot categories and size

| Plot category | Size | Number/Site |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ngarama | Gatungo | Total |
| High Density | 14 X 20 | 1839 | 179 | 2018 |
| Medium Density | 20 X 25 | 613 | 84 | 697 |
| Low Density | 20 X 30 | 535 | 105 | 640 |
| Mixed Use |  | 792 | 178 | 970 |
| Total |  | 3779 | 546 | 4325 |

Source: V.COLTD, 2015

## Detailed zoning

### Purposed of zoning regulations

This chapter lists down the definition of the terms used in the Zoning Plan. Illustrations are also added to ensure clarity of the definition.

“Building” means any covered and enclosed structure with one or more floors that is permanently affixed to the land and is accessible to humans.

“Attached Building”

Is a building that abuts on two side of the plot line or building on an adjacent plot and for which no side setback is provided.

Figure 16: Detached building presentation

“Semi-detached Building”

Is a building that abuts on one side of the plot line or building on an adjacent plot and for which no side setback is provided.

Figure 17: semidetached building presentation

“Detached Building”

Is a freestanding building that does not have any other building or adjacent plot and for which open space is provided around all sides of the building and the plot lines.

Figure 18: Attached building presentation

“Building Coverage” means the percentage of the plot area occupied by the ground area of the primary and all ancillary buildings on such plot, inclusive of the shadow area created by cantilevered building projections, but do not include the following:

• Bay windows with a projection of 0.5m or less.

• Roof eaves and sun shading projections.

• The shadow area of a building that is from the 3rd storey and above (to encourage viable landscaping at the ground level and shaded communal spaces, and promote building articulation and a variety of architectural designs).

Computation of building coverage shall include all existing developments within the plot.

Figure 19: Building Height

Building Height” means the overall height of a building measured from grade or access road to the top of the last storey or the highest point of the building, but does not include:

• External parapets not exceeding 1.5m

• Lift overruns;

• Antennae;

• Rooftop M&E service rooms and structures.

• Any other permitted structures as may be allowed by One Stop Centre from time to time.

Along the slopes or along contoured topography the building height is measured as illustrated in the figure below.

Figure 20: Building Setback

“Building Setback” means the minimum distance (in plan) by which a building must be offset from the plot line, except for any permitted intrusions or structures.

“Front Setback”

Means the minimum required setback as measured from the plot line fronting any road to the external main wall of any primary or ancillary building.

“Rear Setback”

Means the minimum required setback as measured from the plot line directly opposite of the front plot line.

“Side Setback”

Means the minimum required setback as measured from the plot line that extends between the front and rear plot lines.

“Apartments” means a multistory residential building within a building plot that contains three or more residential units that are capable of strata-subdivision.

“Floor Area Ratio (FAR)” means the gross floor area of the building or buildings on any plot divided by the plot area.

“Gross Floor Area (GFA)”means the sum of the gross horizontal areas of all the floors of building, measured from the exterior face of exterior walls or mid-point of common or party walls. The "floor area" of a building shall include basement floor area, staircase blocks, planter boxes and ledges, public areas such as landings, and common lobbies. It shall exclude floor area used for parking facilities. Basements not utilized for any habitable or commercial purposes shall be exempt from gross floor area calculations. M&E floors with 1.5mor less headroom can be excluded from gross floor area computation. Balcony& Bay Window areas are excluded from the GFA calculation, subject to the condition that all the Balcony and Bay Widow area does not exceed 10% of overall GFA quantum of the building Lift shafts and service ducts including the thickness of the walls are counted as GFA once at the ground storey level.

Any existing building not affectedly the new development should clearly be stated in the existing gross floor area. Calculations of gross floor area for any development shall include the GFA of all existing developments within the plot.

“Plot Area” means the total horizontal area (in square meters) included within the Plot boundary lines.

“Plot Coverage” means the percentage of a Plot area occupied by the ground area of primary and all ancillary buildings, structures and driveways, aisles and parking spaces.

“Landscape Coverage” means the percentage of plot area covered by permeable surfaces and meant for aesthetic landscaping and the planting of grass, shrubbery, and trees but may contain such impermeable surface areas that are used as communal gathering spaces, or for landscaping features such as fountains, furniture or shade pergolas.

“Single-Family Housing” means a residential building containing only one dwelling unit and occupied by one family.

“Public Facilities” means all the publicly accessible uses like recreational spaces, religious, cultural, social, educational and health-related uses.

## Ngarama town Zoning Map and interpretation

The Zoning Map indicates the specific boundaries of the various proposed zoning areas for Ngarama center. These boundaries are defined and demarcated based on the following principles:

• Safeguard and respect the existing physical character of the local area by grouping developments which are similar or complementary in terms of use and scale, into a single area.

• To ensure that there is no conflicting uses in the neighboring zoning areas.

## Identification of Zoning Categories for Ngarama Centre

The following are the various zoning categories and sub-categories proposed for

Ngarama town:

#### Residential

- High Density Residential (Apartment)

- Medium Density Residential

- High Density Residential Zone (Rural Residential Zone)

- Low Residential Zone

#### Commercial

- Commercial District (including mixed use)

#### Industrial

- Light Industrial Area

#### Parks and Open Space

  - Passive Recreational Areas

  - Agricultural Area

  - Protected Are

### Proposed Zoning (land use) Map of Ngarama town

Map 23: Proposed land use for entire area

SOURCE: V,COLdt, 2015

Map 24: Proposed land use of Gatungo site

Map 25: Proposed land use for Ngarama site

Table 19: Land use types in Ngarama urban center

| Land Use Type | Area(ha) |
| --- | --- |
| Residential High Density | 219.56 |
| Residential Low Density | 49.68 |
| Residential Medium Density | 89.91 |
| Commercial | 41.39 |
| Market | 2.62 |
| Mixed Use | 60.25 |
| Hotel | 1.82 |
| Community Hall | 3.66 |
| Educational | 27.43 |
| Civic Facilities | 2.24 |
| Religious | 7.77 |
| Health Facility | 9.94 |
| Sports and Recreation | 8.51 |
| Stadium | 2.67 |
| Open Space and Park | 16.11 |
| Tourism | 13.32 |
| Bus Parking | 0.71 |
| Cemetery | 12.88 |
| Military Camp | 31.36 |
| Prison | 5.50 |
| Greening | 1.66 |
| Light Industrial | 9.43 |
| Urban Agriculture | 1123.65 |
| Forest | 163.08 |
| Wetland | 162.49 |
| Roads | 105.40 |
| Landfill | 1.20 |
| Wastes Collection Points | 1.08 |
| Sum | 2175.34 |

Source: V.COLTD, GIS DATA, 2015

Table 20: Land use types in priority site

| Land Use Type | Priority Sites/Area(ha) |  |  |
| --- | --- | --- | --- |
|  | Ngarama | Gatungo | sum |
| Residential High Density | 61.44 | 7.42 | 68.86 |
| Residential Low Density | 33.79 | 6.33 | 40.12 |
| Residential Medium Density | 31.15 | 5.07 | 36.22 |
| Commercial | 17.78 | 6.28 | 24.06 |
| Market | 1.45 | 1.16 | 2.62 |
| Mixed Use | 47.79 | 9.22 | 57.01 |
| Hotel | 1.82 |  | 1.82 |
| Community Hall | 1.35 | 2.31 | 3.66 |
| Educational | 14.65 | 2.12 | 16.77 |
| Civic Facilities | 2.24 | 0.40 | 2.64 |
| Religious | 6.22 | 1.36 | 7.58 |
| Health Facility | 8.63 | 1.32 | 9.94 |
| Sports and Recreation | 3.35 | 1.88 | 5.22 |
| Stadium | 4.70 |  | 4.70 |
| Open Space and Park | 6.17 | 1.53 | 7.71 |
| Tourism | 4.75 | 8.57 | 13.32 |
| Bus Parking | 0.71 |  | 0.71 |
| Cemetery | 4.39 |  | 4.39 |
| Prison |  | 5.50 | 5.50 |
| Greening | 0.80 | 0.74 | 1.54 |
| Light Industrial | 1.33 | 6.61 | 7.93 |
| Urban Agriculture | 64.40 | 10.79 | 75.19 |
| Forest | 17.51 | 6.60 | 24.11 |
| Wetland | 0.49 |  | 0.49 |
| Roads | 63.20 | 17.60 | 80.80 |
| Landfill | 17.30 |  | 17.30 |
| Wastes Collection Points | 0.64 | 0.24 | 0.88 |
| Sum | 418.06 | 103.04 | 521.10 |

Source: V.COLTD, 2015

### Building guideline zoning regulation residential

The regulations and guidelines in this section apply to different uses within the various zones in Ngarama town. Summary of some zoning guidelines, with an illustrative simulation are provided below

#### low density

General Description: The Single Family Residential District (R1) is intended for high-end villa housing and complementary public facilities as needed. The minimum lot areas in this zone are larger to distinguish the area as a low density good class residential neighborhood with a spacious character. The purpose is to create a high-end single family housing area, characterized by large plots, lush greenery and quality housing:

Table 21: Low density zone planning guidelines

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | Single family houses (detached houses) |
| Prohibited uses | - Residential apartments - General commercial use - Industrial uses - Major infrastructure |
| Conditional uses | - Standalone food outlet with less than 50 seats - Hotels (incl. its ancillary uses) - Public facilities with sound proof - Convenience stores not exceeding 60m2 of floor area - Home Office - Schools - G+1 and semi-detached houses with the same facilities as single residential houses |
| Ancillary uses | - Car parking garage - Guard house - Maids’ quarters - Outdoor kitchen - Store rooms |
| Minimum lot size | 600 m2 |
| 2.0. Coverage |  |
| Maximum Building Coverage | 40% Maximum |
| Minimum Landscaping Coverage | 20% Minimum |
| Maximum Floor Area Ratio (FAR) | 0.8 Maximum |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+1+P (villa) - G (ancillary buildings) |
| Floor to floor height | 4 m Maximum |
| Building Form | - Detached villas only - Aesthetic enhancement of the front elevation of the building |
| Building Setbacks (Minimum) |  |
| Building (Above Grade) | - Front: 7 m (min) - Side: 5 m (min) - Rear: 5 m (min) |

#### High density zones

General Description: The High Density Zone is a residential area for low rise developments. The zone is intended to offer high-end low rise housing and complementary public facilities as needed. The minimum lot areas in the zone are smaller than that found in Low density zone in order to distinguish it as a higher density low rise residential neighborhood with shared facilities and to promote condominium developments. The purpose is to create a high-end mix single family area in the prime areas of the town by allowing for intensification of the apartment housing development.

Table 22: High Density Zone planning guidelines

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Low rise apartment (G+2), townhouse & cluster bungalow development - Complementary clubhouse for housing cluster |
| Prohibited uses | - Residential apartments exceeding 3 storey height - General commercial uses - Industrial uses - Major infrastructure |
| Conditional uses | - Standalone food outlet with less than 50 seats - Hotels (incl. its ancillary uses) - Public facilities - Convenience stores not exceeding 60m2 of floor area - Home Office - Schools |
| Ancillary uses | - Car parking garage - Guard house - Maids’ quarters - Outdoor kitchen - Store rooms |
| Minimum lot size | 280 m2 |
| 2.0. Coverage |  |
| Maximum Building Coverage | 50% Maximum |
| Minimum Landscaping Coverage | 20% Minimum |
| Maximum Floor Area Ratio (FAR) | 1.0 Maximum |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+2+P - G (ancillary buildings) |
| Floor to floor height | 4 m Maximum |
| Building Form | - Detached villas - Semi Detached villas - Aesthetic enhancement of the front elevation of the building |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | Detached Buildings - Front: 7 m (min) - Side: 3 m (min) - Rear: 3 m (min) Semi-Detached Buildings - Front: 7 m (min) - Side: 3 m (min) - Common Side: 0 m - Rear: 3 m (min) (Corner plots require Side setback of 3m min) |

      - High Density Zone (Rural Residential)

General Description: The Rural Residential zone is a residential district offering semi-detached family low cost housing in the rural areas. The zone is intended to offer low cost housing as part of the farming community and complementary public facilities as needed. The purpose is to create a pleasant single-family area in the rural areas of the town in form of grouped settlements.

Table 23: High Density Zone (Rural Residential) planning guidelines

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Semi-detached family houses (all types) |
| Prohibited uses | - Residential apartments - General commercial uses - Industrial uses - Major infrastructure - Any high rise houses (of all types) |
| Conditional uses | - Standalone food outlet with less than 50 seats - Public facilities - Convenience stores not exceeding 60m2 of floor area - Churches - Schools |
| Ancillary uses | - Car parking garage - Outdoor kitchen - Store rooms |
| Minimum lot size | 200 m2 |
| 2.0. Coverage |  |
| Maximum Building Coverage | 40% Maximum |
| Minimum Landscaping Coverage | N/A |
| Maximum Floor Area Ratio (FAR) | 0.8 Maximum |
| 3.0. Building area |  |
| Maximum Number of Floors | - Semi-Detached (4 in one) - G (ancillary buildings) |
| Floor to floor height | 4 m Maximum |
| Building Form | - Detached - Semi Detached villas - Attached Villas Additions and alterations necessary to retain an existing approved legally non-conforming building and its use in good order and repair will be allowed, subject to: - No material change in use which does not comply with the prevailing zoning regulations; - No further increase in floor area of the existing nonconforming use |
| 4.0. Building Setbacks (Minimum) |  |

### 4.2.5.4 Medium Density zone

Table 24: Medium Density zone planning guidelines

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Semi-detached Low rise apartments - Single family houses (Bungalows) |
| Prohibited uses | - General Commercial uses - Industrial uses - Major infrastructure - High rise apartments and houses exceeding 2 floors - Petrol stations |
| Conditional uses | - Standalone food outlet with less than 50 seats - Hotels (incl. its ancillary uses) - Public facilities - Shop within apartment complex not exceeding 60 m2 of floor area - Home Office - Single family houses - Schools |
| Ancillary uses | - Car parking garage - Guard house - Maids quarters - Outdoor kitchen - Store rooms |
| Minimum lot size | - 500m2 - Plots less than 500 m2 can be developed as single family houses (for consideration by OSC) |
| 2.0. Coverage |  |
| Maximum Building Coverage | 65% Maximum |
| Minimum Landscaping Coverage | 15% Minimum |
| Maximum Floor Area Ratio (FAR) | 1.4 Maximum |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+1 maximum - G (ancillary buildings) maximum |
| Floor to floor height | 3 m Maximum |
| Building Form | - Detached villas - Semi-Detached low rise - Attached villas |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | - Front: 5 m - Side: 3 m (min) - Rear: 3 m (min) Setback for plots smaller than 600m2 (single family houses) - Front: 3m - Side: 2 m (min) - Rear: 2 m (min) |

  - 4.2.5.5.  Zoning Regulations – Commercial

The regulations and guidelines in this section apply to commercial uses within the various Commercial zones in Ngarama town. A summary of some commercial zoning guidelines are detailed below;

Mixed Use commercial district

General Description: The Mixed Use Commercial zone is a zone established to create housing development with commercial development at the ground level to ensure continuity in the ground level commercial activities. The area offers spaces for goods and services as well as living quarters to create a vibrant mixed use commercial zone.

Table 25: Mixed Use Commercial zone guidelines

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Shopping Centres - Offices - Hotels - Serviced apartments - Leisure and entertainment centres (e.g. cinemas ,bowling alleys) - Galleries - Commercial schools - Multi-family Housing Multi-family residential apartments |
| Prohibited uses | - Industrial Uses - Major Infrastructure Installations |
| Conditional uses | - Public Facilities - Hotels - Service apartments - Petrol stations |
| Ancillary uses | - Electrical substation (ESS) - Refuse area |
| Minimum lot size | - 750 m2 |
| 2.0. Coverage |  |
| Maximum Building Coverage | 85% Maximum |
| Minimum Landscaping Coverage | 5% Minimum |
| Maximum Floor Area Ratio (FAR) | 3 Maximum |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+3 (apartments) maximum - G (ancillary buildings) maximum |
| Floor to floor height | 4 m Maximum |
| Building Form | - Attached buildings - Apartments |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | - Front: 0 m - Side: 0 m(min) for 6m from the front setback line, beyond 6m from of the front setback line the setback shall be 3m (min) - Rear:2 m (min) - For plot size below 250 m2 - Front: 4 m (min) - Side: 0 m(min) for 6m after the front setback, - beyond 5m from of the building frontage setback line the setback shall be 2m (min) - Rear: 2m (min |
| Loading docks | - Loading docks, if any, shall be located to the rear or side of the building. - Where the rear of the building faces residential uses, the loading area must be appropriately screened so as not to be visible from residential uses |
| Parking |  |
| Location | Required parking stalls shall be provided within the lot boundary |
| Surfacing | Parking shall be hard surfaced and provided with adequate drainage |
| Minimum Required Parking Stalls | - Commercial – 2 Parking Lot for the first 200 m2 commercial space and 1 Parking Lot per subsequent 200 m2 commercial space shall be provided - Apartment – 1 Parking Lot per 2 dwelling units - Food Outlet -1 Parking Lot per 10 seats shall be Provided - Hotel - 1 Parking Lot per 200 m2 shall be provided |
| Circulation |  |
| Pedestrian | - 2m(min) covered publicly accessible pedestrian walkway/ covered arcade shall be provided along the perimeter of the buildings fronting the main roads - Pedestrian circulation shall link all buildings and amenities on site - Throughout the site development, all pedestrian linkages shall be a minimum 1.5m wide - All pedestrian circulation shall be accessible to the disabled |

  - Zoning Regulations – Industrial

The regulations and guidelines in this section apply to industrial uses within the various Industrial zones in Ngarama town

4.2.5.6. Light industry district

General Description: The Light Industrial District is specialized land areas carefully located close to residential area with easy vehicular access to offer residents proximity to a range of non-polluting industrial services. It is also established to create a clean and light industrial environment that could blend with the surrounding residential area. The zone can have Clean Industries, Business Parks or Light Industries.

Clean Industries: These are industries that do not generate air and water pollution and do not generate noise and smell nuisance which can affect surrounding developments. The factories also shall not use large quantities of hazardous substances such as solvents, acids and other chemicals. No buffer is necessary for such industries.

Light Industries: These industries shall not generate large quantities of trade effluent or solid waste. They shall also not generate excessive impulsive or continuous noise.

They shall also not use large quantities of hazardous substances such as solvents, acids & other chemicals. A buffer of 50m is necessary from the nearest residential district for such industries.

Business Park: These are developments which include non-pollutive industries such as businesses that are engaged in high technology, research and development (R&D) e.g. Business Parks, Science Parks etc. No buffer is necessary for Business Parks.

Table 26: Light Industrial District

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Petrol stations - Car wash - Minor automotive repair shops - Non-pollutive industrial uses - Warehouse - Dry cleaning / laundry service - Business Parks, Science Parks |
| Prohibited uses | - Residential Uses - Major Infrastructure Installations - Churches |
| Conditional uses | - Commercial uses - Sundries / grocery shop - Fast food outlet ( 200 seating) |
| Ancillary uses | - Electrical substation (ESS) - Refuse area |
| Minimum lot size | 250 m2 |
| 2.0. Coverage |  |
| Maximum Building Coverage | 60% Maximum |
| Minimum Landscaping Coverage | 10% Minimum |
| Maximum Floor Area Ratio (FAR) | N/A |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+3 maximum - G (ancillary buildings) maximum |
| Floor to floor height | 4 m Maximum |
| Building Form | Detached Buildings(can also be flatted) or attached buildings |
| Existing buildings | N/A |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | For Detached Development - Front: 7.0 m (min) - Side: 4.5 m (min) - Rear: 4.5 m (min) For Attached Developments - Front: 7.0 m (min) - Side: 0 m (min) - Rear: 0 m (min) |
| Loading docks | - Loading docks, if any, shall be located to the rear or side of the building. - Where the rear of the building faces residential uses, the loading area must be appropriately screened so as not to be visible from residential uses |
| Parking |  |
| Location | - Required parking stalls shall be provided within the lot boundary for I1 developments |
| Surfacing | Parking shall be hard surfaced and provided with adequate drainage |
| Minimum Required Parking Stalls | - Light Industrial Use – 2 Parking Lot per the first 200 m2 shall be provided and 1 Lot per 200m2 subsequently - Light Industrial Use – 1 Lorry Parking Lot per 3000 m2 Commercial - 1 Parking Lot per 200 m2 for Light Industrial - Use (for office space in excess of 25% of the industrial use) |
| Disabled Parking | - Where required, at least 1 parking stalls per public entrance shall be designated for the disabled - The location shall be immediately adjacent to the building entrance/access |
| Circulation |  |
| Pedestrian | - Layout and design shall give due regard for safe pedestrian circulation throughout the site - All pedestrian circulation shall be accessible to the disabled |

### General industry

General Description: The General Industrial zone is specialized land areas strategically located for easy access for heavy vehicles. As it is a zone that can generate noise, traffic and pollutants, it is kept away from residential areas and the commercial zone. It is established to create a consolidated industrial zone for employment in manufacturing and logistics industries, with imposed nuisance buffer for noise and health safety. The zone can have General Industries or Specialized Industries depending on their pollutive nature. Depending on the type of industry, different buffer requirements apply for General Industries.

General Industries which are not so pollutive are required to be 200m away from the nearest residential areas.

Special Industries are more pollutive industries and are required a larger buffer of 2000m from the nearest residential areas. However some more hazardous industries require larger buffer as specified by the national environmental authority.

Table 27: General Industrial Zone guideline

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Petrol stations - Oil Depots - Pollutive industrial uses - Warehouse - Manufacturing |
| Prohibited uses | - Residential Uses - Major Infrastructure Installations |
| Conditional uses | - Religious Facilities - Workers’ Accommodation - Sundries / grocery shop - Fast food outlet ( 200 seating) - Commercial Use |
| Ancillary uses | - Electrical substation (ESS) - Refuse area |
| Minimum lot size | N/A |
| 2.0. Coverage |  |
| Maximum Building Coverage | 60% Maximum |
| Minimum Landscaping Coverage | 10% Minimum |
| Maximum Floor Area Ratio (FAR) | N/A |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+4 maximum - G (ancillary buildings) maximum |
| Floor to floor height | N/A |
| Building Form | Detached Buildings(can also be flatted) or attached buildings |
| Existing buildings | N/A |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | For Detached Development: - Front: 7.0 m (min) - Side: 4.5 m (min) - Rear: 4.5 m (min) For Attached Developments - Front: 7.0 m (min) - Side: 0 m (min) - Rear: 0 m (min) |
| Loading docks | - Loading docks, if any, shall be located to the rear or side of the building. - Where the rear of the building faces residential uses, the loading area must be appropriately screened so as not to be visible from residential uses |
| Parking |  |
| Location | Required parking stalls shall be provided within the lot boundary for I1 developments |
| Surfacing | Parking shall be hard surfaced and provided with adequate drainage |
| Minimum Required Parking Stalls | - General Industrial Use – Parking Lot per the first 200 m2 shall be provided and 1 Lot per 200m2 subsequently - General Industrial Use – 1 Lorry Parking Lot per 3000 m2 - Commercial - 1 Parking Lot per 200 m2 for General Industrial Use (for office space in excess of 25% of the industrial use) |
| Disabled Parking | - Where required, at least 1 parking stalls per public entrance shall be designated for the disabled - The location shall be immediately adjacent to the building entrance/access |
| Circulation |  |
| Pedestrian | - Layout and design shall give due regard for safe pedestrian circulation throughout the site - All pedestrian circulation shall be accessible to the disabled |

  - Zoning Regulations – Parks and Open Spaces

The regulations and guidelines apply to all developments proposed in future in Ngarama town designated by the green areas presented on the Land Use Map.

#### Passive recreational district

General Description: Passive Recreational Zones are zones established to provide recreational and leisure facilities and activities in selected areas that have unique features (including open spaces, environmentally sensitive areas, buffer areas, or along significant routes). These parks can include recreational commercial or public facilities at the neighborhood, community, and regional level.

Table 28: Passive Recreational zoning guidelines.

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Botanical gardens, arboretums and conservatories. - Outdoor recreational facilities, such as hiking and bicycle trails, greens and commons, sitting areas and picnic areas. - Park related public facilities such as public toilet/ changing room. |
| Prohibited uses | - All types of industrial uses - All types of residential uses - All types of commercial uses - All types of major public facilities - Major infrastructure installations - Agricultural activities |
| Conditional uses | - Minor Public Facilities - Restaurants - Small kiosk not exceeding 100 m2 |
| Ancillary uses | N/A |
| Minimum lot size | N/A |
| 2.0. Coverage |  |
| Maximum Building Coverage | N/A |
| Minimum Landscaping Coverage | N/A |
| Maximum Floor Area Ratio (FAR) | N/A |
| 3.0. Building area |  |
| Maximum Number of Floors | - G+1 maximum - G (ancillary buildings) maximum |
| Floor to floor height | N/A |
| Building Form | N/A |
| Existing buildings | N/A |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | N/A |
| Loading docks | N/A |
| Parking |  |
| Location | N/A |
| Surfacing | Parking shall be hard surfaced and provided with adequate drainage |
| Minimum Required Parking Stalls | - Commercial - 1 Parking Lot per 200 m2 - Cultural centres – 1/50 m2 of GFA - Parks and Open Space – 1/1000 m2 of park area. - Tourism and culture related commercial uses – 1/50 m2 of GFA |
| Disabled Parking | - Where required, at least 1 parking stall per public entrance shall be designated for the disabled |
| Circulation |  |
| Pedestrian | - Layout and design shall give due regard for safe pedestrian circulation throughout the site - All pedestrian circulation shall be accessible to the disabled |

#### 4.4.5.8. Agricultural Area

General Description: Agricultural Area strives to protect the viability of agriculture in the center. The Agricultural zone is meant to prevent farmland from being converted from farm uses, to prevent the fragmentation of farms, and to prevent land use conflicts, as well as to protect agricultural producers from non-intrusion into agricultural areas.

Table 29: Agricultural zoning guidelines

| Categories | Regulations |  |
| --- | --- | --- |
| 1.0 Uses |  |  |
| Permitted Uses |  | - Agriculture Farms - Green houses |
| Prohibited uses |  | - All types of industrial uses - All types of residential uses - All types of commercial uses - All types of public facilities |
| Conditional uses |  | - Supporting Agricultural uses - Granaries not exceeding 150 m2 - Infrastructure |
| Ancillary uses |  | - Storage barns - Parking - Store for agricultural equipment, cattle sheds, etc |
| Minimum lot size |  | N/A |
| 2.0. Coverage |  |  |
| Maximum Building Coverage |  | N/A |
| Minimum Landscaping Coverage |  | N/A |
| Maximum Floor Area Ratio (FAR) |  | N/A |
| 3.0. Building area |  |  |
| Maximum Number of Floors |  | G N/A |
| Floor to floor height |  | N/A |
| Building Form |  | N/A |
| Existing buildings |  | N/A |
| 4.0. Building Setbacks (Minimum) |  |  |
| Building (Above Grade) |  | N/A |
| Loading docks |  | N/A |
| Parking |  |  |
| Location |  | N/A |
| Surfacing |  | N/A |
| Minimum Required Parking Stalls |  | N/A |
| Disabled Parking |  | N/A |
| Circulation |  |  |
| Pedestrian |  | N/A |
| Public Transit |  | N/A |

#### Protected Area

General Description: Protected Area is established to conserve and protect the steep slopes, environmentally sensitive areas, nature, wildlife habitat and non-developable lands for other strategic purposes. In the case of Protected Areas like forests and wetlands, the zoning for the protected areas shall supersede.

Table 30: Protected Area

| Categories | Regulations |
| --- | --- |
| 1.0 Uses |  |
| Permitted Uses | - Forests - Wetlands - Rivers |
| Prohibited uses | - All types of industrial uses - All types of residential uses - All types of commercial uses - All types of public facilities |
| Conditional uses | - Infrastructure - Botanical gardens, arboretums and conservatories. - Outdoor recreational facilities, such as hiking and bicycle trails, greens and commons, sitting areas and picnic areas. - Park related public facilities such as public toilet/ changing room. - Minor Public Facilities - Small kiosk not exceeding 100 m2 |
| Ancillary uses | N/A |
| Minimum lot size | N/A |
| 2.0. Coverage |  |
| Maximum Building Coverage | N/A |
| Minimum Landscaping Coverage | N/A |
| Maximum Floor Area Ratio (FAR) | N/A |
| 3.0. Building area |  |
| Maximum Number of Floors | N/A |
| Floor to floor height | N/A |
| Building Form | N/A |
| Existing buildings | N/A |
| 4.0. Building Setbacks (Minimum) |  |
| Building (Above Grade) | N/A |
| Loading docks | N/A |
| Parking |  |
| Location | N/A |
| Surfacing | N/A |
| Minimum Required Parking Stalls | N/A |
| Disabled Parking | N/A |
| Circulation |  |
| Pedestrian | N/A |
| Public Transit | N/A |

## 4.3. Planning principles, Visioning and proposed land-use plans

### 4.3.1 Overview

Urban planning is an activity carried out to guide socio-economic and spatial developments of a planning area. In principle a planning task takes as its prime goal the objective of the people who inhabit the area that is being planned. However, due to the longevity and spatial dispersion of effects of plans, planners should consider the interests of various stakeholders (both current and future) over an area greater than the jurisdiction of a single urban area. The preparation of an urban plan assumes the existence of a hierarchy of development frameworks at national and regional levels. This Guideline is basically limited to urban plan preparation at single urban area level, irrespective of the grade of an urban area

Land use patterns can be viewed as a reflection of various factors including zoning regulations and environmental and social influences. Land use pattern and its changes are determined by both physical factors such as topography, climate, soil types, vegetation, etc. as well as human factors such as population density, technology, infrastructure and institutional reforms.

It has been suggested that urbanization is both a mirror of broad socio-economic changes in society and an instrument of socio-economic change (Chandra, 1996). Other necessary requisites responsible for socio-economic change in society which also serve as catalyst of urban growth include technological change in industry, transportation, communication and building techniques (Bracken, 1981). In this case, rapidly increasing urban population means changes in economic, social, spatial and environmental issues for the betterment of society.

The literature on planning shows that urban areas in various parts of the world have been developing with some guiding principles, though not written in most cases. Planning in its formal sense implies having the power to guide the future development of an area. Hence planning cannot be practiced in a political vacuum. Then, it is logical to conclude that planning has been following the footsteps of political ideals in the determination of the future and the means to achieve it. Plan types, planning methods and principles are ideals and modes for the expression of ideals.

### 4.3.2. Plan Typologies and Key Planning Principles

The literature on planning shows that urban areas in various parts of the world have been developing with some guiding principles, though not written in most cases. Planning in its formal sense implies having the power to guide the future development of an area. Hence planning cannot be practiced in a political vacuum. Then, it is logical to conclude that planning has been following the footsteps of political ideals in the determination of the future and the means to achieve it. Plan types, planning methods and principles are ideals and modes for the expression of ideals.

Principles are general assumptions, fundamental rules, paradigms that guide the spirit of planning policies, proposals, standards and implementation measures. Principles should be based on community values, generally accepted good planning practices, technological level of a community, and planning objectives. Principles serve the preparation of plans but need not be included in the plan itself. Principles should take contextual situations; but there are some basic ones that emanate from higher policy frameworks such as the Constitution, general development plans, federal urban development policy, etc. The following main principles were considered for all cases of this planning study preparation and implementation.

- The health, safety and property rights of citizens, environmental sustainability, efficiency in resource use, and transparency in management of affairs will be promoted with planned development;

- Participatory approach in planning enhances the relevance of development plans and the chance of plans being implemented.

- Identification of stakeholders at an early stage of the plan preparation process will shorten the preparation and approval of plans;

- Involvement of stakeholders in plan preparation increases plan implement-ability;

- All inhabitants of a planning area are stakeholders, albeit at varying intensity and capacity;

- Devising proper methods for public participation promote level of participation;

- Planning should aim towards sustainable development.

- Resources are finite; planned uses promote effective use of resources;

- Plans should promote opportunities for current inhabitants without compromising the need of future generation

- Promotion of public participation in development and equitable distribution of resources promotes the objectives of sustainable development;

- Plans should address the visions and interests of stakeholders at all levels;

- Providing due consideration for environmental issues increases the life span of natural resources;

- Financially feasible development plans are easy to implement.

- Plans should consider financial implications at the planning stage;

- The user-pays principle promotes responsibility; however, the government should support socially and economically disadvantaged section of society;

- The public must benefit from planning gains.

- Spatial integration at national/regional/local levels promotes overall development of society.

- Hierarchy of plans should be consistent with higher level plans/and policies and other related policies in other sectors;

- Integrated planning and implementation economises on resource requirements while at the same time it increases the multiplier effect of positive components;

- Strategic approach in plan preparation increases relevance of urban plans.

- Identification of critical elements during planning enhances the adoption of better-performing plans;

- Identification of critical elements increases public preparedness for unavoidable consequences.

- Urbanization viewed as a potential for development

- Urbanization is an irreversible process, it should be managed rather than try to curb its development;

- Urbanization is positive development for a well-prepared urban administration.

- Policy formulation is iterative.

- Plan preparation and implementation should be a continuous process;

- Shortening the revision time of plans increases the relevance of plans;

- Learning through practice and a reflective-practitioner promotes local contexts;

- Planning is process oriented;

- Proper consideration of plan implementation capacity of institutions is necessary.

- The adoption of less complex plans without compromising long-term benefits increases relevance of policies and chance of implementation,

- Well-trained staff is cost-effective in the long term.

- Planning policies seen in the light of Federal

arrangement envisages appropriate decentralization at local level and strategic control at the centre.

- Decentralization without proper empowerment is ineffective,

- Conducive financial, manpower and institutional frameworks should support decentralization.

- Mixed use and social integration promote viability of plans.

- Mixed use creates cross-subsidy and better resource distribution,

- Mixed use increases the chance of urban areas to be active throughout the day;

- Mixed use planning helps the development of socially integrated area;

- Phased, flexible and targeted development approach increases the viability of urban plans.

- Properly phased development creates positive feedback to the system;

- Flexible approaches increase the relevance of plans. However, flexibility should not be laissez faire;

- Targeted approach economises on resource use and promotes measurability of development policies.

- Equitable distribution promotes social justice in development.

- Planning that promotes job creation contributes to the poverty reduction policy;

- Promoting the participation of inhabitants in economic activities through the creation of micro and small enterprises increases the production wealth and its equitable distribution.

### 4.3.3. Determinants of Land-Use Patterns in Ngarama town

The study carried out by the consultant on the area of study, there are five main factors which determine land use patterns in the study area. These are:

- natural environment,

- demographic and economic factors,

- Infrastructure,

- land ownership and management,

- Policy regulations and management.

Figure4.2: Determinants of land use patterns

#### Natural Environment

Aspects of the natural environment that may affect urban land use include climate, topography, insurmountable barriers (e.g., mountains or rivers) and water sources. The topography of Ngarama which is characterised by fairly flat terrain with over 60% of the study area having between 0-15% gradient.

#### Demographic and Economic Factors

Rural to urban migration is happening on a massive scale due to population pressure and lack of resources in rural areas. The ‘push’ factors are stronger here compared with the pull’ factors. People living in rural areas are pulled’ to the town because they often believe that the standard of living in urban areas will be much better than in rural areas. People also hope for well-paid jobs, the greater opportunities to find casual or ‘informal’ work, better health care and education.

#### Infrastructure

The nature of transportation network has generally led to changes in urban form. The more radical the changes in transport technology, the more the urban form have been altered.

Among the most fundamental changes in urban form is the emergence of new clusters and linear patterns expressing new urban activities and new relationships between elements of the urban system.

#### Land Ownership, Policy Regulations and Management

Gatsibo District authorities pursue urban land policy objectives, and they rely on a vast range of policy tools and institutions to achieve them. Examples are master plans, zoning, subdivision regulations, building codes, and other public policies to shape development. These regulations are normally adopted to help protect the urban and natural environment, gear infrastructure investments with development, and maintain and enhance property values. This in turn also affects land ownership pattern and use of the land.

      - Determinants of Urbanization in Ngarama town

Figure4.3: Determinants of urbanization

Tracking different land uses over time and space provides a history of their development, the dynamics of which, once understood, can be useful for future urban planning.  A number of factors that are important in determining land use patterns. In the case of Ngarama urban centre, it is important to determine the forces that bring change over time to create spatial dynamics. The main factors are migration, envelopment and economic factors, as explained below;

Rural‐urban migration

(in‐migrants minus out‐migrants) is the primary driver of urbanization in most developing countries. Residents of the neighbouring sectors of Ngarama have migrated to the urban centre as it provides many opportunities for commerce and trade due to its strategic location along Gicumbi – Nyagatare National road.

Envelopment is the physical incorporation of existing settlements previously beyond the built up edge of the city into extended urban regions through spatial expansion. Although the periphery of Ngarama urban centre is mainly agricultural land, much of it is gradually being absorbed into the urban neighbourhood as a result of urban sprawl.

#### Economic factors:

Aspects of the economy that can affect urban land use include: level of economic development; economic structure; land markets; differences in household incomes; degree of employment decentralization; level of development of real estate finance markets; level and effectiveness of property taxation; and cycles of high inflation. Land use is often regarded more as a product than as a driver of economic performance. All these economic factors play a role in growth of Ngarama.

### Planning Model and Vision Based on Ngarama Settlement Patterns.

The planning of Ngarama town was done using smart growth model. Smart Growth model is an urban planning and transportation theory that concentrates growth in compact walk-able urban centers to avoid sprawl. It also advocates compact, transit-oriented, walk-able, bicycle-friendly land use, including neighborhood schools, complete streets, and mixed-use development with a range of housing choices.

Smart growth values long-range, regional considerations of sustainability over a short-term focus. Its sustainable development goals are to achieve a unique sense of community and place; expand the range of transportation, employment, and housing choices; equitably distribute the costs and benefits of development; preserve and enhance natural and cultural resources; and promote public health.

The concept of "smart growth" emerged in 1992 from the United Nation's adoption of Agenda 21 at the UN Conference on Environment and Development (UNCED) held in Rio de Janeiro, Brazil. Driven by "new guard" urban planners, architects, developers, community activists, and historic preservationists, it accepts that growth and development will continue to occur, and so seeks to direct that growth in an intentional, comprehensive way. Smart growth principles are directed at developing sustainable communities that are good places to live, to do business, to work, and to raise families. Some of the fundamental aims for the benefits of residents and the communities are increasing family income and wealth, improving access to quality education, fostering livable, safe and healthy places, stimulating economic activity (both locally and regionally), and developing, preserving and investing in physical resource

Figure 21: Smart growth concept

Source: Maryland University 2007

#### Basic Principles of Smart Growth

There are 10 accepted principles that define smart growth;

- Mixed land uses;

- Take advantage of compact building design;

- Create a range of housing opportunities and choices;

- Create walk-able neighborhoods;

- Foster distinctive, attractive communities with a strong sense of place;

- Preserve open space, farmland, natural beauty, and critical environmental areas;

- Strengthen and direct development towards existing communities;

- Provide a variety of transportation choices;

- Make development decisions predictable, fair, and cost effective;

- Encourage community and stakeholder collaboration in development decisions;

#### Rationale for Smart Growth

It’s an alternative to;

- urban sprawl,

- traffic congestion,

- disconnected neighborhoods, and

- urban decay.

    - Principles of Intelligent Urbanism

#### 4.3.5.1 Principles of Intelligent Urbanism as a form of Smart Growth

A transit-oriented development (TOD) is a mixed-use residential and commercial area designed to maximize access to public transport, and often incorporates features to encourage transit ridership. A TOD neighbourhood typically has a centre with a transit station or stop (train station, metro station, tram stop, or bus stop), surrounded by relatively high-density development with progressively lower-density development spreading outward from the centre. TODs generally are located within a radius of one-quarter to one-half mile (400 to 800 m) from a transit stop, as this is considered to be an appropriate scale for pedestrians, thus solving the last mile problem.

Principles of Intelligent Urbanism (PIU) is a theory of urban planning composed of a set of ten axioms intended to guide the formulation of city plans and urban designs. They are intended to reconcile and integrate diverse urban planning and management concerns. These axioms include:

- Environmental sustainability,

- Heritage conservation,

- Appropriate technology,

- Infrastructure-efficiency, place making, "Social Access,"

- Transit oriented development,  and

- Regional integration, human scale, and institutional-integrity.

#### Principle one: a balance with nature

A balance with nature emphasizes the distinction between utilizing resources and exploiting them. It focuses on the thresholds beyond which deforestation, soil erosion, aquifer depletion, siltation and flooding reinforce one another in urban development, saving or destroying life support systems. The principle promotes environmental assessments to identify fragile zones, threatened ecosystems and habitats that can be enhanced through conservation, density control, land use planning and open space design (McCarg: 1975). This principle promotes life cycle building energy consumption and pollutant emission analysis.

#### Principle two: a balance with tradition

Balance with Tradition is intended to integrate plan interventions with existing cultural assets, respecting traditional practices and precedents of style (Spreiregen: 1965). This urban planning principle demands respect for the cultural heritage of a place. It seeks out traditional wisdom in the layout of human settlements, in the order of building plans, in the precedents of style, in the symbols and signs that transfer meanings through decoration and motifs. This principle respects the order engendered into building systems through years of adaptation to climate, to social circumstances, to available materials and to technology. It promotes architectural styles and motifs designed to communicate cultural values.

#### Principle three: appropriate technology

Appropriate technology emphasizes the employment of building materials, construction techniques, infrastructural systems and project management which are consistent with local contexts (situation, setting or circumstances).  People's capacities, geo-climatic conditions, locally available resources, and suitable capital investments all temper technology. Where there are abundant craftspeople, labor-intensive methods are appropriate. Where there is surplus savings, capital intensive methods are appropriate. For every problem there is a range of potential technologies, which can be applied, and an appropriate fit between technology and other resources must be established. Proponents argue that accountability and transparency are enhanced by overlaying the physical spread of urban utilities and services upon electoral constituencies, such that people’s representatives are interlinked with the urban technical systems needed for a civil society. This principle is in sync with "small is beautiful" concepts and with the use of local resources.

#### Principle four: conviviality

The fourth principle sponsors social interaction through public domains, in a hierarchy of places, devised for personal solace, companionship, romance, domesticity, "neighborliness," community and civic life (Jacobs:1993). According to proponents of Intelligent Urbanism, vibrant societies are interactive, socially engaging and offer their members numerous opportunities for gathering and meeting one another. The PIU maintain that this can be achieved through design and that society operates within hierarchies of social relations which are space specific. The hierarchies can be conceptualized as a system of social tiers, with each tier having a corresponding physical place in the settlement structure. The main considerations are;

#### A place for the individual:

#### A goal of Intelligent Urbanism is to create places of solitude. These may be in urban forests, along urban hills, beside quiet streams, in public gardens and in parks where one can escape to meditate and contemplate. According to proponents, these are the quiet places wherein the individual consciousness dialogues with the rational mind.

#### A place for the neighborhood:

#### Smaller household domains must cluster into a higher social domain, the neighborhood social group. Good city planning practice sponsors, through design, such units of social space. It is in this fourth tier of social life that public conduct takes on new dimensions and groups learn to live peacefully among one another

#### A place for communities:

#### The next social tier, or hierarchy, is the community. Historically, communities were tribes who shared social mores and cultural behavioral patterns. In contemporary urban settings communities are formed of diverse people.

#### A place for the city domain:

#### The Principles of Intelligent Urbanism call for city level domains. These can be plazas, parks, stadia, transport hubs, promenades, "passages" or gallerias. These are social spaces where everyone can go. These domains would include all freely accessible large spaces. These are places where outdoor exhibits are held, sports matches take place, vegetables are sold and goods are on display.

#### Principle five: efficiency

The principle of efficiency promotes a balance between the consumption of resources such as energy, time and fiscal resources, with planned achievements in comfort, safety, security, access, tenure, productivity and hygiene. It encourages optimum sharing of public land, roads, facilities, services and infrastructural networks, reducing per household costs, while increasing affordability, productivity, access and civic viability.

#### Principle six: human scale

An abiding axiom of urban planning, urban design and city planning has been the promotion of people friendly places, pedestrian walkways and public domains where people can meet freely. These can be parks, gardens, glass-covered gallerias, arcades, courtyards, street side cafes, river- and hill-side stroll ways, and a variety of semi-covered spaces.

#### Principle seven: opportunity matrix

The PIU envisions the city as a vehicle for personal, social, and [economic development], through access to a range of organizations, services, facilities and information providing a variety of opportunities for enhanced employment, economic engagement, education, and recreation. This principle aims to increase access to shelter, health care and human resources

Intelligent urbanism promotes opportunities through access to:

- Basic and primary education, skill development and knowledge about the urban world;

- Basic health care, potable water, solid waste disposal and hygiene;

- Urban facilities like storm drainage, street lights, roads and footpaths;

- Recreation and entertainment;

- Transport, energy, communications;

- Public participation and debate;

- Finance and investment mechanisms;

- Land and/or built-up space where goods and services can be produced;

- Rudimentary economic infrastructure;

- Intelligent urbanism provides a wide range of zones, districts and precincts where activities and functions can occur without detracting from one another.

#### Principle eight: regional integration

Intelligent Urbanism envisions the town as an organic part of a larger environmental, socio-economic and cultural-geographic system, essential for its sustainability. This zone of influence is the region. Likewise, it sees the region as integrally connected to the city. Intelligent Urbanism sees the planning of the city and its hinterland is a single holistic process.

#### Principle nine: balanced movement

Intelligent Urbanism advocates integrated transport systems comprising walkways, cycle paths, bus lanes, light rail corridors, under-ground metros and automobile channels. A balance between appropriate modes of movement is proposed. More capital intensive transport systems should move between high density nodes and hubs, which interchange with lower technology movement options. These modal split nodes become the public domains around which cluster high density, pedestrian, mixed-use urban villages (Taniguchi: 2001).

#### Principle ten: institutional integrity

Intelligent Urbanism holds that good practices inherent in considered principles can only be realized through accountable, transparent, competent and participatory local governance, founded on appropriate data bases, due entitlements, civic responsibilities and duties. The PIU promotes a range of facilitative and promotive urban development management tools to achieve appropriate urban practices, systems and forms (Islam: 2000). None of the principles or practices the PIU promotes can be implemented unless there is a strong and rational institutional framework to define, channel and legalize urban development, in all of its aspects. Intelligent Urbanism envisions the institutional framework as being very clear about the rules and regulations it sponsors and that those using discretion in implementing these measures must do so in a totally open, recorded and transparent manner.

Smart growth development can be applied for the case of Ngarama urban center whereby transit movement – especially for trucks and vehicles plying Gicumbi – Nyagatare – Kagitumba, is common. The principles of smart growth to be utilized in the elaboration of the LUDP are:

- Walkability: more buildings, residences, shops, and services closer together for ease of walking.

- Connectivity: interconnected street grid network disperses traffic & eases walking and high quality pedestrian networks.

- Mixed used and diversity: a mix of shops, offices, apartments, and homes on site, mixed-use within neighbourhoods.

- Mixed housing: housing composing a range of types, sizes and prices in closer proximity.

- Quality Architecture & Urban Design: The town should contain a range of uses and densities within 10-minute walk with highest densities at town center; progressively less dense towards the edge. More diversity and smaller, unique shops and services with local owners who are involved in community.

- Green transportation which promotes pedestrian-friendly design that encourages a greater use of bicycles, scooters, and walking as daily transportation.

- Sustainability: minimal environmental impact of development and its operations, eco-friendly technologies, respect for ecology and value of natural systems, energy efficiency, more walking and less driving.

CHAPTER FIVE: LAND USE AND LUDP

5.1. Restructuring

There is indication of informal settlement especially areas around the CBD which are covered or not by the priority areas. Evidence of this informal settlement can be seen by structures that do not have access or access is less than 3m.  The other areas with informal settlements are planned areas where people have constructed more houses within a parcel recommended for a single household hence exerting pressure on the available facilities. We have restructured these areas by applying the following principles;

### Self-Upgrading

This form of informal settlements upgrading is the cheapest for governments. This form of upgrading involves the creation of a system where the informal settlement dwellers improve the condition of their housing on their own. However, for this system to succeed, it is crucial that the dwellers’ economic capacity is grown. This requires intense involvement of Community Based Organizations, Non-Governmental organizations and the implementing government agencies.

Civic Education is recommended for these areas in order to explain to the residents the need to have access to their property and therefore necessary to widen the roads. We have proposed road widening roads and created new ones where they do not exist.

### Incremental Improvement:

Incremental improvement is an informal settlement upgrading system that involves gradual improvement of these settlements. This is done in several phases that add on to the previous stages eventually resulting in completely upgraded settlements. This system is very cost-effective as funds are not required in huge lump-sum amounts as in the case of un phased development.

Initial phases of improvement should be those involving infrastructural facilities upgrading. Creation of good road networks, widening of roads and provision of water reticulation systems and development of proper waste disposal systems are the activities that are of immediate importance. The second phase can touch on improvement of housing conditions and also on land tenure issues. All these stages among others must involve the community for success to be achieved.

### Creation of Access:

Access is a crucial socio-economic tool in all human settlements. Access opens up settlements to economic development and also the interaction of communities with their surroundings. It is also important to adopt labour-intensive road construction methods so as to create jobs for the community. Sustainability is further increased by creating road maintenance systems that are run by the communities creating both jobs and ownership of projects.

### Identification of Resettlement Sites:

Due to the unplanned nature of informal settlements, it is natural that in whichever form of upgrading is selected that some structures must be removed. It is mainly in the process of restructuring the road network and in development of infrastructural facilities that structures are affected and hence the aspect of resettlement sites becomes crucial. Other structures that require relocation would also include those located on environmentally sensitive areas.

Several factors are key in selection of resettlement sites such as:

- The site should not be so far away as to disrupt the already existing socio-economic structures of persons being relocated;

- The site should be suitable for human settlement;

- The site should have minimum infrastructure provided prior to resettlement;

Within the sites selected for implementation studies, it is expected that approximately 39 structures will be affected;

# CHAPTER SIX:  PROPOSED HOUSE MODELS

## 6.1. Introduction

The  Government of Rwanda  recognizes  that  housing  is  a  basic  right  for  its  citizens  as stated  in  international  declarations  such  as  the  Istanbul Declaration of  June, 1996,  the Millennium Development Goals (February, 2002), and the World Summit on Sustainable Development  (July-August, 2002).

In this framework, it aims to ease access to decent housing and basic infrastructure facilities , is  dedicated  to  supporting  the  controlled  development  and  the  sustainability  of human  settlements both  in urban and  rural areas, which  is economically accessible and socially  integrated, where  the  rights of  all people  are  recognized, particularly  the  rights women, children, people with disabilities, and people  living  in poverty,  the vulnerable and disadvantaged groups. In so doing the government is seeking through RHA to address this policy through implementation of LUDPs. It is therefore in this regard and in line with this policy that we are proposing different housing models that will gather for the different income groups in Ngarama urban center.

Our housing models seek to provide urban housing for a cross section of the population as provided by the ToR. This include low, medium, high density residential as well commercial building, these are detailed below.

## 6.2. Guiding principles for designing housing typologies

The Building Control and Regulations has been used as standard reference for the regulation of building design and hopefully eventual construction.  The major benefits to be gained in applying this document is that it will in the long run harmonise building construction industry in Rwanda and curtailment of informal developments so as to ensure well planned, well-maintained, safe, cost effective and decent building Developments and housing settlements throughout the country (Rwanda Building Control and Regulation 2009).

Based on how housing survey and market analysis the following key standards will applied in the long run:

One family per household and dwelling; Average household size should be about 3-4 Persons according to the survey; Average number of bedrooms per dwelling could be 2-4; Average bedroom area could be 12-22 sqm; Average kitchen area could be 12-15 sqm; A living room area could be 14-22 sqm;

6.3. Definition of terms used in housing typology design

- Building Setback: means the minimum distance (in plan) by which a building must be offset from the plot line, except for any permitted intrusions or structures.

- Front Setback: means the minimum required setback as measured from the plot line fronting any road to the external main wall of any primary or ancillary building.

- Rear Setback: means the minimum required setback as measured from the plot line directly opposite of the front plot line.

- Side Setback: means the minimum required setback as measured from the plot line that extends between the front and rear plot lines.

- Floor Area Ratio (FAR): means the gross floor area of the building or buildings on any plot divided by the plot area.

- Gross Floor Area (GFA): means the sum of the gross horizontal areas of all the floors of a building, measured from the exterior face of exterior walls or mid-point of common or party walls. The "floor area" of a building includes basement floor area, staircase blocks, planter boxes and ledges, public areas such as landings and common lobbies. It excludes floor area used for parking facilities. Basements not utilized for any habitable or commercial purposes are exempt from gross floor area calculations. Balcony & bay window areas are excluded from the GFA calculation.

- Plot Area: means the total horizontal area (in square metres) included within the Plot boundary lines.

- Plot Coverage means the percentage of a Plot area occupied by the ground area of primary and all ancillary buildings, structures and driveways, aisles and parking spaces.

- Semi-detached: building is one that abuts on one side of the plot line or building on an adjacent plot and for which no side setback is provided while attached building: is a freestanding building that does not abut any other building or adjacent plot and for which open space is provided around all sides of the building and the plot line.

Figure 22: Setback for middle and corner plot

Figure 23: Plot and building coverage

Figure 24: Attached and semidetached housing models

## 4.4 Affordable housing and demand

Conferring to affordable house project initiated by a team of professionals at RHA, for government officials or those in the civil service, the aim of any affordable housing typology is to increase economic growth and reduce poverty by providing suitable and sustainable housing units for better settlement in both rural and urban areas of Rwanda.

Although the  vision  of  the  project  proposal  was to  create an economically, easily  accessible  affordable  housing  pilot  project  that  will  act  as  a model  of  sustainable  development  addressing  issues  of  population’s  economic  status, environmental quality while meeting needs of the economically fragile group of the community and first house buyers, the report notes that it is only possible for people earning secure salary!

For Ngarama Urban center, we have designed the houses taking into consideration a number of factor key among them are:

- The Country’s vision of becoming a middle income by 2020 through various strategies which is already bearing fruit for example EDPRS I;

- The strategic Location of Ngarama urban center which near the road from Gatsibo to Nyagatare ( Kagitumba) boarder  passing through Ngarama urban center which means with a well-planned  urban center with quality service provision, it is possible to attract investors to construct houses in Ngarama urban center.

- The upcoming projects are electricity power station, valorization of Ngarama rocks, construction  warehouse,…., which will see an influx of people coming  to Ngarama urban center to take up these opportunities and many more other directly and indirectly related projects that will follow.

- The fairly undulating terrain makes the place relatively cheap to carry out development in  Ngarama urban center.

### 4.4.1 Density and dwelling size

A. High density residential

This term describes a type of housing where more than one new home is constructed and each home does not have its own footprint on the land, rather the homes occupy airspace.  These types of homes are the apartments that are part of mixed-use or high density residential developments.  They normally share a driveway as well as stair-cases. Sometimes there’ll also be a lift or a basement car parking area as well.  A shop-top apartment is a high density home; an apartment in a low-rise block of flats is a high density home as well.

The average parcel size for a single-story high density residential development in an undeveloped area would be about 450 sq meters. One house per parcel would result in a density range of 15-18 housing units per hectare.

On the dwelling size we have provided space for  activities such as cooking, eating, sleeping, conversation, entertainment, washing, childcare, religious observance and home crafts. In all this, space is of primary importance and its effective use and organization is basic to the success of any housing unit. The minimum floor area provided is 81sqm for two bedroom bungalow while apartments have 56sqm.

B. Medium density residential

This term describes a type of housing where more than one new home is constructed and each new home has its own, separate footprint on the land. A medium density housing development provides for a higher density housing outcome and includes circumstances where two new homes replace an existing home on a property. For Ngarama urban center the medium density residential will occupy a plot of 25x30m.

C. Low density residential

This term describes a type of housing that if dominant in an area, would result in a low average housing density. Low density housing can look like a large detached home on a very large residential block.  These types of homes are often associated with rural residential areas where the housing density is very low.

Low density housing is also a term used to describe the larger blocks of residential land, where separated large dwellings were constructed on properties of about 600 to 700 square metres.  Nowadays an average housing density of about 1 home for each 600 to 700 square metres is considered low in comparison to modern residential subdivision design standards.

### 6.4.2 High density residential two in on

Figure 25: High density residential two in one

Perspective

Table 31: Estimate cost for high density residential two in one house

|  | CONSTRUCTION COST SUMMARY |  |  |  |
| --- | --- | --- | --- | --- |
| ITEM | ELEMENT DESCRIPTION |  |  | AMOUNT(FRW) |
| 1 | PRELIMINARIES |  |  | 2,000,000 |
| 2 | SUBSTRUCTURE |  |  | 1,455,608 |
| 3 | SUPERSTRUCTURE |  |  | 4,039,930 |
| 4 | ROOF STRUCTURE |  |  | 908,563 |
| 5 | DOORS AND WINDOWS |  |  | 2,049,154 |
| 6 | ELECTRICAL INSTALLATION |  |  | 299,000 |
| 7 | PLUMBING INSTALLATION |  |  | 2,965,000 |
| 8 | FINISHES |  |  | 1,962,803 |
| 9 | EXTERNAL WORKS |  |  | 2,220,000 |
| 10 | LABOUR |  |  | 3,268,459 |
| 11 | OVERHEADS |  |  | 653,692 |
|  | GRAND TOTAL |  |  | 22,747,174 |

Proposed layout Plan for high density residential house on 15 by 20m plot.  The total area of the floor plan is 81quare metres which is ideal for low income earners. Size leaves enough green space and a possibility of kitchen farm for vegetables that can also be a source of income and or sustenance promoting urban agriculture. Two bedrooms Sketch on a 15 by 20m plot and the proposed fencing for these plots. The front fencing is open for ease of security.

## 6.5 Tree bed room single houses

Figure 26: tree bedroom single house front and plan vues

Plan vues

Table 32: Estimate cost for single house tree bedroom

| ITEM | ELEMENT DESCRIPTION |  |  | AMOUNT(FRW) |
| --- | --- | --- | --- | --- |
| 1 | PRELIMINARIES |  |  | 157,991 |
| 2 | SUBSTRUCTURE |  |  | 1,437,734 |
| 3 | SUPERSTRUCTURE |  |  | 3,896,188 |
| 4 | ROOF STRUCTURE |  |  | 3,051,661 |
| 5 | DOORS AND WINDOWS |  |  | 1,030,424 |
| 6 | ELECTRICAL INSTALLATION |  |  | 116,993 |
| 6 | PLUMBING INSTALLATION |  |  | 339,879 |
| 7 | FINISHES |  |  | 1,892,331 |
| 8 | EXTERNAL WORKS |  |  | 1,461,421 |
| 9 | LABOUR |  |  | 3,346,156 |
| 10 | OVERHEADS |  |  | 669,231 |
|  | GRAND TOTAL |  |  | 17,400,010 |

The three bedroom house has been designed for plot measuring 20 by 25m which can help in merging the areas or transiting between low income and medium income.

## 6.6 Maisonette

Figure 27: Perspective and plan vues for Maisonette (villas)

Table 33: Estimate cost for maisonette (villas)

| ITEM | ELEMENT DESCRIPTION |  |  | AMOUNT(FRW) |
| --- | --- | --- | --- | --- |
| 1 | PRELIMINARIES |  |  | 588,320 |
| 2 | SUBSTRUCTURE |  |  | 5,353,756 |
| 3 | SUPERSTRUCTURE |  |  | 14,508,411 |
| 4 | ROOF STRUCTURE |  |  | 11,363,607 |
| 5 | DOORS AND WINDOWS |  |  | 3,837,037 |
| 6 | ELECTRICAL INSTALLATION |  |  | 435,651 |
| 6 | PLUMBING INSTALLATION |  |  | 1,265,623 |
| 7 | FINISHES |  |  | 7,046,559 |
| 8 | EXTERNAL WORKS |  |  | 5,441,960 |
| 9 | LABOUR |  |  | 12,460,231 |
| 10 | OVERHEADS |  |  | 2,492,046 |
|  | GRAND TOTAL |  |  | 64,793,201 |

## 6.7 Guest house

Figure 28: Front and plan vues for a proposed guest house

*The estimate cost of the proposed Guest house is 712,057,740Rwf

## 6.8 Mixed commercial building

Figure 29: mixed commercial building front and plan vue

## 6.9 Proposal for community facilities

### 6.9.1 Proposed School

Figure 30: Front and plan vues for proposed primary school

Source: V.COLTD, 2015

Figure 31: perspective and plan vue of proposed secondary school

### 6.9.2 Proposed stadium

Figure 32: Perspective and plan vue of proposed stadium

# CHAPTER SEVEN: AERIAL VIEW OF NGARAMA AFTER IMPLEMENTATION

## 7.1. Aerial view Ngarama after implementation of LUDP

Map(annexes)

## 7.2. Cost reducing measures for housing

- Local material use

To moderate the cost of building, we recommend the use of locally available materials like timber and tiles for roofs and burned bricks compact earth blocks for walls. This will enhance the chances of low-income groups to own houses. In addition, we recommend government subsidies on other materials e.g. cement which are used with locally available materials. Through private-public-partnerships, the government can also spearhead improved housing construction by developing policies and laws which guide this strategy. For example, landowners can provide land for housing while private developers liaise with financial institutions for funding. Joint ownership of the developed property can then be secured in well-structured legal framework.

- New construction technologies

Interlocking bricks: Soil tests conducted during diagnosis stage showed that the selected priority sites exhibit soil characteristics which are favorable for production of pre-fabricated bricks; also known as interlocking bricks.

Interlocking bricks are hydraulically compressed into uniform  shape and size. These blocks have a high crushing strength, smooth finished surface and have excellent thermal insulation properties comparable to other masonry bloc

Benefits

- Interlocking bricks do not need cement mortar in joints;

- Only simple pointing is required;

- Reduces labor cost for construction as fixing is very simple;

- Eco bricks have beautiful tile finish, hence no plaster is required;

- Time required for construction is reduced by 1/3rd;

- Ideal for boundary walls works;

- Smooth finished surface;

- Excellent thermal insulation.

## 7.3. Planning scenario for houses

### 7.3.1. Low standing housing scenario

As above-mentioned, the main strategy of cutting down the cost of building is to use locally-available materials. Using conventional materials, it is estimated that the cost of low-standing house is Rwf. 21.8million. With the use of materials such as interlocking bricks/blocks; it’s estimated that this cost will reduce by 30% hence the final cost is Rwf. 13.3 million

### 7.3.2. Medium standing housing scenario

Medium standing means houses with the occupation of people with medium income. In Ngarama urban center, the planning is to use the local materials in order to reduce cost as it was done in the previous scenario. Medium standing is made by a plot  of 20*25m and can hold a house for 3 rooms, car paved parking, garden,...Concerning houses materials, we are using a modern roof material and in the walls we use local burned clay bricks.

By using hydra form bricks-maker machine the cost must be reduced enormously. Instead of using steel material in houses walls construction, the using of the hydraform bricks can reduce cost.

*The sample of housing typologies are presented above, the details will appear in the annexe.

# CHAPTER EIGHT: PROPOSED AND EXISTING ROAD AND DRAINAGE DESIGN

## Introduction

One of the main concerns for our road design is to maximize access to the neighboring parcels. Unfortunately while doing this it affects mobility since both access and mobility are inversely related. We have therefore applied the new standards that is being applied worldwide, where by the shift is from mobility oriented analysis to accessibility based analysis that looks at the broader range of impacts and options. Travel statistics often undercount and undervalue non-motorized travel by ignoring short trips, children’s travel, non-commute trips, and non-motorized links of motorized trips. We have also considered the relationship between workability and land uses. Among the key issues that we have focused on include;

- Mobility, particularly vehicle travel;

- Geographic accessibility (distances between activities);

- Telecommunication quality (such as the portion of Internet services);

- Accessibility options for specific groups to specific services (such as disabled people’s ability to reach medical clinics and recreation centres).

While focusing on the above it has also been paramount for us to consider the following issues that impact either positively or negatively on the overall travel demand;

- Demographics (age, income, employment status, gender, etc.)

- Purpose (commuting, personal errands, recreation, etc.);

- Destination (school, job, stores, restaurants, parks, friends, families, etc.).These can be divided into common destinations (goods and services available at many locations) and unique destinations (activities at a particular destination, such as a friend’s house).

- Time (hour, day, season);

- Mode (walking, cycling, automobile driver, automobile passenger, transit passenger, etc.). Mode split (the portion of trips made by different modes) is affected by factors such as vehicle availability, the quality of alternative modes and community design.

- Distance (from origin to destination and from origin to access each mode, such as walking distance to transit stations).

Table 34: Suitability mode for each category considered for NMT

| MODE | NON DRIVERS | POOR | HANDICAPPED | LIMITATIONS | APPROPRIATE USE |
| --- | --- | --- | --- | --- | --- |
| Walking | Yes | Yes | Varies | Requires physical ability. Limited distance and carrying capacity. Difficult or unsafe in some areas | Short trips by physically able people |
| Wheelchair | Yes | Yes | Yes | Requires sidewalk or path. Limited distance and carrying capacity | Short urban trips by people with physical disabilities. |
| Bicycle | Yes | Yes | Varies | Requires bicycle and physical ability. Limited distance and carrying capacity | Short to medium length trips by physically able people on suitable routes. |

## Upgrading of existing road and footpath

One of the main considerations in this design is minimal interferences of already existing situation. To that end we have adopted the strategy of widening the existing footpaths and designing them into standard road all with a minimum of six meters or more. These ensure less expropriation and little disruption of the daily activities of the residents of these areas. Apart from design we have also ensured that they have very well designed drainages that can accommodate the surface runoff.

## Accessibility to each plot in the planned area

While producing road layout design we have considered accessibility to each parcel that has been considered in this area. This way it ensures that every individual can easily get to his/her parcel in a comfortable, flexible and affordable means of transport.

## Road dimensions

Depending on hierarchical street (roads) networks composed of neighborhood, collector, arterial, and highway roads tends to force traffic onto a small number of major roads, which provide drivers with few alternate routes in the event of congestion or accidents. Also, because major roads concentrate traffic, they generally do not provide a good environment for pedestrians or residential. With this in mind and the analysis that we have done, based on the development that are underway and the vision of the sector, district and the country at large we have design all road width in the study area to be 15m. This ensures that there is sufficient space to gather for facilities will include space for sidewalks, drainage, street lighting, NMT among others. The cross-sectional elements that have been considered is illustrated in the figure below

The cross-sectional elements provided include the following:

- Right of way which is the total land reserve for all road facilities and any future expansion in the interest of the public. We have set it at 11m.

- Roadway  width include the carriageway and the shoulders and we have allocated this 11m

- Carriageway is  where the traffic lanes will be and on this  we propose 7m

- NMT reserve which is not common in most cases has been assigned 1.00m

- Drainage is  for storm water has been allocated 1.5m

- Pedestrian way 1.5m

The national roads, district road class one and two have to follow the dimensions indicate by RTDA, respectively 44m and 22m.

Ngarama urban center has a network wich is divided in two National roads, district roads class1, district road class2, bypass, secondary rods and tertiary roads as follows:

Table 35: Road category in study area

| No | Category | Proposed width(m) | Length(m) |
| --- | --- | --- | --- |
| 1 | National road | 44 | 10074.56 |
| 2 | District road class1 | 22 | 7968.19 |
| 3 | Bypass | 22 | 1664.79 |
| 4 | Secondary roads | 12 | 20991.90 |
|  | Tertiary | 9 | 9837.92 |

## Proposed road network in Ngarama urban center entire area

MAP8.1: Proposed road network

Map 26: Proposed road network in Ngarama site

Map 27: Proposed road network for Gatungo

Figure 33: Cross section and drainage system along the road.

## Soil analysis (SEE ANNEXE)

### Penetration test

### Dry density

### CBR test

### Compaction test

### Sieve analysis

### Atterberg limit analysis

## Non-motorized transport

NMT infrastructure has been designed such that they require very little maintenance and construction materials are sturdy with long life. We have proposed dedicated bicycle lanes, carts, and pulled non-motorized movement which is separated from the pedestrian ways. For safety purposes at intersection we have proposed pedestrian crossing which can also include indications of NMT crossings. As shown above this type of transport is also the main source of income for some individuals and should therefore be well designed.

Road and path connectivity (also called permeability) affect accessibility. Increased connectivity tends to increase accessibility. A grid network with shorter

Connected roads, tends to provide relatively good accessibility due to multiple routes, more direct connections between destinations, and narrower streets with lower traffic speeds that are better suited to walking and cycling, and therefore to public transit travel (since most transit trips involve walking links).

## Transportation network connectivity

Figure 34: Transportation network connectivity

# CHAPTER NINE:  ROAD DESIGN AND COSTING

## 9.1. Road design and cost estimate for Ngarama site

The roads and drainage have been designed in such a manner that they follow the contours. This way it ensures that the drainage which is also running along the roads have an easy flow along the edge of the road. The length of all the roads to be designed measure( table..) estimate to cost RWF 400million per km for tamalc roads, 100million for marram roads and 200milion for stones roads. Detailed volume and costing estimate in the appendix.

# CHAPTER TEN:  STORM WATER MANAGEMENT

## Catchment area

The capacity of the storm water drainage channel along the roads networks as well as standalone canals are designed according to the defined catchment areas. Figure8 present the defined catchment areas for the road networks and exist drainage around the selected site.

Figure 35: Rainfall intensity graph in rural and urban areas

It should be noted that the main objective of storm water management is to ensure that it minimizes soil erosion, flooding and pollution of water along the route. In designing for this we have taken into consideration the watershed. It is in this context therefore that we have designed for surface runoff along the roads (see cross-sectional elements)

## Drainage map of the study area

Map 28: Ngarama site drainage map

Source: V.COLtd, 2015

Map 29: Gatungo site drainage map

## Drainage design

To calculate the discharge from catchment area we use the formula below:

=

Where

C: Run-off coefficient

i: Critical intensity of Storm in cm/h occurring during the time of concentration

A: Catchment area in ha,  S: Longitudinal slop of surface of the road (m/1000m)

Design of drainage as trapezoidal cross- section with following sizes and compare the discharges

If Q for drainage is greater than Q for Catchment (area of road and foot way)  The drainage dimensions meet the requirements. Otherwise the dimension of drainage will change to meet the requirements.

Where;

C is the roughness coefficient

P is the wetted perimeter,

A is the area of flow section,

R is the Hydraulic radius,

So is the bed slope which provide uniform flow

- Drainage Type T2

To calculate the discharge form catchment area we are use the formula below:

=

Where

C: Run-off coefficient

i: Critical intensity of Storm in cm/h occurring during the time of concentration.

A: Catchment area in ha, S: Longitudinal slop of surface of the road (m/100,

Where C: is the roughness coefficient

P: is the wetted perimeter,   A: is the area of flow section,    R: is the Hydraulic radius

So: is the bed slope which provide uniform flow

# CHAPTER ELEVEN: WATER SUPPLY

## Water network in Ngarama Urban center

Map 30: Existing water network

As seen from the map above, the areas under study have some connection of piped water supply. However field survey indicated that supply is unreliable connections. There are a number of storage tanks at strategic points which does no longer containing water due to the miss management of the water network.

### Proposed water network in Ngarama town

Objectives

The objectives of a water supply project include the following:

- Provision of water for domestic consumption and personal hygiene in terms of the Water Services.

- Design standards require that a minimum of 25 liters per person per day be provided;

- The improvement of the quality of the existing supplies protection of the sources being the first consideration;

- The improvement of the availability of water to the community (both reliability and accessibility):

- Community involvement (acceptability) and commitment

- Improvement of public health;

- The improvement of the living standards of the community;

- Development of local  technical, financial and administrative skills;

### Existing water consumption

The rural water coverage is 49.2% (EWSA 2012). In fact Gatsibo District is considered the first water-scarce District in the Eastern Province. This means that majority of the population use dirty water from streams, dams, valleys or swamps

This shows that there has not been significant improvement in terms of increased water coverage in the District yet securing water for improved rural livelihoods is essential. This shows that more than 50% of the populations in Gatsibo District do not have access to safe and reliable supplies of water for productive and domestic uses. According to EICV3, 35.9%of the population takes 5-14 minutes to access water while 25.6% take 15-29 minutes and15.9 % take 30-59 minutes. A big percentage of the population experiencing some kind of physical or economic water scarcity are rural poor people who depend on agriculture for their livelihoods especially women. According to the survey opperations.

  - Water demand  for Ngarama Urban center regarding the population increase

Table 36: Projection of population and water demand

| Years | 2015 | 2020 | 2025 | 2030 |
| --- | --- | --- | --- | --- |
| Population of the study area | 16828 | 17202 | 17584 | 17974 |
| Water demand per dayper en L | 420700 | 430050 | 439600 | 449350 |

## Other water sources that can be used

Wells; Springs ; Bore holes; Rainwater harvested

## Water supply options

The system comprises a water reticulation system with standpipes in open areas or in road reserves. Basic water as access to 25 liters of potable water per person (capita) per day at a communal street tap which is within 200 meters of the dwelling, with 98% reliability and a 10 liters/min flow rate. Provision is usually made in the design for upgrading.

The design of the standpipe installation requires careful planning, and special attention is given to drainage of excess water and avoiding wastage, in order to minimize health risks. A typical example is shown below .

Communal standpipes serving dwelling houses should satisfy the following criteria:

- One tap required per 25-50 dwellings;

- Maximum number of people served per water point: 300;

- Maximum number of people served per tap: 150;

- Maximum walking distance from a dwelling to a standpipe: 200 m.

- An acceptable discharge capacity from a standpipe is about 10 f/mm per tap. For commonly used taps the calculated discharge range, at an assumed efficiency of 80%.

## Proposed water distribution network map

Map 31: Proposed water distribution network map for Ngarama site

Source: V.COLTD, 2015

Map 32: Proposed water distribution network map for Gatungo site

Source: V.COLTD, 2015

## Conclusion on water supply planning

The existing water network combined with planned water network should respond to the needs of population and the target of EDPRS of having water not far from 200m per household and…

# CHAPTER TWELVE:  PROPOSED ENERGY PLANNING

## Existing situation of the energy in Ngarama urban centre

Our energy proposal seeks to use the already existing connection an extend them to reach all the households. Since the project is phased then connection will be on a ‘when is ready basis’ to minimize costs of putting poles on areas where there is no activity going on.

Map 33: Existing situation of the energy in Ngarama urban center

Source: V.COLTD, 2015

## Proposed electricity network in the study area

Map 34: Proposed electricity network in Ngarama site

Source: V.COLTD, 2015

Map 35: Proposed electricity network in Gatungo site

Source: V.COLTD, 2015

Table 37: Proposed electricity network

| Category | Length (m)/Site |  | Total Length(m) |
| --- | --- | --- | --- |
|  | Ngarama | Gatungo |  |
| Medium Voltage | 11,573.81 | 638.00 | 12,211.81 |
| Low Voltage | 45,208.61 | 13,759.68 | 58,968.29 |

Source: V.COLTD, Collected data, 2015

## Alternative source of energy

The team is proposing the utility of alternative sources of energy that include:

### Biogaz

This will majorly be utilized for cooking, lighting, water heating, house warming amongst other household activities. The survey realized that there was evident livestock rearing in the area which will be an opportunity for generation of biogas on a household level.

    - Solar energy

Ngarama study area as others regions of EASTERN PROVINCE experiences a significant number of sunlight hours that are on average eight hours per day. This is quite significant for installation of solar panels as alternative energy source. The solar panels can be installed by a household which proves to be a bit expensive but the cost effective method recommended will be for a neighborhood to come together and collectively purchase, install and manage the solar system. This source of energy can be utilized for lighting, heating and water warming. It should be noted that this type of energy can be utilised especially for apartments or four in one houses. With the cost of acquiring them coming down due to technology, and there size becoming small, it is becoming appropriate type of energy.

### Air energy

The wind energy is also applicable in this area since the speed of wind is high in most of period during the rain and dry seasons.

## Conclusion on energy planning

Connection to the national grid is recommended as the main source of energy in the planning areas. Alternative sources will be utilized to minimize costs and as backup whenever there will be power shortage.

# CHAPTER THIRTEEN: SANITATION

## Introduction

Sanitation covers, according to the 1997 Kampala Declaration on Sanitation (KDS), “the isolation/management of excreta from the environment, maintenance of personal, domestic and food hygiene, safe disposal of solid and liquid wastes, maintaining a safe drinking-water chain and vector control”.

The Rwanda National Water and Sanitation Policy (2010) is a very important document which guides Water and Sanitation provision in the country. It is grounded on “Vision 2020”, “Millennium Development Goals” and “Poverty Reduction Strategy” (MININFRA, 2010).

Provision of adequate Water supply and sanitation services is therefore a fundamental element of development strategies and indicators, including Rwanda’s Vision 2020 and EDPRS as well as the international MDG No.7C which is to halve, by 2015, the proportion of the population without sustainable access to safe drinking water and basic sanitation.

## In situ disposal option

As identified by Tsinda (2011), the formulation of sanitation policy guiding Rwanda is grounded on a number of key principles. These are:

      - Priority to basic services: each person and community has equal right to access basic water services. Priority will be given to “some for all” rather than “all for some”, until the Vision 2020 goal of access to safe drinking water for all is reached. Due attention will be given to affordability considerations;

      - Decentralization: The responsibility for service delivery is vested at the de-centralized level. The water and sanitation sector is committed to building and strengthening decentralized planning, implementation and management capacities;

      - Community participation: The beneficiaries of water supply and sanitation services shall be actively involved in planning, decision making and oversight throughout the project implementation cycle. In particular, they will choose the service level that responds to their needs and capacities;

      - Cost recovery and financial sustainability: Operation and maintenance costs of water supply and sanitation infrastructure shall be borne by the users, in order to ensure sustainable service delivery. Affordability shall be addressed by the choice of appropriate technologies and by enhancing efficiency, not usually by granting subsidies. The polluter-pays and user-pays principles are to be applied in sewerage and waste management;

      - Private sector participation: The sector will continue to promote delegated management through private operators, which is the key strategy to enhance the sustainability of rural water infrastructure. The private sector will also be encouraged and supported in developing capacities for investment, construction and service delivery in water supply, sanitation and solid waste management;

      - Emphasis on sanitation and hygiene: The sector recognizes the critical importance of sanitation and hygiene behavior change for the achievement of sustainable health benefits. Sanitation and hygiene activities and projects shall be developed through strategic cooperation with the health and education sectors. Any water supply projects shall systematically consider sanitation implications and hygiene education requirements;

      - The polluter pays principle: Polluters must pay the cost of cleaning up the impact of their pollution on the environment. Sanitation services must be financially sustainable. Sanitation services must be sustainable, in terms of both capital and recurrent costs.

For practical purposes sanitation can be divided into on-site and off-site technologies. On-site systems (e.g. latrines), store and/or treat excreta at the point of generation. In off-site systems (e.g. sewerage) excreta is transported to another location for treatment, disposal or use.

### In situ disposal

In several places, mainly in areas with low population densities, it is common to store and treat wastes where they are produced - on-site. There are a number of technical options for on-site waste management which if designed, constructed, operated and maintained correctly will provide adequate service and health benefits when combined with decent hygiene.

Dry sanitation or eco-sanitation is an onsite disposal method that requires the separation of urine and faeces. Building and operating these systems is often much less expensive than off-site alternatives. Some on-site systems (e.g. septic tanks or latrines in densely packed urban areas) require sludge to be pumped out and treated off-site. Composting latrines allow waste to be used as a fertilizer after it has been stored under suitable conditions.

Figure 36: Single-chamber septic tank

Ventilated improved pit (VIP) toilet

The VIP toilet is a pit toilet with an external ventilation pipe. It is both hygienic and relatively inexpensive, provided that it is properly designed, constructed, used and maintained.

VIP toilet offers improved sanitation by eliminating flies and smell, through air circulation. The addition of a chimney draws air currents into the structure and through squat hole. Odours rise through the chimney and disperse. The structure of the toilet means that any flies attracted to the pit through the squat hole will try to escape by heading towards the strongest light source, which comes from the chimney. The flies exit is blocked by a wire mesh so the flies eventually die and fall back into the pit. The spiral structure prevents too much light entering the toilet while allowing a free flow of air.

It is possible to construct the entire toilet from local materials e.g. burned clay, although it is more usual to use commercial products for the vent pipe and the pedestal. With basic, locally available materials and some training, community members can build and maintain latrines that suit them and their environment. When the pit is full, the superstructure, pedestal, vent pipe and slab are normally moved to a freshly dug pit and the old pit is covered with soil, The pit can then be emptied when required, using a suitable vacuum tanker, without the danger of the sides of the pit collapsing. Water (sullage) poured into the pit may increase the fill-up rate (depending on soil conditions) and should be avoided; this sanitation technology is therefore not recommended where a water supply is available on the site itself. Single pit latrines are very low costs, and are among the cheapest and easiest systems although almost no treatment is included (WHO 1992).

### Off-site disposal

### Material for building pit latrine

Figu Figure37: pit latrine

As indicated in the chart, majority of the people in the study areas of Bugesera utilise pit latrines. The design and structural strength of these latrines vary from house to house depending on among others: financial ability of a household owner, availability of building materials and taste of an individual. We recommend use of local materials which are structurally sound e.g. clay bricks. While some residents have taken the initiative to improve their pit latrines, others still use poor materials like in-situ dried clay, iron-sheets or wood (rukarakara)

Figu   Figure 38: Ventilated vault (VV) toilets

The common solution for sites without running water is what is called a "vault toilet." Waste is held in an underground vault or tank. These vaults are pumped out periodically, and the waste is hauled out to wastewater treatment plants.

Vaults are made of concrete or plastic material. The most popular type of vault is made of reinforced cross-linked polyethylene. This material will not crack or leak, so it meets all environmental standards.

A typical vault toilet installation has the vault buried in the ground with a concrete slab poured in place directly over it. The building is installed on the slab with connections to the vault for the toilet and vent pipe. This type of system provides a permanent, odor-free and accessible toilet at a fraction of the cost of site-built structures.

Prefabricated vault toilet choices range from buildings made of cross linked polyethylene (same material as the vault), to wood frame construction, to reinforced concrete structures. Single, "unisex," and double vault toilet systems are available. The concrete designs are often chosen for their superior durability in vandal-prone areas.

#### Full waterborne sanitation

#### Most off-site strategies (and some on-site systems) require wastes to be treated at a facility before it can be safely used or released into the environment. One approach is to use mechanical and biological processes (primary and secondary treatment) to remove suspended solids, biological oxygen demanding substances (BOD) and other pollutants. Pathogens and nutrients are typically only minimally removed in these processes. The problem is that these conventional or mechanical processes are expensive to operate: they require energy, skilled labour, infrastructure, and maintenance. To further reduce the pathogens and nutrients requires additional processes, which pushes up the cost still further.. The system requires a water supply connection to each property. The water is used to flush the excreta from the toilet pan and into the sewer, as well as to maintain a water seal in the pan. The excreta are conveyed by the water in underground pipes, to a treatment works that may be a considerable distance from the source. The treatment works must be able to handle the high volume of liquid required to convey the excreta. The quantity of water required (usually 6-10 liters per flush) can be reduced by using low-flush pans designed to flush efficiently with as little as three liters. In an area where water is costly or scarce, it may be counter-productive to purify water only to pollute it by conveying excreta to a treatment and disposal facility.

### Waste water and excreta treatment

Map 36: Sewarage system in Ngarama site

Source: V.COLTD, 2015

Map 37: Sewage system in Gatungo site

Source: V.COLTD, 2015

Table 38: Drainage system categories and length

| Category | Width (m) | Length (m)/Site |  | Total Length(m) |
| --- | --- | --- | --- | --- |
|  |  | Ngarama | Gatungo |  |
| Main Drainage | 44.00 | 9,153.96 | 2,298.04 | 11,452.00 |
| Secondar Drainage | 12.00 | 6,926.71 | 3,386.15 | 10,312.86 |
| Tertiary Drainage | 9.00 | 56,505.11 | 10,510.99 | 67,016.10 |

Waste needs to be treated to remove or inactivate pathogens before it can be safely reused or disposed of safely. Many on-site waste disposal methods treat excreta by storing it for enough time to kill the pathogens. Local sewerage networks can be connected to small treatment plants (baffled aerobic reactors) to cost-effectively treat more waste.

In warm climates where land is available at low cost, waste stabilization ponds (WSP) provide an excellent method for treating wastewater. When designed properly, WSP are more effective at removing pathogens - particularly intestinal worm eggs, than most conventional secondary treatments. Moreover, WSP remove pathogens without the addition of chemicals such as chlorine are simple to operate and maintain, and promote the use of the water and nutrient resources in the wastewater.

In many areas of the world, particularly in water scarce regions, treated wastewater has been used for numerous activities in urban settings including: irrigation of golf courses and public parks, creation of recreational lakes, street cleaning, environmental restoration, car/train washing, fire protection, and for toilet flushing. In many urban areas non-potable uses require larger amounts of water than potable uses. Using properly treated wastewater for these purposes is one way of extending water supplies. This could allow communities to protect high quality water sources, reserving them exclusively as drinking water sources (WHO, 2010).

In the selected implementation sites of Ngarama urban center, it is recommended that WSPs be used as means of treating sewage once a centralized system is established. This predicts well with the economic status of the residents in these areas as well improve environmental conditions as set by REMA regulations.

Figure 39: Waste water treatment in WSP

## Solid waste management

The key objective of urban solid waste management is to collect, treat and dispose of solid wastes generated by all urban population groups in an environmentally and socially satisfactory manner using the most economical means available. The responsibility of providing solid waste management services normally repose with local authorities although the emerging trend is involvement of private waste collectors with households bearing the cost. As cities grow both spatially and economically, business activity and consumption patterns raise solid waste quantities.

## Waste generation and composting

### Generation

Most of the solid wastes are generated from households, while other sources include agricultural and light industrial activities.

### Compositions

Some of the common sources of solid waste include paper, wood and yard materials, food waste, plastic materials, metals and glass. This represents both organic and inorganic waste.

    - On site storage

Functions of waste storage are at the place where the waste is generated. For residential waste, this means at home in the household. Onsite handling is the very first step in waste management. It involves individual family members, households and communities, all of whom need to know how to handle waste properly at this level.

In the selected sites of Ngarama urban centre, we recommend separation of wastes into their different types so they can be allocated with in the most appropriate way, for example, separating putrescible waste for composting. The main benefit of this procedure is reducing the volume of waste for final disposal and recovering usable materials.

It is important that waste is stored in proper containers. These could be baskets, preferably made from locally available materials, plastic buckets or metal containers. Larger containers or dustbins, especially those used for food waste, should be leak proof, have tight lids and be long-lasting. The size of the container should be sufficient to hold at least the amount of solid waste that is generated per day at household level. Institutions and businesses should consider having onsite storage facilities with greater capacity. The proper location of storage containers and the frequency and time of emptying are important factors to be considered for efficient onsite storage.

### Collection of waste

Generally, waste is collected and held at central transfer stations where waste is stored before it is transported to a final disposal site.  Once the quantity of waste necessitating collection is determined, it is important to plan collection routes to ensure a productive and economical service. This can be done by obtaining a map or plan of the area in order to identify the following:

- All service points;

- All one way streets;

- Any culs-de-saç and

- Areas that do not require a service.

Areas of daily collection should be compact and not fragmented and, where possible, natural boundaries should be used. Collection routes should be planned to maximize vehicle capacities. For the convenience of householders it is preferable to maintain a regular routine, to ensure their waste is ready for collection.

The plans below depict the system of collection depending on the nature of routes.

Figure 40: Route collection from two sides of the road

- Transfer stations/systems

Regardless of their degree of sophistication, transfer stations can assist in the reduction of haulage costs; reduce the congestion of traffic at the landfill; and provide opportunities for recycling.

- Incineration

Large-scale incineration is capital-intensive, but has the advantage of: reducing the volume of waste needing final disposal, with a resultant reduction in land use; combating the spread of disease; and providing a potential energy source.

- Recycling

Before considering a recycling program it is necessary to consider the involvement of the general public recycling agencies; entrepreneurial development; education of the community; secure markets; and economic viability. Paper recycling plant is recommended using office stationery including files, envelopes and cards.

The total amount of waste generated is a combination of collected and recycled waste or;

Generated waste = Collected waste + Recycled waste.

This is illustrated in the figure below.

Figure 41: Total generated waste

- Disposal

With land filling being the final step in the waste management cycle, consideration should be given the method used which would reduce cover material requirements; reduce both windblown litter and vermin; reduce leach production; and influence the type of landfill equipment needed on the site.

## Land fill location

### Location

According to RURA regulations of 2011, solid waste landfills shall meet the following minimum technical location standards:

The bottom of the landfill shall be at least 3 meters above the seasonal high ground water level;

- The maximum height of the Site above ground level must not exceed three (3) meters.

- The slope of the sides of the Site must be constructed in such a manner

that little or no erosion occurs.

- The edge of the landfill shall be not closer than sixty (60) meters to a

surface water body and hundred (100) meters upstream from a public water supply well;

- The landfill shall not be located in aquifer recharge areas or public water

supply

- Not close than 3km from an airport or proposed site for an airport. We

have therefore proposed 7km from the proposed Bugesera airport.

Walsh and O'Leary (2002) add the following as important considerations in siting a landfill:

- Is easily accessible by solid waste vehicles in all weather conditions;

- Safely protects surface and groundwater quality;

- Controls landfill gas;

- Has access to earth cover material that can be easily handled and

compacted;

- Is located where the landfill's operation will not affect external

environmentally sensitive areas;

- Comprises enough land and internal capacity to provide a buffer zone

from neighbouring properties and is able to be expanded; and

- Will be the most economic site available given haul distances to user

communities and other economic considerations.

We recommend the use of a natural attenuation landfill (Figure 23) because it’s cheaper to construct and naturally environmentally friendly.

Figure 42: Natural Attenuation Landfill

Source: REMA, 2015  Site criteria

Map 38: Location of land fill and cemetery

### Conclusion

Waste management in the changing societies is a dynamic process, in terms of both collection and disposal. It is the responsibility of the local authority to ensure the service is provided to its communities. Town and city engineers therefore need to be vigilant in identifying changes, and be innovative in making the service affordable, while meeting the standards expected by these communities .

# CHAPTER QUATEREIZE: GREENING AND BEAUTIFICATION

## Beautification of residential area

Landscaping refers to any activity that modifies the visible features of an area of land, including: living elements, such as flora or fauna and gardening.

We recommend that the residents in the selected area of Ngarama LUDP be encouraged and sensitised on the need to carry out beautification within and around their areas of residence. The Figure below represents a typical beautification model that can be applied in residential areas. A combination of low-height umbrella trees with greenery running parallel to pedestrians’ walks is ideal on non-sloped terrain.

Figure 43: Example of land scape

Streetscape design can achieve improved air quality by encouraging people to walk rather than drive, resulting in fewer greenhouse gas emissions. The following are design priorities for creating successful pedestrian environments:

- Attractive Environments: shade and site furniture

- Civic Art: Quality Materials

- Way-finding : Signage and symbology

- Community Identification: Materials and textures

- Security: Reduced Visual Obstructions and adequate Lighting

## Street trees

Street trees are capable of significantly lowering urban air temperatures. Where street trees create a continuous overhead canopy, temperature differentials are felt. However, human comfort is not the only benefit of shady streets. Well shaded streets also shade homes and create a cooler microclimate around them which can reduce household energy bills

Figure 44: Typical street beautification layout

Source: Adapted from Tetra Tech Architects & Engineers

## Beautification of schools, hospitals and administrative domains

Figure 45: Beautification of school

Source: adapted from wordpress.com

## Trees planting on streets

Figure14.4:

Figure 46: Trees planting along the streets

Source: Kigali city street photo

## Improved pedestrian environment

Providing safe and effective pedestrian facilities is a long-established goal for urban planners as they work to reduce dependency on the automobile. While many traditional engineering techniques, such as grade-separated facilities and pedestrian signals, have been employed to provide safe crossings at high-volume locations, the city has recently engaged in an effort to reinforce existing at-grade facilities with alternative treatments designed to make motorists more aware of pedestrians crossing the roadway. This, in turn, has created a safer, more positive experience for pedestrians.

# CHAPTER QUINZE: conlusion CHALLENGE AND RECOMMANDATIONS

Conclusion

Although the visioning of urban planning and general development of the town has been concise and dynamic, sporadic expansion of the town is still rampant. This makes it difficult to provide basic infrastructural services of the residents of the town. It is however widely acknowledged that despite of urban sprawling evident from spatial information; some effort has been put in provision of basic infrastructural services to the residents.

The existing land use patterns depict reliance on agriculture as the main source of income for the residents of Ngarama. This poses a challenge to improvement of housing which requires adequate funding. Farm crops cannot be collateralized by farmers to obtain funding for housing, which is the ultimate objective of this study.

By the end of this study, it is anticipated that the local leaders of Gatsibo district and the urban center of Ngarama will set-out clear agenda for implementation of the elaborated LUDP to guide and advance orderly growth and development of the town. Some of the key recommendations emerging from the diagnostic study are as follows:

- Urban sprawl in Ngarama urban center – mainly along transport corridors (main highway along the road Gatsibo –Nyagatare passing trough Ngarana Urban Center), is spreading at a rapid rate. This has to be curbed though prompt implementation of the developed LUDP which is a product at the end of this study;

- More effort and resources need to be allocated to implementation of proposed urban development plans and other related visionary documents;

- Synergy of various proposals that affect the lives of urban dwellers of Ngarama center including environment, urban trade and agriculture, hospitality and tourism sectors, infrastructure plans and harnessing the advantages brought about by hilly terrain;

- Lastly, public awareness need to be conducted to educate the residents on ways and strategies of improving their housing conditions e.g. by joining SACCOs to improve their credit-worthiness and incremental house development.

# CHAPTER SEIZE: IDENTIFICATION OF IMPACT WHEN LUDP IMPLEMENTED

## Legal framework and policies:

By law, EIA must be conducted the developer before implementation, during implementation phase and after completion of a project. This is done purposely to protect Rwanda’s natural environment from potentially significant and avoidable impacts caused by development projects and to facilitate efficient and modern development activities whilst considering the needs of present and future generations.  Basing on the organic law No 4/2005 articles 67-70, the developer intends to carry out the activities on the sites within the required and laid laws and policies. The EIA expert carried out an EIA to physically see/observe and assess the possible environmental impacts of the project during the implementation phase. According to REMA guidelines, the following was observed on the ground and organized under the leitmotifs below:

## Land conditions

The EIA was carried out on the sites so that anticipated impacts are assessed earlier as regards the land/soil aspects. Since the damage on the sites will be more similar on NGARAMA site and GITINDA site they are discussed together:

### Ngarama and Gatungo;

The anticipated impacts on the soil in these areas will be great disturbance since there will be general clearance of the vegetation. This will lead to accelerated soil erosion since the soil will be exposed. Then there is going to be a total change the appearance of the area because, the vegetation will be cut and then excavation will take place.

Not only that, with increased digging of soil, when it rains there will be siltation thus water pollution. More so there is potential of land disturbance that will affect underground organisms. There will be increased runoff since some of the roads will be tarmacked. The construction activity of roads does spoil the good backdrop of the area. Due to road construction in this area, the drainage trenches that will be dug to allow water to flow will instead enhance rapid soil erosion. Since the sites have slopes, this will accelerate the water runoff and reduces the stability of the land surface.. There will be great disturbance especially the areas that are under cultivation. This implies that some of the areas will have permanent scars which will take time to repair especially where paving and use of tarmac is done

  - Biotic community (plants and animals);

Since REMA guidelines also demand a report on biotic community it was prudent these elements were taken into consideration. The following observations were made on all the sites in general;

Basing on the observations made during the initial phase, predicting impacts on these sites is crucial. Since it was observed that on these sites the vegetation cover exists, the implementation phase will involve cutting and clearing the vegetation for smooth work flow. This implies most vegetation in areas that the project carried out will destroy the vegetation. The vegetation is important because it plays a role in reducing soil erosion; some is used for fire wood and grazing of the domestic animals as well as the good scenery it gives to the area. It is also a good habitat for bids, insects like butterflies, snakes among others but most of the habitat area will be cleared during the construction phase. More to that, the sites are hilly and the fact that the vegetation will be cleared, the it implies that more soil loss and flooding in future will take place. There is high risk of losing unique species of plants as well as the special key species for breeding of insects, bird and animals. This means the local communities in future will have to look for new grazing grounds and may be travel long distances in search of grazing areas.

## Water bodies

During the site visits, Ngarama site is surrounded by   swamps containing water bodies which will be contaminated. This means the soil erosion and water runoff will end up in these water bodies when finally the project activities are implemented. The swamp is at the end of the sloppy land on this site and so it is going to be collection area for most of the soil debris and water that runs down the slope. This will affect the water that the local communities have been using for domestic purposes. The swamp has vegetation which is a habitat to some birds and other animals like snakes, fish and amphibians.

When the project starts then the problem of siltation will arise and in future may lead to loss of some animals. If the situation is not handled in future in time, then the whole swamp composition will be affected.  However it should noted that when the project diverts the water that pours in the swamp, then there is high possibility of the swamp reducing or disappearing which will result into knock on effects in the swamp ecosystem.

## Socio-economic state

The majority of the local communities rely on agriculture as a main source of income and so they carry out subsistence farming to supplement on their incomes. The main crops grown in all the sites include bananas, maize, beans and vegetables among others. A few animals like cows, chicken and goats are kept. The implementation of the project will displace some members of the community while others will be forced to migrate to new areas. This means that the social setting of the local communities will change and even the economic activities will change. Apart from that some of the local communities will be employed by the construction company and this will change their economic activities. This may have a long term impact as those used to working on road construction may be influenced to leave agriculture. All these sites maram roads that the local communities use to transport their products or to travel for other businesses or for other economic activities will be upgraded. This will involve expanding the present roads and diverting others which in turn will lead to relocation of the people in those areas that are going to be affected. Not only that the problem of noise from the tractors, bulldozers, excavators and soil compactors.

## Mitigation measures

As a requirement by REMA that all projects should point out the impacts and mitigation measures, it was our obligation to highlight them as indicated below:

- Total loss of vegetation in the sites that will have construction of roads and houses will be mitigated by ensuring that roadsides are planted with grass to retain the appearance and also reduce on soil erosion;

- The  construction of drainages on the sides of the road will be done  in such a way  that they are not so steep  to minimise the speed of runoff and also the points where the trench empties in water body or bigger trench there will be wire mesh to cut off the bigger debris particles and materials that my block the drainage or end up contaminating the water body;

- Total displacement of the animals such as birds that have been nesting and breeding from the vegetation that is currently on site will be handled by ensuring that not  all the trees or vegetation especially that which is not directly on the line of clearance are spared to limit on displacement of the birds, insects and other organisms;

- Change of the scenery of the sites and disappearance of the aesthetic appeal of the sites will be handled by ensuring that the planting of fat growing trees and vegetation to replace those cut so that the appearance is restored. For areas where houses will be constructed there will be portion for planting grass in compound of the new houses to restore the green;

- There will be displacement of some of the local communities from their current locations to other different locations when the project starts. The solution to this will be by first sensitizing the people will be affected so that it is not a surprise to them. This will prepare them psychologically so that it will be easy to resettle them. There will also be compensations to those who will lose a lot of their property or want to go to their preferred areas. So meetings together with the community leaders or authorities will be organized to come to an agreement with those who will be affected in advance.

- Increased soil erosion and water runoff after the vegetation is cleared will be handled by planting vegetation in most areas that will be cleared. In case where water runoff can end up in a water body then the drainage will be designed to reduce on the amount that ends in the water body as well reduce on flooding. In case of runoff from iron roofs the local communities will be sensitized on water harvest methods to cut off the water that may end up causing erosion.;

- Destroying the agricultural crops of certain local communities in the sites that the project will take place. This will be handled by sensitizing the communities and enabling them to access the fast maturing seeds to be able to have food for consumption.

- Destruction of the food web of certain organisms since others may die will be handled by being conscious and ensuring unnecessary killing of organisms is avoided. Those in charge of excavation and use of bull dozers will be advised in advance so that in case they happen to encounter such a case will be aware of the measure to take.

- On top of that there is potential of air and noise pollution from the machines and graders that shall be used during construction. This will be mitigated by ensuring the work is done only working hours and even during these hours if the machine is not in use is switched off. The construction activity will not be carried out at night or awkward hours to avoid inconveniencing the people within the vicinity of the project with noise or air pollution. The problem of vibrations due soil compacting by the machines will be limited and only used where the tractors cannot do well.

# CHAPTER DIXSEPT: IMPLEMENTATION STRATEGIES AND PHASING

## 17.1. Introduction

The implementation and phasing strategy proposed for Ngarama town is aimed at pursuing sustainable development which involves seeking positive improvements in the quality of the built, natural and historic environment, as well as in people’s quality of life, including:

- Making it easier for jobs to be created in the town, market centres and even in the farms;

- Moving from a net loss of bio-diversity to achieving net gains for nature

- Strategies for informal settlement restructuring and preventing creation of new ones;

- Improving the conditions in which people live, work, travel and take leisure; and;

- Widening the choice of high quality homes.

To kick-start the implementation process of 500ha as clearly captured in the TOR have been selected in town sites of Ngarama and Gitinda for detailed urban design studies and identification of projects that will power the plan and motivate other areas into doing the same.

### 17.1.1. Implementation Strategies

The tools and strategies proposed for the elaboration of conceptual and detailed studies of the plan of Ngarama urban center have been identified from basic rules based on the existing situation, Gatsibo District Development Plan and other important Government programs. The strategies are;

- Land Readjustment and land pooling

- Zoning Plan (Zoning Map and Zoning Regulation)

- Capital Improvement Plan

- Quick Impact projects.

### Land Readjustment and Land Pooling

The concept of land-pooling consists in acquisition of a plot of land divided into a large number of small parcels belonging to an equally large number of land owners; plan and provide all necessary infrastructure such as road, water supply, drainage, electricity and telephone, open spaces, community service area; consolidate and re-plot the parcels and give back to the owners. The cost of planning and providing infrastructure is covered from the land itself to be contributed by each landowner. The steps below elaborate the standard process of land-pooling which is also applicable in Ngarama urban center.

The urban land re-development for Ngarama and Gitinda can be carried out through the following ways:

- Land acquisition, infrastructures development and distribution back to local residence and other people.

- Guided land development

- Land pooling and

- Land development by private organization.

### Capital Improvement Plan

The Capital Improvement Plan aims at improving infrastructure capacity by providing public facilities required to support the Vision, Land Use and Zoning Plans proposed for the Priority areas which can also apply for other areas. The proposed capital improvement projects should be funded by the government and should include the following:

- Infrastructure development, which includes development of roads, water supply, power Supply, sewerage and drainage systems.

- Development of public facilities required in the priority sites such as development of schools, clinics and other government buildings, museums, public libraries etc.

- Development of parks and public spaces such as the greening and beautification of roads along the center core.

### Identified Projects under CIP

It is understood that CIP projects are generally funded by the Government, but the question of how the Local Government will generate the resources still exist. We have studied the budget making guidelines for Local Government from MINECOFIN and what the law allows the Local Governments to tax in order to generate revenue which include sales taxes, service fees, fines and revenue. Some of the projects that fall under Public facilities projects are public buildings, such as government offices, schools, libraries, religious institutions, hospitals or health centres, sport clubs and complexes, cultural centres, parks and public open spaces.

On transportation projects we have recommended under the proposed layout network of roads which recommends tarmac on existing roads, widening areas which are narrow joining and linking new roads and numbering and naming of the roads.

Under public utility projects like water distribution networks and water storage reservoirs, sewerage networks, electricity networks and electrical substations which are expected to be funded by all relevant bodies.

### Other Development (Primary School, Hotel to be constructed)

The hotel to be developed will also attract hospitality and service geared towards the increasing number of Visitor coming to Ngarama to experience the King’s drum on the famous Ngarama rock. The primary school developed under the 9YBE will also be an attraction for most people living in this compact vibrant neighborhood and Imidugudu proposed in the area. These initiatives are pure private but in case the private sector is reluctant to intervene, the government can take a lead and develop the facilities and later privatize them for more effectiveness and efficiency. But again Public Private Partnerships is encouraged especially through BOT (Built Operate and Transfer) mechanism.

### The Estates with Rain water Harvesting

This proposed estates will be a demonstration of sustainable neighborhood through various mechanisms like water harvesting and use of alternative energy. Together with this estate is the plan improved through plot servicing which will be a clear demonstration of how to start sustainability small but continue to aim big by avoiding urban sprawl and creating vibrant urban neighborhood.

### Key projects to transform the area

To ensure that we generate projects that will transform the area, we have carried out one key project for the area under the urban design. This area selected from the site enhances and solidifies the mixed compact urban neighborhood where we propose 4 in one housing typologies just aimed at promoting efficient land use. We also aim to have all the categories of density mix, neighborhood commercial, as active open space and social and public facilities to promote social mix and mixed zoning principal.

### Quick Impact Projects to spur the development

To keep the initial Momentum for Ngarama, Nyarubungo and Gatungo based on the existing conditions and aimed at ensuring sustainability by avoiding sprawl, short term projects that will bring immediate benefits have been proposed. The reason for proposing quick impact projects is aimed at setting the standard for the surrounding developments to live up to. The important issue with these projects is that they need to be implemented in a timely manner in order to trigger the development on time and in the right direction. These projects will also help speed up the development process and therefore aid employment generation, housing supply, and rejuvenating urban areas with existing developments.

The projects have been proposed so that the infrastructure is first put in place so as to help supporting the future developments

### Selection Criteria

- Minimal expropriation costs that can cause delays in implementation

- Capability to showcase potential solutions for neighborhood development.

- Possibility of execution of the project through community work (Umuganda) as a starting point and finally within time frame of 5 to 10 years.

- Capacity to attract investments within a short time

- Feasibility of execution under the PPP mechanism by selecting the appropriate scale of the project.

- Possibility of taking advantage of the existing infrastructure like the health center and the electricity dam areas

- Roads opening by removing top soil in most of the areas through Umuganda

    - Identified Projects for immediate action to achieve quick impact

- Ngarama Market

This proposal seeks to take advantage of the informal open air market that sometimes operates within the area. Also important to note is the national road running through the area which will also spur development as traffic stop by to purchase groceries.

- Costing

The proposal has been estimated to cost about 20million to put up and will create a busting market area for groceries and food stuff which will mostly be coming from the surrounding farms and agricultural plantations.

- Clinic, Church and workshops

This area already has a health center, a church and a dam. This project has been identified in the design process so as to spur the development. By putting more resources especially on the improvement of the workshops which we recommend private partnership with the District and directing all wood and building materials acquisition to this site, it will create a one stop point where these services can be acquired hence a vibrant Bursting area generating economic activity.

- Road the one cutting across the site

This road estimated at 2.3km has also been proposed as a priority for tarmac. Since this road proceeds all the way to the other site of Gitinda then it will link this new development with other developments further opening up the area. We note also that a section of this area will be done in the plot servicing to ensure good connectivity. The second road measuring 1.6km has been proposed also as a priority to further open the area and generate traffic. This road links directly to the neighborhoods and also links with the Gicumbi national Road and will form part of the inner city transport that will generate active public transport under EDPRS II.

- Costing

The estimate of doing a 1km of tarmac for this area for is 450million Rwandan Francs. The total of 3.9km will cost a total of 1.8billion Rwandan Francs.

Density control

Cost of service provision and Density control

Different densities have been proposed as we seek to establish a long range planning process, we have used density to propose building heights, bulk, and setbacks, increase open space and thereby enhance the livability of Ngarama urban center by supporting neighborhood development with vibrant retail and services. We have proposed a minimum lot size of 300sqm which calculates to approximately 26 lots per hectare. We propose a long term sustainable plan of having 60 dwelling units per hectare. In order to achieve this, we need a floor area ratio of approximately 0.35

Table 39: Proposed phase

| Phase | Area | Time | Period |
| --- | --- | --- | --- |
| Phase 1 |  | 5 years | 2015-2020 |
| Phase 2 |  | 7 years | 2015-2022 |
| Phase 3 |  | 10 years | 2015-2025 |
| Phase 4,5,&6 |  | 20 years. | 2015-2035 |

Phase1 (2015-2020)

- Development of Schools in Ngarama and Gatungo.

The schools are badly needed because our analysis of the existing situation indicated that the population will increase in Ngarama urban center due to the proposed projects mainly the National road that passes through the area from Gicumbi to Nyagatare and other development projects. The proposed schools include the nursery, primary, secondary, 9 and 12 YBE, technical schools, and later a University.

- Hotel in Ngarama

The hotel will attract more investment in this area. The development which will be private developer needs support from the public through the District by developing the necessary infrastructure so that more investment is attracted. The hotel is proposed because of the untapped tourism activities that are in the area including the King’s drum on the rock. It’s envisaged that this rock will attract many visitors after the completion of the national road and therefore a hotel is a prerequisite.

- Demarcations and marking of roads

As a way of protecting the road reserve it is recommended that all roads in virgin areas of Ngarama and Gitinda where very little development has taken place, should have all roads being marked and where possible remove top soil through Umuganda and other community works

e) Ngarama and Gatungo sites Roads development

These sites have been highlighted as the two selected areas for purposes of developing the LUDP, A number of projects have been highlighted.

Phase 2 and 3 (2020-2030)

- This phase will include among others;

- Allocating and aiding acquisition of land to developers

- Continue with Infrastructure development and expansion

- Construction of cultural community hall

- Development of the workshop facility and green parks

- Road expansion in Gatungo and Ngarama

As the area most adjacent to the urban core, these areas is recommended for high density mixed use and create a bustling urban area of mixed use.

- Land acquisition of the area marked for the development of the CBD core.

- Infrastructure development linking with the CBD

- Land sale of mixed use parcels.

Development of other are

Other areas that lie outside the selected priority areas of Ngarama and Gitinda will also be converted into mixed use urban neighborhoods, commercial, light industrial, and other important uses

- Expansion of roads

- Development of infrastructure

- Development of pedestrian areas

Phase 4, 5&6 (2030 onwards)

Continuous review of the projects and the ones that have been carried out will continuously be evaluated. More consideration in terms of scope, time frame and boundary will be monitored.

Map 39: Phasing map of Ngarama LUDP

Source:  V.COLTD, 2015

Table17.2: Project phasing for implementation

| PROJECT |  | PHASE 1 | PHASE II | PHASE III | PHASE IV |
| --- | --- | --- | --- | --- | --- |
| Water | Public spaces |  |  |  |  |
|  | Commercial |  |  |  |  |
|  | Residential |  |  |  |  |
| Housing | Low Density |  |  |  |  |
|  | Medium Density |  |  |  |  |
|  | High Density |  |  |  |  |
|  | Apartment |  |  |  |  |
| Roads | Murram |  |  |  |  |
|  | Tarmac/Stones |  |  |  |  |
|  | Concrete |  |  |  |  |
| Electricity | Public spaces |  |  |  |  |
|  | Commercial |  |  |  |  |
|  | Residential |  |  |  |  |

## Implementation Strategy Activities, Action Plans, Actors and Source of Funding

Table 40: Implementation strategy

| Tab Strategies/ Targets. | Activities | ACTION AREA | WAYS TO ACHIEVE | PERIOD | Responsible person/Actors | Source of Funding |
| --- | --- | --- | --- | --- | --- | --- |
| Phased development based on priority areas | Break-down proposed implementation plans into smaller components based on availability of funds Engage both private and public players in development Improving housing, water and infrastructure. Regularization and incremental upgrading of informal settlements Low-cost housing and efficient energy supply | Phase 1 | Demarcate and place boundary on plots especially the section with less buildings Clear and remove top soil for road reserve to avoid development/encroachment on road reserve. Build priority roads in phases e.g. from earth to murram then tarmac over a specified period of time. Set water distribution points at strategic locations and later distribute closer to residents before connecting each household. Allow people to upgrade their housing conditions in a regulated manner and provide formal credit where possible Encourage and assist in the development of alternative sources of energy e.g. biogas, solar | Medium term Short term | District Authority -RTDA -RHA -PPP Community through Umuganda | RLSDF CUIP Donors Loans District finance Private sector Cooperatives |
| Stimulate development through incentives | Promote investment in priority areas Develop a strategic land bank for potential investors especially for high-rise residential houses. | Phase II | Provide serviced land (with infrastructure and services) Give tax breaks to investors Quick approvals of building plans Encourage forming of cooperatives to develop houses in serviced areas Encourage production and use of locally available materials like bricks and tiles from clay | Short term Medium term | District Authority RTDA, RHA, Local leaders, Community, Youth groups, Women groups, BOT, RDB, MINICOM, RRA, PSF, MINIEAC | CDF, Donors Loans, Cooperatives, Saccos e.g Uwalimu and Umurenge Sacco VUP Religious institutions to build offices |
| Improve existing development | Improve environmental condition, better sanitation, provide basic services & utilities. Encourage residents to improve their living condition through economic empowerment e.g. brick-making, carpentry, Home-based economic activities like tailoring, bread-baking etc | CBD | Expand Roads, provide drainage, expand water points, introduce open spaces, provide dumping containers within neighborhood, encourage urban renewal –Repainting of buildings, replace roofs, conserve existing vegetation | Short term Medium term | District Authority REMA RDB (One-stop center) RTDA, RHA, EHC, WSC | Community contributions CDF Donors Loans |
| Enhance access to formal credit | Facilitate access to credit for house improvement, redevelopment, for setting up small-scale business activities, Develop rotating fund for housing finance, sensitize residents on use of credit facilities and proper financial management | Women and youth groups | Use land titles as collateral to access credit Mortgage existing property Form cooperatives and Sacco’s for periodic savings and advancing loans to members | Medium term Long term | Banks MFIs Local community Local leaders | Banks BRD |
| Capacity building | Train district staff on how to implement the proposed plans. Liaise with one-stop centre staff and consultants in implementation | District and sector urban planning and lands office | Strengthen the urban planning and land office Introduce planning and development control unit Set up resource mobilization committees | Short term Medium term | Consultants District Authority RIAM WDA RNRA,PSCBS | Donors CBOs Government agencies NGOS PSF |
| Apply/use of performance targets (Imihigo) | Set specific timelines | Cell and sector | Set timelines for demarcation of plots and issuing all documents including building permits, clearing and grading of new roads, widening of existing roads | Short term | District Authority RTDA, RHA EWSA, RECO Central government Citizens through Umuganda | Central Government NGOs, Donors Religious institutions MINALOC |
| Zoning Map | Combine all Data dealing with elaboration, implementation and Registration as well as issuance of building permit in a GIS format so as to make it easy to know the zone and the plans for the area | All | In GIS format develop on zoning map of the priority areas to be implemented | Immediate | Land office RNRA Consultants RHA | Central Government Local Government |

CHAPTER SIXHUIT: ENVIRONMENTAL AND SOCIO ECONOMIC

MANAGEMENT FRAME WORK

18.1. Executive Summary

This section summarizes identified Environmental and Social Impacts arising from proposed implementation of NGARAMA Local Urban Development Plan. It also outline project activities, materials, products, possible environmental and social impacts, recommended mitigation measures to reduce consequences of negative impacts, while improving on the positive impacts.

## Overview

The primary objective of the proposed project is the Implementation of NGARAMA Local Urban Development Plan which entails

- Understanding development needs and aspirations of GATSIBO District

- Development of general layout plan of the selected areas covering 2000 hectares and 500Ha for detailed study in terms of producing the plans and the development of plot details that will ensure rational exploitation and intervention by the District.

- Preparation of draft CAD plans for roads and other utilities that will form the basis of actual works in the exploitation and intervention of these areas.

- Preparation of final implementation plans for roads, electricity, drainage, fiber optic, sewerage, and housing among other utilities.

### Main Project Components

The proposed development consists of the following key components-

- Layout plans NGARAMA area and proposing land uses based on compatibility and suitability

- Layout of roads and grading of the same

- Infrastructural utilities- electricity, water reticulation, fiber optic, communication networks

- Parking spaces

- Sanitation proposals-Sewerage, Septic tanks, Pit latrines

- Housing alternatives together with proposed building materials and location of the settlement

- Greening, aesthetics of the plots and settlements

- Solid waste management systems and sites together with foul water/waste water treatment areas

- Light industries (non-polluting)

- Resettlement, resettlement sites and their social impacts

- Informal settlement upgrading

- Social-Economic impacts to NGARAMA and surrounding area

- Access to basic services

Disasters caused by natural hazards obstruct development in the District as well as in Rwanda, and efforts to address disaster risks prevention and climate change adaptation are key priorities in the DDP for the next five years. In this regard, the District has taken a big step by establishing strategies to address the disaster issues. The consultant intents to partner with the District during the implementation study. Climate change and disaster management will be mainstreamed in the DDP through the following strategies:

- Increase afforestation from the current 121,732ha to 122,732 ha as total area forest plantations in the District;

- Ensure forest biodiversity and critical ecosystems sustainably managed;

- Reduced pollution and vulnerability to climate change;

- Environment committees and clubs be supported at all levels in the District;

- Large capital projects to incorporate EIA (Environmental Impact Assessment);

- Rehabilitation and management of the existing forest resources will be given a lot of emphasis;

- Development, protection and rehabilitation of the existing mining and quarry will be emphasized;

- Environment conservation and regulatory standards will be promoted in the process of building a clean environment in mined sites;

- Implement public places greening initiatives.

### Main Implementation Activities

The main project activities include, but not limited to the following:

- Site preparation, including site clearance and leveling for roads and other infrastructure lying

- Excavation works for main building, access roads, and infrastructure works

- Construction of building foundation works

- Construction of superstructures works

- Installation of fittings, internal/utility services

- Development of external works- lighting, water supply, sewerage, drainage, parking

- Landscaping, greening and leveling of sites

- Final inspection of works

- Decommissioning of works

## Introduction and Baseline Information

### Methodology

    - The Environmental Assessment Process

The following steps were followed in undertaking the environmental Impact assessment (EIA) process for the preparation of the Environmental and Social Impact Assessment (ESIA): 
Project description and design (based on previous detailed Feasibility Study, ESIA TOR and discussion with the Project Proponent/team) 
Formal Stakeholders’ Workshops in NGARAMA which included local communities from the study areas as well as NGOs

- Scoping and bounding of environmental investigations based on Feasibility Study and Stakeholders Workshop.

- Field investigations in which local communities were employed to assist with socio economic surveys, heritage and ecological investigations.

- Environmental baseline description.

- Description and assessment of impacts

- Environmental Management Planning

- Mitigation and enhancement

- Environmental monitoring

- Institutional support

- Preparation of an ESIA

Throughout the above process, the Consultant attended regular monthly community meetings in NGARAMA to take note of any concerns raised by local community.

The EIA process was based principally on the Environment Acts and regulations

    - Scoping

Scoping was the process of identifying, defining and prioritizing the environmental components addressed in the EIA. The method of scoping used for this EIA is based on: 
Feasibility which considered the engineering, economic and environmental aspects of the Project. It provides a summary of the environmental and social issues identified during the Feasibility Study.

    - Literature review:

Reviews of all available engineering and environmental documentation on the subject of the plan;

    - Source of Information and Data Analysis

The methodologies outlined in the following Sections have been formulated on the basis of: 
Relevant documents, including REMA directives, guidelines and other documents; relevant International, national and local legislation, policy papers and guidelines of’ Rwanda

    - Available survey plans and related reports.

Practical considerations including timeframe for Environmental Social Impact Assessment, and the accessibility of the study area

Site visit and consultations with local officials and affected people

## Detailed site survey

- Interviews with government authorities, relevant institutions and NGOs

- Settlement Plan investigations

- GEO-INFO AFRICA also carried out site reconnaissance for vegetation cover, forest, topographical and geological maps, soil maps, aerial photographs, and previous survey work undertaken

- Data collection relied on both primary and secondary sources, and included an environmental factors survey for discussion with selected samples of local residents and project affected persons who have knowledge of the local ecosystem.

- The data collected was validated by experts in the team.

The main output was to:

- Identify environmentally sensitive areas and to assess the area’s sensitivity to negative effects from construction and operation of the plan and associated works.

- Assess impacts and changes that may be induced by the Project, and identify mitigation measures to avoid or lessen the negative impacts.

Terms of Reference (ToR)

The TOR of this EIA Project Report for the proposed Master Plan implementation plan addresses the following key objectives:

- Review existing legal and institutional framework related to proposed project.

- To collect and collate baseline information relevant to the proposed development

- To analyze collected primary data through observations, interviews and public consultations.

- Identify and assess positive and negative impacts of the proposed project

- Identify and analyze possible options for the proposed project

- Develop mitigation measures and cost estimates for the negative impacts of project.

- Design an Environmental Management Plan (including cost estimates)

- Design monitoring framework for the environmental impact assessment of the project.

Site Location

The implementation project is proposed for a site in NGARAMA Sector that sums up to 2000 Hectares in GATSIBO District.

The Developers

The project developers are many but the key stakeholders include the local authorities of GATSIBO District, NGARAMA Sector, NGARAMA Residents, Rwanda Housing Authority, Rwanda Roads and Transport Development Authority, REMA, REG/WSC, Private Sector Federation, RSSB amongst others.

Land Tenure

The properties are under varied tenure system but the ultimate owner of the land are the people leasing it up to more than 20 years and others 99 years depending on the location.

Area size

The total area of the sites is 2000 hectares.

Site location maps 1 and 2.

Site Physical Conditions

Relief

The relief of the Sector of Ngarama is characterized by scarcely weak hills separated by valleys that are dry during a big period of the year.  The region of Ngarama is located in a granite depression of which the average altitude is of 1550 Mr.  The Sector of Ngarama spreads itself on the plateau and the savannah of the East of the country.  The topography of the zone is not largely mountainous and constitutes a potentiality for the introduction of the agricultural mechanization and human settlement.  The agricultural development of the District on the part will necessitate well heard fitting installations for the protection of the grounds.  This relief offers to Ngarama a vocation agro pastoral and tourist.

Drainage

The drainages utilized in the sites are mostly along the roads

Rocks/Soil Condition

The area is covered by rich volcanic soils ideal for agricultural production. There are areas rich with rocks for quarrying while the soils are also suitable for brick production.

Climate

The Sector of NGARAMA is characterized by two principal seasons: a long dry season of which the annual average temperature varies between {PN20,3° CPN} and 21,7° C.  The rain season is short and influences negatively hydraulic availability for the activities agro-sylvo one-pastoral.  The center forecast of Kiramuruzi a trump of followed by the microphone climate of the region.

Rainfall

Average annual rainfall is 1200mm, which may actually vary from 500mm to more than 1100 mm. Thunderstorms, may occur, nearly always during the afternoon or evening, during most months of the year but they are rare during the period June/August. Hailstorms are common here.

- Temperature

Average, daily temperature varies from 22o C in January to March, to 30o C in July/ August. The maximum daily range of temperature is 21 o C to 26 o C in May and February respectively.

- Wind Patterns

A significant feature of the climate is the frequency with which the wind comes from the South East. These are South East Monsoon, which blow very steadily but without high intensity. The mean wind speed is at maximum in December. Wind speed also remains high during January, February and March which coincides with the period of higher potential evaporation. The strongest winds occur during the dry season just prior to the “Long Rains” when speeds of 15 to 20 miles per hour are not uncommon from Mid-Morning to early afternoon; at other times of the year winds speeds are usually 6 to 12 miles per hour. During the night the wind is usually light.

- Sunshine and Solar Radiation

Solar radiation and sunshine are considered together since they are so closely connected. The region experiences a total of about 2,450 hours of bright sunshine per annum, which is equivalent to annual mean of approximately 7 hours of sunshine per day. December to May is characterized by cloudiness and during these months the average daily sunshine in the county is 6 hours. Frequently there are several days in succession when the sun fails to penetrate the thick stratocumulus cover.

- Smog

Smog is common during the rainy season. This is mostly associated with the development of towering cumulus and cumulonimbus clouds. There is the formation of low stratus clouds during the early morning.

- Heat Balance

On hot sunny days, when the wind is light, considerable turbulence is experienced in the first few 80 feet above the ground due to differential heating of the surface. Dangerous down droughts frequently occur in areas where the configuration of the ground is uneven.

Land Use of the surrounding

The surrounding depicts commercial use being the Ngarama town centre. There are also pronounced residential and surrounding agriculture activities. The sites are proposed for mixed uses according to the Master Plan.

Population and Demography

NGARAMA Sector where the detailed studies for the implementation of Gatsibo LUDP will effected will have in 2020 a population of approximately 60,000

Need of the plan

Due to the rapidly growing population in the NGARAMA urban area, demand for a well functioning urban system is on the rise to cater for the needs of the population. This has brought about the need to implement the Master Plan of NGARAMA Sector on an area of a minimum of 2000 Ha and 500Ha with detailed study.

Employment Trends in the Neighbourhoods

There are many employment opportunities in NGARAMA urban area together with the ever increasing commercial activities especially in agricultural products and in the service industry. Many people move into the region to seek for employment. This has led to increased demand for services in the town hence service provider’s move into the region resulting to an increased demand for various facilities.

Socio-Economic Importance of Office Development

Accessibility to services is a significant input in the process of economic production. Urban areas are where people take refuge as suitable grounds for increased economic production. Development of a functional urban system is vital in socio-economic development hence the need to implement NGARAMA Local Urban Development Plan.

  - Economic Importance of implementation of NGARAMA LUDP

Urban development and construction industry plays a very important role in the economy. Indeed construction index is one of the indicators of overall economic performance. Construction plays an important role in the economy - it is a labor intensive activity that utilizes both skilled and unskilled labor.

It also makes use of locally available materials both from the formal industry and informal industry.

The construction industry creates a lot of forward and backward linkages. It also means that money spent in a construction project circulate in the local economy.

## Legal Policy and Institutional Framework

### The Constitution of the Republic of Rwanda

The Constitution of the Republic of Rwanda of 04.06.2003, as amended to date, provides that:

- Every citizen has the right to live in a safe and satisfactory environment,

- Every person is obliged to protect, safeguard and ensure environment promotion,

- The State ensures environment protection,

- Every person has the right to have a private, individual or collective property (art.29 al.1);

- Private, individual or collective property is inviolable and it shall not be interfered with except in public interest, in cases and manner established by the law, and after payment of a fair and prior compensation. (art. 29 al. 2-3);

- Land ownership and other real rights governing the land are granted by the State.

### Organic Law on Environment

The legislative framework of environment management was set up by the Government of Rwanda by organic law No 4/2005 of April 8, 2005 establishing modes of protecting, safeguarding, and promoting environment in Rwanda. This law governs environment in the broadest sense of the term (lands, agriculture, forests, water, biodiversity, etc.). It focuses on the following principles:

- The whole population must contribute to the protection and efficient management of environment in all its components,

- A particular emphasis must be put on education and sensitization on environment preservation at all levels, especially among women and the youth

- Prevention rather than rehabilitation must be seen as a priority,

- Impact studies must be efficiently conducted before any activity regarding swamps development or slopes,

- The principle of fair sharing of benefits from efficient environment conservation and resources sharing must be respected and explained to all people concerned,

- Advantages of local, national, regional and international interdependence drawn from efficient environment management must also be explained and made understood to all the people countrywide.

Moreover, the country adheres to several international agreements, treaties and conventions, though management legal tools are not yet well developed. Among other conventions ratified by the Republic of Rwanda, the most important ones which have influenced or influence the national policy with regard to environment are:

- Convention on Biological Diversity of June 10, 1992 ratified on March 18, 1995.

- United Nations Convention Framework on Climatic Changes of June 10, 1992 ratified on August 18, 1998.

- United Nations Convention on Desertification Control of June 17, 1991 and ratified on October 22, 1998.

- Vienna Convention on Ozone layer Protection of September 22, 1987 and Montreal Protocol on substances impoverishing Ozone layer of September 16, 1987, ratified on December 6, 2000.

- Stockholm Convention on Persistent Organic Pollutants (POP) adopted and ratified by the Presidential Order No 78N) 1 of July 8, 2002.

- Basic Convention on Dangerous Wastes, adopted on March 22, 1989 in Basic and by the Presidential Order No 29 ‘Vl of August 24, 2003 establishing Rwanda adhesion.

- RAMSAR Convention on February 2, 1971 on wetlands

- Kyoto Protocol to the Convention Framework on Climatic Changes of March 16, 1998.

### Vision 2020

Environment protection and management rank among the main pillars of vision 2020. By 2020, the Government intends to have built a nation where pressure on natural resources mainly lands, water, bio-rnass; biodiversity will have reasonably been decreased and the pollution process and environmental degradation reversed. The management and protection of these resources and environment are more rational and strictly under control in order to preserve and conserve for the future generations a basic heritage which is likely to ensure sustainable development.

Therefore, with regard to natural resources and environment protection and management, the Government of the Republic of Rwanda has set on mission of decreasing the percentage of households involved in direct exploitation of primary agriculture from 90% to 50%; setting up efficient and updated regulations which are appropriate for sustainable protection and management of natural resources and environment; decreasing within the national energy assessment rates of diseases related to environmental degradation and firewood from 60% and 94% to 50% each.

### National Policy on Environment

Rwanda National policy on Environment (NPE) of November 2003 provides for sustainable environment protection and management. The policy is benchmarked on the following principles among others that: 
It is every person’s right to live in a safe and stable environment, but on the other hand, they must keep it salubrious

- The national economic growth must be based on rational use of resources and take into account environmental dimensions

- Active and effective participation of the whole population for environment protection and management.

- A special emphasis must be laid on environmental education and sensitization program at all levels with more involvement of women and the youth,

- Environmental impact is to be analyzed while conducting studies of development projects.

With regard to health and sanitation, the NPE proposes among others:

- to set up a system of waste collection, transport, disposal and elimination,

- to establish norms of zone protection between dumps, human buildings and water sources,

- To set up an appropriate canal and evacuation system for waste waters and rainwater in towns and resettlement sites “imidugudu”.

### Institutional Framework on Environment

In Rwanda, institutional responsibility as regards environment and natural resources management falls under several ministries, decentralized entities (Districts and Sectors), local and International Non-Government Organizations (NGOs), research institutions and’ or higher learning institutions.

    - Districts

With the recent administrative reforms, Districts have been entrusted with new powers in terms of environment management and this will certainly have a positive impact on environment and natural resources management in Rwanda. Therefore, the new organic framework within districts provides a post of a professional in charge of environment and natural resources at each level with the following duties:

- To follow up the implementation process of plans and master plan of land in terms of their usage (agriculture, livestock, housing, natural reserves, etc) with regard to environment protection and natural resources conservation;

- To develop a sensitization program on environment protection for the benefit of the population;

- To develop sensitization programs for private entrepreneurs so that they invest in activities aimed at environment protection;

- To set up pollution control mechanisms;

- To control mine exploitation;

- To promote new energy renewable sources;

- To promote rational use of energy and water,

- To receive and analyze bids on mine and carry extraction;

- To check up whether laws and regulations relating to activities of mine and quarry exploitation are respected;

- To set up development programs for swamps and conservation;

- To warn competent authorities in case of breach of environment related regulations and disasters that constitute a threat to environment quality,

- To check up whether recommendations from studies on environmental impact are respected during the execution.

Ministry of Lands, Environment, Forests and Natural Resources

MINIRENA is responsible for land use development policies (including research, land classification, agrarian Laws, and land regime), development of environmental policies and procedures (including impact evaluation); natural resources protection (water, land, flora and fauna), environmental legislation, biodiversity and other environmental aspects.

The ministry is headed by a minister and two Secretaries of State- one in charge of lands and environment, and the other in charge of water and mines.

Rwanda Environment Management Authority

As regards biophysical environment management throughout the whole country, the entire responsibility is entrusted to REMA whose main mission is the implementation of the national policy on environment. This institution has among other duties:

- To coordinate various environment protection activities which are carried out by environment promotion organs, and promote integration of environment issues into various development policies, projects, plans and programs in a bid to ensure appropriate management and rational use of environmental resources basing on sustainable production to improve the welfare in Rwanda.

- To co-ordinate the implementation of government policies and decisions taken by the Board of Directors, and ensure an efficient integration of environment issues into the national planning, services and institutions within the government.

- To advise the government on legislation and other measures relating to environment management or implementation of conventions, treaties, and relevant international agreements in the environment domain wherever necessary.

- To make proposals relating to environmental policies and strategies to the Government.

Ministry of Agriculture and Animal Resources

MINAGRI is mainly mandated to develop, transform and modernize the Rwandan agriculture in general (including fishing and livestock). It includes 3 units namely the planning unit, finance and internal resources management unit and ICT unit. In addition, this ministry supervises 2 new merged agencies namely NAEB (National Agriculture Export Board), and RAB (Rwanda Agriculture Board)

Ministry of Infrastructures (MININFRA)

MININFRA is in charge all public infrastructures (buildings, roads, etc.), energy, transport and communication. This ministry is also in charge of housing and town planning. MININFRA mainly gets involved in environment management through urban planning and development.

RDB Department of Tourism and National Parks

Rwanda Development Board’s Department of Tourism and National Parks (ORTPN) under Rwanda Development Board was established by the decree of 18/611973 which was confirmed and modified by Decree-law of 26.) 4/1974 but the department was later merged with other institutions to form RDB. The overall mission of department is to preserve the valuable biodiversity and values of protected areas, and ensure the long run promotion of tourism in collaboration with all parties involved in the benefit of bordering communities and the Rwanda population in general.

The department is responsible for preserving the Rwandan precious biodiversity to ensure the national sustainable development, and as an international heritage, by applying efficient ecological principles and the promotion of strategic partnerships with local communities and other partners. RDB is responsible for making tourism more profitable to all Rwandans through the development of a unique tourist product including natural and cultural attractions and by facilitating the creation, marketing and promotion of high quality educational experiences for the targeted tourist customers, and by ensuring sustainable development through efficient environment conservation. Other institutions include: IRST, NGO

## Nature and Design Components

### Existing Land-Use

The site are characterized by residential, commercial, administrative and infrastructural development

Proposed Development

  - The actual project works and activity components include-

- Excavation works for main buildings and infrastructure works

- Development of residential houses for different income groups together with commercial blocks.

- Development of schools, health facilities

- Development of parking area.

- Development of water supply and rainwater harvesting.

- Development of septic tank and sewerage networks and treatment plant

- Development of surface water drainage system

- Development of electricity supply, lighting

- Site landscaping and trees/grass cover etc.

- Development of the site for light industries

- Clearing of debris and construction waste

- Resettlement of some households

- Reforestation

Parking Spaces

Parking space provision will be 1 parking lot per 80m2 floor space.

Utilities and Services

- Water Supply

The average water demand rate of 250 litres / day, with an estimated population of 60,000 the proposed development will have water demand estimated at 18,000,000litres per day.

The proposed development will be connected to the town WSC water supply. The project will also have complementary water supply from springs, wells, boreholes and rain water harvesting.

- Foul Water Drainage

The proposed development plan will generate about 1,254,159L/ day of waste water. The proposed developments will be connected to a central sewer network and complementary pit latrines and septic tank in the planning areas.

- Storm Water Drainage

The proposed development storm water will be channeled to the storm water drainage along roads and more drainage will be constructed

- Waste Disposal

The per capita waste generation stands at 0.5kg/day. The proposed development will generate about 3257 kg/day of solid waste.  The solid waste will be collected regularly by the local authority, or their agent, for final treatment at the disposal plant.

The refuse collected on site is then transferred to public collection points after which the local authority will transport it to the designated land fill

- Electricity Supply

The proposed developments will be connected to REG electricity supply line.  Considering the frequent power stoppage in the region, plan proposes diversification of energy sources to include biogas, solar panels and wind power. In the long run there will be installation of solar panels of all building rooftops.

- Telecommunication

Both fixed landline and cell phone services are available in Gatsibo for connection.

## Site Landscaping

The site development involves landscaping with excavated soil and rock material. Excess material will be disposed off-site. The site will be landscaped with adequate green cover- trees, shrubs, and grass.

## Project Activities

### Overview

- The proposed project will include the following activities–

- Site preparation

- Excavation works for housing, access roads, and infrastructure works

- Construction of building foundation works

- Construction of superstructure works

- Installation of fittings, internal/utility services

- Development of external works- street lighting, water supply, sewerage, drainage, parking

- Landscaping

- Final inspection of works

- Decommissioning of works

### Site Preparation for Development

- Fencing

All sites will be fenced and secured before construction work starts using a specified quality dust net and protection from falling objects.

- Site Clearance

The site clearance entails removal of any obstructions on the way of the intended construction activity.  In the proposed project, site clearance will not involve the use of heavy machinery or explosives.

- Laying Out the Sites

The site will then be laid out to identify the location of the proposed building structure unit on the site.  The corner points and edges of the proposed building units will be established accordingly.  The marking out will use stakes and strings as well as chalk lines.

## Excavation and Earth Works

In order to develop the proposed development, excavation and earth works are involved. The main method of excavation to be used is trenching in order to accommodate the building foundation / footing and the infrastructural levels. The sites are on stable soil as found out by the soil tests carried out, well drained and underlain by shallow rock.  Construction of the Foundation

The concrete is to be mixed on site and poured using simple pulley system. Minimal amount of ground water is expected to accumulate below the ground surface thus installation of sub-surface drainage system will not be required.

### Utility Services

- Plumbing System

- Water Supply

The internal water supply will have cold water system.  Since the supply is under pressure, the whole water supply system is designed leak proof and has valves to control the flow of water.

  - Waste Water Drainage

The waste water drainage system consists of both drain and vent pipes.  These pipes also incorporate traps, gullies and other assorted fittings.

- Electrical System

The installation of electrical wiring and fittings will cater for lighting, appliances, and heating system.  The installation will also cater for internal communication, telecommunication, television and alarm system.  All installations shall be to REG approval.

    - Works

- Parking

Paved parking shall be constructed to give motor vehicle traffic proper surface on which to move and to avoid dust that can cause diseases to dwellers.

- Water Supply

The development will be connected to the WSC Ltd water supply line available in the neighborhood. Rainwater harvesting will also be incorporated.

- Foul Water Drainage

Ngarama site will be connected to sewer system and will allow households a septic tank and pit latrines etc. however, all commercial buildings will be required to have their own improved waste treatment plants where all the refuses from homes will be treated. This will solve the sanitation issues in the short run pending the centralized treatment plant that is too costly.

- Surface Water Drainage

Surface run off from the proposed development site will be collected in the storm water network. The drainage system will discharge into the existing storm water drains along the roads.

- Solid Waste Disposal

Households will procure a disposal can and a cubicle will be provided for storage of solid waste for a group of 10 households. The storage capacity is one week and waste will then be collected by the local authority twice a week for final disposal. In addition, the local authority will build dustbins along the main roads and avenues where pedestrians will be putting the rubbish especially papers pending their collection.

- Landscaping

In cases of landscaping, it is to be landscaped to plan. This will entail planting of trees, shrubs, grass and related ground cover.  The common lawn/garden will be planted with continuous bed of grass lawn and provide aesthetically pleasing view. It’s planned that 10% of every plot will be reserved for landscaping.

- Perimeter Fence, including Gate/Guard House

A perimeter fence is to be constructed.  This fence will be enhanced and covered by planted green shrubs and/or flowers.  Access gate and guard house are to be provided at the entrance.

- Clearing of Site

The sites will be given a general cleaning, and any leftover material and debris will be carted away.  Similarly, any tools and equipment still on site will be removed.

## Final Inspection

Final inspection is undertaken to ensure that the project has been done properly and according to the terms of the contract. The inspection team will normally include the project proponent/client, the architect, the engineer, town planner, and EIA expert, QS and the contractor or their representatives.

    - Decommissioning

Upon certification of the development project for use, and upon receipt and consideration of necessary legal and other relevant documentation from the contractor, the project proponent will file a formal ‘Notice of Completion’.  The project will then be handed over to the proponent.

## Project Material and Products

### Project Material

The main material input in the project include –

- Natural stone

- Concrete- coarse aggregates, sand, cement

- Crushed stone

- Gravel

- Soil

- Timber

- Steel (reinforcement, casement, wiring, pipe etc)

- Roofing sheets, tiles

- Glass

- PVC material (tiles pipes, conduits and fittings)

- Concrete tiles and paving blocks

- Sand

- Paint

- Plant material – grass, tree seedlings etc.

- Water

- Fiber optic etc

### Project Products

The main products from the project are –

- Residential spaces

- Commercial

- Public facilities- Schools, hospitals,

- Public utilities

- Recreation spaces

- Roads, car parks, pedestrian walkways etc.

- Landscaped site planted with grass, shrubs and tree cover

- Gate and guard house

- Increase surface water runoff

- Increased foul water discharge

- Increased solid waste generation

- Increased traffic (motor vehicle, cycle,  and pedestrian traffic)

## Potential Environmental Impacts

### Overview

Construction of the proposed development is likely to present several environmental impacts. These are either positive or negative environmental impacts.

Table 41: Detailed Impact Matrix for the Proposed Developments

| Potential Impacts | Positive Impacts |  |  | Negative impacts |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Insignificant (Minor) | Moderate | Significant (Major) | Insignificant (Minor) | Moderate | Significant (Major) |
| Fire out break |  |  |  |  |  |  |
| Architectural distortion of the neighborhood |  |  |  |  |  |  |
| Proliferation of uncollected solid and hazardous/toxic wastes |  |  |  |  |  |  |
| Inadequate disposal of human waste during construction phase |  |  |  |  |  |  |
| Modification of micro-climate and disruption of vegetation |  |  |  |  |  |  |
| Increased development without commensurate services |  |  |  |  |  |  |
| Air pollution generated by dust during construction |  |  |  |  |  |  |
| Noise pollution generated by construction activities and generator during operation phase |  |  |  |  |  |  |
| Workers accidents during construction |  |  |  |  |  |  |
| Increased flooding leading to flooding from paved ground and expansive roofs |  |  |  |  |  |  |
| Pressure on infrastructure and services, e.g., traffic related conflicts, water, power and sanitation, |  |  |  |  |  |  |
| Mushrooming of food kiosks |  |  |  |  |  |  |
| Insecurity |  |  |  |  |  |  |
| Creation of employment opportunities |  |  |  |  |  |  |
| Increased tax revenue to central and local governments |  |  |  |  |  |  |
| Improved aesthetics |  |  |  |  |  |  |
| Optimal use of land in the area |  |  |  |  |  |  |
| Increased land use stock |  |  |  |  |  |  |

Table 42: Anticipated Environmental Impacts and Mitigation Measures at the end of the project Life Cycle

| Environmental Impacts | Proposed Mitigation Measures | Responsibility for Mitigation |
| --- | --- | --- |
| Air pollution by dust generated during demolition process | - The demolition exercise will be limited at day time only - All personnel working on the project will be trained prior to commencing the demolition exercise on methods for minimizing negative impacts on air quality. - Construction vehicle drivers will be under strict instructions to minimize unnecessary trips, refill petrol fuel tanks in the afternoon and minimize idling of engines. - All active demolition areas will be watered at least twice a day to reduce dust. - All trucks hauling demolition debris/wastes shall be covered. - Careful screening to contain and arrest demolition related dust will be adopted - Exposed demolition debris of e.g. dust and sand, will be enclosed, covered, and watered daily before transported to disposal site. - Demolition activities will be suspended when wind speed exceeds 25 mph. - Windbreakers will be installed at the windward side of the construction site. - All workers on the site will be required to wear protective helmets and clothing while on duty | - Project proponent - Ngarama Sector - REMA |
| Noise pollution by demolition activities. | - Explosives will not be used during the exercise - Portable barriers will be installed to shield compressors and other small stationery equipment where applicable. - Use of equipment designed with noise control elements will be adopted where necessary. - Trucks used during demolition exercise on site shall be routed away from noise sensitive areas in the neighborhood, where feasible. - Sound barriers are to be installed for pile driving activities. - Idling time for pickup trucks and other small equipment will be minimized to limited time. - Use of very noisy equipment will be limited to daytime only. - All workers operating in noisy areas or operating noisy equipment will be provided with earpieces to protect against extreme noise. - The demolition exercise will be limited at day time only | - Project proponent - Ngarama Sector - REMA |
| Proliferation of uncollected demolition debris and related wastes | - Private contractor will be engaged to collect demolition debris/wastes - All debris/wastes to be collected regularly to control air pollution and injury etc - Receptacles will be provided for storage of light demolition products e.g. timber, plastics tiles etc - Refuse collection vehicles will be covered to prevent scatter of waste by wind. - A licensed operator will collect demolition debris and avoid illegal dumping at unauthorized sites. - All persons involved in refuse collection shall be in full protective attire. | - Project Proponent - Ngarama Sector - RHA |
| Workers accidents during demolition process | - All workers will be sensitized before the exercise begins, on how to control accidents related to the demolition exercise - A comprehensive contingency plan will be prepared before demolition begins, on accident response. - Adherence to safety procedures will be enforced at all stages of the exercise - All workers, pursuant to labour laws, shall be accordingly insured against accidents. - All workers will be instructed to wear protective clothing and head helmet during demolition. - Demolition work will be limited to daytime only avoid workers accidents due to poor visibility | - Project proponent |

| Themes | Possible aspects to consider | Interventions/mitigations |
| --- | --- | --- |
| Mining. extraction of minerals for construction and excavation | There will be extraction of rocks from quarries for building materials. Clay soils will be utilized for brick production | Backfilling of quarries with suitable materials and compaction is recommended |
| Water use and management | Water extraction (surface- and groundwater) for consumption by the population together with major construction. Waste water discharges to rivers, streams, swamps | Continued extraction is discouraged, natural water points should be given at least 8 hours a day to revitalize Waste waters especially from construction sites should be discharged to a central point awaiting tests before being discharged further |
| Land management | Land use planning –incompatibility and sustainability | All plans proposed must be approved by stakeholders with the guide of land use planning experts |
| Forest exploitation. biodiversity | Forest extraction Use of TFP (non-timber forest products), Fires Introduction of alien species | Forest extraction should be limited to a few chosen trees. Permission from the district and REMA officials at the district must be acquired before any forest is harvested. It is proposed that alternative sources other than timber should be used for construction |
| Energy production and use | Sources of energy Energy consumption Energy efficiency | Connection to the national grid is proposed to the site to minimize burning of fossil fuels and use of wood products. Utility of energy efficient gadgets is also recommended for example the use of energy saving bulbs |
| Urbanization, infrastructure and industries | Urban growth and sprawl, urban planning. dams, roads, major infrastructure, polluting industries, tourism | All plans for urban development are recommended for implementation and urban boundaries should be respected. Further development on agricultural land is discouraged and all development should be approved by the district before commencement. |
| Waste disposal and management | Waste production Waste management Public behavior and practices, existing systems, hazardous waste management | Waste production should be minimized and any waste generated should be separated as per the types. Collection for recycling and reuse should be given priority and collection should be systematic and timely Any development that generates hazardous waste should be located away from settlement and licensed handlers should be contracted. All commercial buildings including hotels are required to have their own treatment plants where the wastes from toilets, bathrooms will be treated. |
| Atmospheric emissions | Emissions of greenhouse gases and ozone- depleting substances Air pollutants affecting local or regional air quality | Harmful emissions are not allowed in the area. Any development that may lead to production of harmful gases should not be licensed to operate until they adequately put in place measures to curb these emissions |

## Social Impact Assessment Aspects

Information collected for the SIA was acquired through a range of qualitative and quantitative methods, as described below.

### Consultations with Interested and Affected Parties

Discussions were held with a range of stakeholders during the course of the study. The consultations entailed: 
The community meetings and group discussions in villages around the Implementation sites, as well as in selected areas around the sites that may be affected (through temporary land occupation) by the transmission pipelines. This included attendance of the periodic district meetings.

### Census of Households in the Plan Implementation

The consultation process registered general consensus to the implementation of the plan with the district encouraging the stakeholders to adopt the plan.

### Socio-Economic Survey

This survey was conducted through literature review, GIS mapping and associated technologies such as remote sensing through satellite and aerial images, GPS, and social economic surveys through administration of questionnaires and structured interviewing of key informants. The sampling procedure utilized for household questionnaires was probability sampling that enabled each HH to have a chance of being chosen, thus minimizing bias. The team further utilized systematic random sample. The sample size for household questionnaires was 100 households

### Recording of Assets

The exercise went further to record assets of those people who may be affected by resettlement

### Stakeholders Consultation

During the field survey for the proposed development, public consultation formed an integral part of project development; this was done pursuant to the provisions of REMA. The following sections present the results of the public consultation:

    - Public Consultation in the Neighbourhood

During the field survey, the residents consulted have no objection to the proposed development as long as the developer will adhere to the zoning regulations of the area. Others felt they be educated on what the implementation entails.

    - Creation of Employment

During the field survey, the residents consulted have no objection to the proposed development because it will create employment opportunities and raise the living standard in Ngarama Sector.

    - Length of Stay in the Area

The respondents who were consulted during the survey have stayed in the neighborhood for appreciable length of time. They felt that this development will enhance property free of the area.

Summary

From the foregoing, it is noted that:

The proposed project has actively involved the key neighborhood stakeholders who did not object the development.

The proposed project does not pose adverse environmental impacts and where they are; mitigation measures have been developed, and are an initiative towards increasing office space supply in the town.

## Social Impact Aspects

    - Quality of life

Once again, there is no doubt that increased access to facilities is a major contributor to the enhancement of the quality of life of the hundreds of individuals in the recipient populations. For those living in the vicinity of Ngarama, the situation is more complex, with a mixture of factors both positive and negative. Elements that will result in a deterioration of the quality of life during the construction phase include such stressors as dust, combustion gases, noise, the increased risk of road traffic accidents, a likely increase in the incidence of HIV/AIDS and STI, a probable deterioration of the general safety conditions in the area, as well as the loss of some resources such as agricultural land, trees, medicinal plants.

Elements contributing positively to the quality of life of the local population are related to compensations that will be paid for the loss of resources in restructuring, the temporary employment during the construction phase of the project, the expected boom in local businesses during the same period, as well as the long-term improvement of the local road and other infrastructure. It can be seen from this mixed set of positive and negative elements that the nature of the mitigations measures that will be implemented as well as their efficient application will be a major factor in the balance between an improved or worsened quality of life for the local population.

Culture, traditions and heritage

Impact on the culture and traditions is evident. Implementation of the plan will mean opening up of Ngarama for people from different walks of life. The cultural inclination may change due to adoption of new ways of doing things but the community is sensitized not to be too quick in changing ways of life.

## Health, Safety and Accident Prevention Plan

### Site Organization

To ensure health and safety conditions and prevent accidents on site, efforts will be made to have a clear site organization plan.  These include:

- Developing a clear organization plan and construction schedule

- Delivery and storage of material at appropriate locations

- Right size of staff/workers with clear work schedule and appropriate dress gear

- Control staff and vehicle movement on the sites and keep out unwanted persons

- Site office with safety kit

- Accessible sanitation facility

- Adequate water supply for both construction work and worker use.

### Project Team

In order to ensure proper organization of activities during planning, design and construction of the project, there must be appropriate project team. This team includes-

- Project Manager

- Town / physical planner

- Environmental Impact Assessment Expert

- Project Architect

- Structural / Civil Engineer

### Enforcement of Standards and Legal Requirement

The project must ensure that appropriate standards and legal requirements are met. These include:

- That building works is in accordance to approved drawings and plans

- That building operations to meet the building code specifications

- That requirements of the Factories and other places of Work Act are followed

- That requirements of the Public Health Act are followed

- Those requirements as outlined in the Environmental Action Plan are observed.

### Activities of Workers

The following activities by workers are clearly identified and must be closely monitored and organized to ensure health, safety and accident standards on site:

- High level construction

- Excavation using pick axes and shovels

- Pushing of wheel barrows

- Hand packing of stones on road surface

- Lifting and laying of building material – stone, concrete etc.

- Plastering of walls and ceiling

- Bending, cutting and laying of reinforcement steel

- Other general building work activities.

### Activities by Machinery and Light Equipment

The activities of machinery and plant must also be properly organized and monitored in order to ensure health and safety conditions and prevents accidents.

The project proponent and building contractor will take appropriate insurance cover for the various project activities and personnel and/or workers

## Environmental Management Plan

### Introduction

Integrating environmental issues in business management, such as those related to real estate development is that it increases efficiency while enhancing the project proponent financial and environmental management.

Table 43: Environmental Management Plan Matrix for the Proposed Development

| Environmental Impacts | Proposed Mitigation Measures | Responsibility for Mitigation | Monitoring Agent | Frequency for Monitoring |
| --- | --- | --- | --- | --- |
| Modification of Micro -Climate | - Careful layout and orientation of buildings to respect microclimate: wind and sun direction. - The project will use minimum reflective building materials and finishes for roof, walls and paving. | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MINIRENA | Periodic checks |
| Increased development density likely to adversely affect ecological carrying capacity. | - The proposed development will strictly adhere to the conditions attached to its approval such as zoning guidelines that include plot/ground coverage and floor index/plot ratio. | - Project Proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MININFRA | Periodic checks |
| Air pollution and dust generated during construction process. | - All personnel working on the project will be trained prior to starting construction on methods for minimizing air quality impacts during construction. - Construction vehicle drivers will be under strict instructions to minimize unnecessary trips, refill petrol fuel tanks in the afternoon and minimize idling of engines. - All active construction areas will be watered at least twice a day to reduce dust. - All trucks hauling soil, sand and other loose materials shall be covered. - This includes all paved parking areas and staging areas at construction site. - Traffic speed of construction/other vehicles will be restricted to not more than 15 mph. - Use of dust nets to reduce spreading of dust - Use of low sulphur diesel - Careful screening of construction site to contain and arrest construction-related dust. - Vegetation will be replanted in disturbed areas as soon as possible. - Exposed stockpiles of e.g. dust and sand, will be enclosed, covered, and watered daily, or treated with non-toxic soil binders. - Excavation and grading activities will be suspended when wind speed exceeds 25 mph. - Windbreakers will be installed at the windward side of the construction site. - All workers on the site will be required to wear protective clothing while on duty | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MINIRENA | Periodic and surprise checks |
| Noise pollution by construction activities. | - Portable barriers will be installed to shield compressors and other small stationery equipment where applicable. - Use of equipment designed with noise control elements will be adopted where necessary. - Trucks used at construction site shall be routed away from noise sensitive areas in the neighborhood, where feasible. - Sound barriers are to be installed for pile driving activities. - Idling time for pickup trucks and other small equipment will be minimized to limited time. - Use of very noisy equipment will be limited to daytime only. - All workers operating in noisy areas or operating noisy equipment will be provided with earpieces to protect against extreme noise. - Use of manual labor as much s possible | - Project proponent - contractor | - Ngarama Sector officials - Public Health Officer - REMA inspectors - Ministry of Labor | Periodic and surprise checks |
| Workers accidents and hazards when handling hazardous wastes. | - Adequate collection and storage of waste will be provided on site, and safe transportation to, and display methods at designated areas. - All receptacles for storing hazardous wastes shall be adequately covered. - All employees will be required to wear protective clothing when handling hazardous wastes. - All workers will be adequately insured against unforeseen accidents. | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - Ministry of Labor | Periodic and surprise checks |
| Proliferation of uncollected solid waste. | - Private contractor will be engaged to collect solid waste generated. - Wastes to be collected regularly to control air pollution and vermin/insects etc. - Resource recovery will be encouraged once the project takes off so as to shrink waste stream and recover non-recyclables. - Refuse collection vehicles will be covered to prevent scatter of wastes by wind. - Waste should be disposed off at a designated site/landfill. - Provide collection bins at designated area - All persons involved in refuse collection shall be in full protective attire. | - Project Proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MINIRENA | Periodic and surprise checks |
| Workers accidents during construction process. | - All workers will be sensitized before construction begins, on how to control accidents related to construction. - A comprehensive contingency plan will be prepared before construction begins, on accident response. - Accordingly, adherence to safety procedures will be enforced. - All workers, pursuant to labour laws, shall be insured against accidents. - All workers will be instructed to wear protective clothing during construction, including helmets. - Construction work will be limited to daytime only | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - Ministry of Labor |  |
| Increased surface run off leading to flooding, from paved grounds and expansive roofs. | - Surface run off and roof water will be harvested and stored. - Storm water management plan that minimizes impervious area increases infiltration by use of recharge areas, and use of retention, and/or retention with graduated outlet control structures, will be used. | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MININFRA | Periodic checks |
| Pressure on infrastructure and services, i.e. traffic related conflicts, water, power and sanitation. | - Delivery and collection hours by service vehicles will be limited to off-peak hours. - Service deceleration line will be provided at the entry point. - Connection to Council water supply line - Adequate roof and water storage tanks will be provided. - Promote recycling of water - Installation of solar panels - Standby generator will be installed to ensure uninterrupted power supply. | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - Ministry of Labor | Periodic checks |
| Mushrooming of informal business | - Pursuant to the relevant laws and policy, development control is to be enforced around the project site. | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MINICOM | Periodic and surprise checks |
| Water supply and sewer/storm water breakdown. | - Regular inspection and maintenance of water and sewer system. - Before construction begins, it will be determined where sewer and water pipes are located to avoid reticulation break down. | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - REG/WSC - MININFRA | Periodic checks |
| Inadequate human waste disposal by workers during construction process | - As provided for by the Building Code, a temporary latrine will be provided on site to be used by construction workers | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - Ministry of Labor - REG/WSC | Periodic checks |
| Fire Outbreak | - All occupants to be trained on fire fighting - Regular fire drills to be conducted to enhance emergency response - Fire extinguishers and related equipment to be strategically installed within the building - Building to be adequately insured against fire - Instructions on how to respond during fire emergencies to be strategically placed in the building - Building to be regularly inspected to ensure that all standards related to fire disaster control are adhered to - Adequate water storage will be maintained for fire fighting | - Project proponent | - Ngarama Sector officials - Public Health Officer - REMA inspectors - MININTER - MIDIMAR | Periodic checks |

| Environmental Impact I Issue | Mitigation Measures | Location | Timing | Responsible Organization | Contractual Clause |
| --- | --- | --- | --- | --- | --- |
| Implementation | Supervision I Monitoring |  |  |  |  |
| Clearances. Approvals and Permits | Only licensed quarries and sand suppliers shall be used. Obtain consent for borrow pit operation from the land owner prior approval of the rehabilitation proposal of the borrow areas from the Engineer Provide a copy of all necessary permits to the Engineer Adhere to all permit terms and conditions. | Project site | Before the commencement of construction Throughout contact period | Contractor | Engineer |
|  | Obtain written permission from private landholders to conduct activities on their land prior to commencing these activities, and provide copies to the Engineer | All work sites | As required prior to commencing the intended activities | Contractor | Engineer |
| Acquisition of land, building and other assets | Complete all necessary land and building acquisition in accordance with Plan and Entitlement Framework prior to the commencement of any construction works. Provide copies of land acquisition details to the Engineer and Contractor. | Areas of impact Area of impact | Before the commencement of construction | Land office | Land office Engineer |
| Acquisition of land, building and other assets | Provide a list of affected property owners to the district and Contractor. | Area of impact | Before the commencement of construction | Land office | Land office |
| Training | Organize environment management and safely training, Contractors and Construction Supervisor’s should attend the training | On site | At least 1 month prior to | Supervision Consultant’ | Lands office |
| Implementation of Environmental Management Requirements | Preparation of Contractor Environments Management Plan Establish Panel of Experts to oversee social and environmental management issues. | All work sites | Before commencement of construction | Contractor | Engineer |
| Health and Safety Issues | Preparation of a health and Safety Plan for workers and impacted communities addressing issues including: • Measures to prevent the spread of HIV/AIDS such as free condoms, awareness campaign • Education of workers and impacted communities • Provision of safety equipment for workers • Use of child labor to be prohibited | All work sites | Before commencement of construction | Contractor | Health officers |
| Construction Plants, machinery and vehicles | Trial run on the plants, | Construction Camp Vehicle depot | Before commencement of construction, | Contractor | Engineer |

## Mitigation Measures

### Overview

A careful assessment affirms that most of the potential negative impacts in the project can be mitigated with significant level of success.

### Appraisal of Alternative Development Options

No Development Investment

The nil intervention describes a situation in which the proponent does not undertake the proposed development. This option would imply economic loss to the proponent, local and national economy. The project site is currently underutilized while there is substantial demand in the area, yet the supply side has been limited. This means that if the current status is to prevail it will not be commensurate to the area value.

In case the authorities such as REMA settle for no development intervention, the project proponent would lose in terms of heavy financial commitments already made in design and planning of the project. The option would similarly make the jobs that the project envisioned to create to be forgone. The local and central government will also lose the tax income that the project would generate if implemented. Needless to add that the nil projects will lead to uncontrolled development.

    - Relocation Option

The other option available for the project implementation is for the proponent to relocate it to an alternative site either within or outside the town. At the moment, the Ngarama Sector does not have an alternative site. The proponent will need to re-engage professionals like valuers, EIA lead/audit experts and physical planners to assess the viability of the new site. New costs will also arise from seeking an EIA approval from REMA for the new site. Additional costs will arise from the design and approval of the architectural drawings for the new site. By the time the proposal is approved by authorities, economic parameters such as cost of building materials would have changed. This would lead to a situation like zero option and the project may no longer be viable leading to eventual abandonment.

    - Exploration of Alternative Land uses

The development could change uses for the sites such as institutional, commercial, or industrial development. If studies establish that these are better suited to the site in functional and economic sense, then the developer could apply for change/extension of use to allow for such development. These options do not offer significant advantages over the current proposal.

    - Preferred Development Option

All the alternative options analyzed have implications, which make the current design options proposed for Ngarama to be more viable economically, socially and environmentally. It is concluded that:

- The alternatives are likely to reduce the returns to investment that the proponent would have realized if the current proposed design were to be approved

- Prime land in the town is scarce and costly. Because of this, the proposed project may not be relocated to an alternative site.

- There are several development of the same nature approved. The proposed development will therefore blend easily with the current development trend in the neighborhood.

    - Potential Negative Impacts and Mitigation Measures

The potential negative impacts and possible mitigation measures for the proposed building development are summarized below –Table 4.

Table 44: Potential Negative Impacts and Mitigation Measures

| Potential Negative Impacts | Mitigation Measures |
| --- | --- |
| Architectural incompatibility leading to distortion of neighborhood aesthetic and image | - Harmonize building scale with existing development in neighborhood. - Harmonize detail, material and finishes for roofs and walls with existing development in the neighborhood |
| Disruption of existing natural environment and modification of micro-climate: -Increased development density -Increased glare/solar reflection -Reduced natural ground cover and increased surface run-off | - Development restricted to approve density – building line, plot coverage and plot ratio. - Careful layout and orientation of buildings to respect wind and sun direction. - -Minimum use of reflective building material and finishes for roof, wall and pavement. - Adequate provision of green and open space planted with grass, shrub and tree cover. |
| Air Pollution and dust generation | - Use of dust nets to reduce spreading of generated dust. - Sprinkling of water on dusty spots - Use of well maintained trucks and machinery that do not produce excess fumes. - Use low sulphur diesel for diesel operated machinery |
| Oil Leak and Spills | - Use well serviced machinery - No servicing of machinery on site - Proper storage of petroleum products. |
| Noise and Vibration | - Usage of manual labor as much as possible - Avoid unnecessary hooting of vehicles - Ensure all generators and heavy machinery are insulated or placed in enclosures to minimize noise levels |
| Increased Water Demand | - Buildings to have water harvesting and storage facilities. - Promote recycling and reuse of water as much as possible. - Detect and repair broken water pipes. - Install water conservation taps that turn off automatically when water is not in use. |
| Increased Waste Water Production | - Provide adequate capacity septic tank |
| Increased Solid Waste Production | - All waste should be disposed off at designated disposal site/land fill. - Re use or recycle packaging material such as cartons and plastic containers - Provide collection bins on site at designated area. - Engage private contractor to collect solid waste |
| Increased vehicular traffic | - Provide paved local access road network |
| Increased pedestrian/cycle traffic | - Provide paved walk way /cycle path network |
| Increase surface water runoff | - Provide adequate storm water drainage system. |
| Increased electricity demand | - Connection to REGL supply line - Provide solar panels supply system - Installation of a generator |
| Worker accidents and health risks | - Employ skilled and trained workers - Educated workers on construction site safety procedures. - Provide workers with protective clothing and head gear. - Prepare clear work schedule and the organization plan. - Have adequate worker insurance cover - Enforce occupational health and safety standards. |
| Increased human conflict | - Encourage formation of community policing - Establish neighborhood association. |
| Loss of property/income | - Compensation for loss of investment - Create awareness on alternative employment activities - Early Notification to affected victims |

# REFERENCE :

  - Affordable Housing Development project Report by RHA, 2010.

  - District’s Baseline Survey, 2009, National Institute of Statistics Rwanda, Republic of Rwanda.

  - Economic Development and Poverty Reduction Strategy (EDPRS), National Institute of Statistics Rwanda, Republic of Rwanda.

  - Environmental Profile of Rwanda, 2006

  - FAO (2007): Community Development and Empowerment

  - Gatsibo District Development Plan (2013 – 2018)

  - Gatsibo Urban Development Master Plan

  - MINIRENA 2012, www.minirena.gov.rw

  - National Urban Housing Policy for Rwanda, Ministry of Infrastrucure 2008

  - NISR: EICV3, District Profile East – Gatsibo 2010/11

  - Rwanda  Development  Indicators,  2010,  National  Institute  of  Statistics  Rwanda,  Republic  of  Rwanda.

  - Rwanda  Vision  2020,  July  2000,  Ministry  of  Finance  &  Economic  planning,  Republic  of Rwanda.

  - Rwanda Building Control Regulations, Ministry of Infrastructure, Republic of Rwanda.

  - Rwanda National Population Projections, 2007-2022, National Institute of Statistics Rwanda, Republic of Rwanda.

  - Rwanda Statistics & Figures, 2008, National Institute of Statistics Rwanda, Republic of Rwanda.

  - The Constitution the Republic of Rwanda

  - The National Land Use & Development Master Plan, National Land Centre, Republic of Rwanda.

  - The Organic Law No. 04/ 2005 of 08/04/2005, Republic of Rwanda. National Land Policy of Rwanda, Republic of Rwanda.

# APPENDIX

- Parcels with coordinate map

- Estimate cost for roads

- Summary for road costing and implementation plans

- Soil test results and recommendation based on test results

- Housing typologies